IRRIGATION AND DRAINAGE

Level-I

Based on March 2022, Version 3 Occupational standard

Module Title:- Irrigation Water Source and harvesting techniques

LG Code: AGR IRD1 M01 LO (1-3) LG (1-3)

TTLM Code: AGR IRD1 TTLM 0822V1

August, 2022

Addis Ababa, Ethiopia

Table of Contents

Introduction to the Module	1

LO #1. Identify potential Irrigation Water sources	2

Instruction sheet -1	2

Information Sheet - 1	3

Self-check 1	14

Operation Sheet -1	16

LAP TEST - 1	17

LO #2 - Water harvesting techniques	18

Instruction sheet - 2	18

Information Sheet -   2	19

Self-Check – 2	30

Operation Sheet -2	32

LAP TEST-2	33

LO #3- Catchment area	34

Instruction sheet	34

Information Sheet    - 3	35

Self-Check – 3	42

Operation Sheet -3	43

LAP TEST-3	44

Performance Test	44

Reference Materials	45

# Introduction to the Module

This learning guide covers the knowledge, skills and attitudes required for the identification of potential irrigation water source and identify basic water harvesting techniques under direct supervision. It requires the ability to identify potential irrigation water sources, water harvesting techniques and the ability of catchment area identification.

The main sources of water for irrigation water are groundwater from wells, surface water, drainage ponds, rain and municipal water. To tap groundwater, a well must be dug deeper than the groundwater level. The groundwater then seeps through the pores of the surrounding soil or porous rock into the well until the level of water in the well is the same as the level of the groundwater

| # LG#1 | # LO #1. Identify potential Irrigation Water sources |
| --- | --- |

## Instruction sheet -1

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Introduction - Identifying potential water sources - Water contributors - Soil moisture status - Recharge ground water table - Afforestation on degraded land This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify potential water sources - Water contributors - Soil moisture status - Recharge ground water table - Afforestation on degraded land

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks - Perform Operation Sheets - Do the “LAP test”

## Information Sheet - 1

Introduction

Irrigation potential means a total area which is technically feasible, economically and financially profitable, socially viable and environmentally acceptable that is irrigated or capable of being irrigated on the basis of water availability, land availability and suitability. The area which can potentially be irrigated depends on the physical resources 'soil' and 'water’, combined with the irrigation water requirements as determined by the cropping patterns and climate. Therefore, physical irrigation potential represents a combination of information on gross irrigation water requirements, area of soils suitable for irrigation and available water resources by basin.

Source water refers to sources of water (such as rivers, streams, lakes, reservoirs, springs, and groundwater) that provide water to public drinking water supplies and private wells. The water needed to supply an irrigation scheme is taken from a water source.

  - Identifying Potential water source

Sources of irrigation water can be groundwater extracted from springs or by using wells, surface water with drawn from rivers, lakes or reservoirs or non-conventional sources like  treated wastewater, desalinated water or drainage water. The main sources for irrigation water are groundwater from wells, surface water, drainage ponds, rain and municipal water

- Groundwater: - is found in aquifers that are located below the earth surface. As rainfall occurs, some of it evaporates, some of it is removed by plant transpiration and the remaining water filters down through the topsoil and flows into sand, gravel and fractured rock. It reaches a depth where all the pore spaces are filled. This saturated zone is calling the aquifer.Groundwater is an important source of irrigation water, especially for small-scale irrigation projects. Because groundwater is only available below ground level, it must be lifted, or pumped before it can be used. Pumping groundwater from wells is a well-known method of utilizing groundwater the world over.

Groundwater may be found close to the surface or at profound depths. In coastal plains the groundwater is often brackish or saline due to the proximity of the sea. Inland groundwater may also be brackish in places where the soil contains many soluble salts. Such water cannot be used for irrigation.

Shallow groundwater lies within a few meters of the surface, exploitation is possible with shallow wells which are mostly dug by hand. These wells normally have a diameter of 1 meter or more and are dug below the groundwater table. Water is pumped from these wells, often using human or animal power but, increasingly, with small diesel-powered pumps. The amount of water that can be abstracted from shallow wells is limited, and, as a result, the areas which are irrigated from these water sources will also be limited.

Deep groundwater is very deep, constructing a hand-dug well becomes impossible. Deep wells must be drilled into the ground. Generally, submersible pumps are installed below the groundwater table to lift water to the surface. They may be driven by an electric motor or a diesel-powered engine on the surface with a long vertical shaft, or by a submerged electric motor inside a waterproof casing.  Drilled wells are a clean source of water for many greenhouse operations. However; the water yield from drilled wells is usually limited.

To tap groundwater, a well must be dug deeper than the groundwater level. The groundwater then seeps through the pores of the surrounding soil or porous rock into the well until the level of water in the well is the same as the level of the groundwater.

There are two broad classes of drilled-well types, based on the type of aquifer  the well is in:

- Shallow or unconfined well

- Deep or confined wells

Shallow or unconfined well are completed in the uppermost saturated aquifer at that location (the upper unconfined aquifer). There are three types of shallow well.

- Hand dug well

- Bored shallow wells

- Drilled shallow well

- Surface water source: - is found on the surface of the earth.  Surface water is subject to contamination from sources such as sediment, chemicals and plant growth. High levels of particles can reduce the life of pumps and clog irrigation systems and multiple filters may be required. It is also possible that surface waters can become contaminated with road salt, industrial, agricultural chemicals, algae and plant pathogens. Surface water are dependent on runoff amount from adjacent land or from ground water springs

Surface water includes:-

- Rivers,

- Streams,

- Lakes and

- Pond

Rain water can be collected from greenhouses or building roofs without contacting the ground and held in a concrete cistern, fiberglass or polyethylene tank, water silo or other holding tank. It is clean except for any debris that gets into the system. Rain water will be very low in elemental or chemical contamination unless there is industrial air pollution or fallout on the roofs. The ph of collected rain may be low (4.0 – 5.0) but is not considered detrimental to crops because it is not buffered (does not resist change in ph) and changes readily. Rain water is an excellent and underutilized source of irrigation water.

Municipal water includes water supplied by city, county or municipality. Either, ground, rain, and/or surface water may be used. The cost and quality are typically high since much of the water is for residential use and drinking water and is treated. The key concerns are whether supply is guaranteed in times of shortages and what water treatment procedures are used that may influence plant growth. Municipal water may have fluoride and/or chlorine added at rates which is not a problem for most crops. Occasionally, sodium compounds are added to treat hard water.

Spring water is water that is harvested from a natural spring.  A spring is a point where water flows out of the ground. A spring may flow the whole year or only sometimes. This depends on the water getting into the ground all of the time (rain) or only once in a while (snow melting).  The water continues to seep downwards until it reaches an impermeable layer, flowing along the top of this layer until it reaches the surface. This is known as a gravity contact spring. There are other types of spring also (fracture and tubular, back-stowing, mountain slide and artesian). The objective of a spring development is to make groundwater available for use that is unused, under used, or wasted. A spring often sends water down, along the land. This is how rivers start.

Figure 1.1:  Natural spring

An aquifer is a body of porous rock or sediment saturated with groundwater. Groundwater enters an aquifer as precipitation seeps through the soil. It can move through the aquifer and resurface through springs and wells.

Figure 1.2. Water table and Aquifer

  - Water contributors

Surface water can be withdrawn from :rivers, lakes or reservoirs or non-conventional sources like treated wastewater, desalinated water or drainage water or rain water catchment .while the main contributors of ground water may include :-

- Spreading Basins: This method involves surface flooding of water in basins that are excavated in the existing terrain. For effective recharge highly permeable soils are suitable and maintenance of a layer of water over the highly permeable soil is necessary.

- Recharge Pits and Shafts: Conditions that permit surface flooding methods for artificial recharge are relatively rare. Often lenses of low permeability lie between the land surface and water table. In such situation artificial recharge systems such as pits and shafts could be effective in order to access the dewatered aquifer. The rate of recharge has been being found to increase as the side slope of the pits increased. Unfiltered runoff water leaves a thin film of sediments on the sides and bottom of the pits, which require maintenance in order to sustain the high recharge rates. Shafts may be circular, rectangular or square cross-section and may be back filled by porous materials. Excavation may be terminating above the water table. Recharge rates in both shafts and pits may decrease with time due to accumulation of fine-grained materials and the plugging effect brought by microbial activity.

- Ditches: A ditch is described as a long narrow trench, with its bottom width less than its depth. A ditch system is designed to suit topographic and geological condition that exists at the given site. A layout for a ditch and flooding recharge project could include a series of trenches running down the topographic slope. The ditches could terminate in a collection ditch designed to carry away the water that does not infiltrate in order to avoid ponding and to reduce the accumulation of fine materials.

- Recharge Wells: Recharge or injection wells are used to directly recharge the deep-water bearing strata. Recharge wells could be dug through the material overlaying the aquifer and if the earth materials are unconsolidated, a screen can be placed in the well in zone of injection. Recharge wells are suitable only in areas where thick impervious layer exists between the surface of the soil and the aquifer to be replenished. They are also advantageous in areas where land is scarce. A relatively high rate of recharge can be attained by this method. Clogging of the well screen or aquifer may lead to excessive buildup of water level in the recharge well.

- Harvesting in Cistern from Hill Sides:

In this method construction of small drains along contours of hilly area are done so that the runoff in these drains are collected in a cistern, which is located at the bottom of a hill or a mountain. This water is used for irrigation or for drinking purpose and the water is of good quality.

- Subsurface Dams: Ground water moves from higher-pressure head to lower one. This will help in semi-arid zone regions especially in upper reaches where the ground water velocity is high. By exploiting more ground water in upper reaches more surface water can be utilized indirectly, thereby reducing inflow into lower reaches of supply. Ground water is stored either in natural aquifer materials in sub-surface dams or in artificial sand storage dam.

- Farm Ponds: These are traditional structures in rain water harvesting. Farm ponds are small storage structures collecting and storing runoff waste for drinking as well as irrigation purposes. As per the method of construction and their suitability for different topographic conditions farm ponds are classified into three categories such as excavated farm ponds suited for flat topography, embankment ponds suited for hilly and ragged terrains and excavated cum embankment type ponds. Selection of location of farm ponds depend on several factors such as rainfall, land topography, soil type, texture, permeability, water holding capacity, land-use pattern, etc.

- Historical Large Well across Streamlet: If any historical wells are located near the streamlet, then allow the water into the well from streamlet by connecting drains. In this case the historical wells act as a recharge well so that ground water can be improved.

- Check Dams: Check dams are small barriers built across the direction of water flow on shallow river and streams for the purpose of rain water harvesting. The small dams retain excess water flow during monsoon rains in a small catchment area behind the structure. Pressures created in the catchments area send the impounded water into the ground. The major environmental benefit is the replenishment of nearby ground water reserves and wells. The most common case of check dams is to decrease the slope and velocity of a stream to control erosion.

  - Identifying Potential water ways

Water ways are Systems in which irrigation water is conveyed from source to the required field of Irrigation purpose.  It consists of:-

- diversion structures (diverts water from the source to the canal system) such as weirs, barrage,

- intake structures and canal system  (main canal, sub main canal, distribution canal  ) and

- Pump system for pressurized irrigation systems.

Channels are the irrigation structure or hydraulic structures through which irrigation water flows into required areas. It is defined as opened or closed conduit structure through which water flow. Example, canal (open) and pipe (closed). In an irrigation scheme, water is taken from a water source, passes through a network of irrigation canals and is delivered to the farmers’ fields by using the following field delivery systems.

- Breach

- Gated intake

- Siphons

- Spiles

- Pumping

  - Soil moisture status

Soil moisture is the water stored in the soil and is affected by precipitation, temperature, soil characteristics, and more. These same factors help determine the type of biome present, and the suitability of land for growing crops. In turn, soil moisture levels affect a range of soil and plant dynamics. Surface soil moisture is the water that is in the upper 10 cm of soil, whereas root zone soil moisture is the water that is available to plants generally considered to be in the upper 200 cm of soil. Understanding some basic terms, definitions and concepts will help you make irrigation management choices. Below are some general soil moisture definitions:

- Saturation: At saturation all pore space in the soil is filled with water, no air. Most agriculture soils have between 40 and 50 percent (4.8 to 6 inches per foot) voids (pore space) that are filled with water and/or air.

- Field Capacity: Soil water content after water has drained by gravity. Field capacity of most agriculture soils ranges between 20 and 45 percent by volume (2.4 to 5.4 inches per foot).

- Permanent Wilting Point: Soil water content when plants or crops cannot obtain water from the soil. Permanent wilting point ranges between 7 (sand) and 24 (clay) percent by volume (0.8 and 2.9 inches per foot) for most agriculture soils.

- Available (Usable) Water: The soil water content between field capacity and permanent wilting point. Although plants can utilize the water, plant stress occurs as soil water content approaches permanent wilting point.

- Allowable Depletion: The soil water content available to crops without causing stress that impacts yield or crop quality. The allowable depletion is dependent on crop type, crop growth stage, and climate. Allowable depletion can range between 25 percent of available water for crops very sensitive to small changes in soil moisture to over 50 percent of available water for crops that are less sensitive to water stress.

- Dry Bulk Density: The oven-dried weight of soil in a known volume of field-extracted sample (e.g., using the sample length and diameter from a sampling tube).

- Soil Porosity: The pore volume of soil divided by the total volume of a soil sample.

1.5. Recharge ground water table

Groundwater recharge or deep drainage or deep percolation is a hydrologic process where water moves downward from surface water to groundwater. Recharge occurs both naturally (through the water cycle) and through anthropogenic processes (i.e., "artificial groundwater recharge"), where rainwater and or reclaimed water is routed to the subsurface.

1.5.1. Underground water table recharging process

Groundwater recharge is an important process for sustainable groundwater management, since the volume-rate abstracted from an aquifer in the long term should be less than or equal to the volume rate that is recharged.

Groundwater is recharged naturally by rain and snow melt and to a smaller extent by surface water (rivers and lakes). Recharge may be impeded somewhat by human activities including paving, development, or logging. These activities can result in loss of topsoil resulting in reduced water infiltration, enhanced surface runoff and reduction in recharge.

Recharge can help move excess salts that accumulate in the root zone to deeper soil layers, or into the groundwater system. Tree roots increase water saturation into groundwater reducing water runoff. Flooding temporarily increases river bed permeability by moving clay soils downstream, and this increases aquifer recharge.

Wetlands help maintain the level of the water table and exert control on the hydraulic head. This provides force for groundwater recharge and discharge to other waters as well. The extent of groundwater recharge by a wetland is dependent upon soil, vegetation, site, perimeter to volume ratio, and water table gradient. Groundwater recharge occurs through mineral soils found primarily around the edges of wetlands the soil under most wetlands is relatively impermeable. A high perimeter to volume ratio, such as in small wetlands, means that the surface area through which water can infiltrate into the groundwater is high. Groundwater recharge is typical in small wetlands such as prairie potholes, which can contribute significantly to recharge of regional groundwater resources). Researchers have discovered groundwater recharge of up to 20% of wetland volume per season.

- Appropriate methods to recharge underground water table

Rates of groundwater recharge are difficult to quantify, since other related processes, such as evaporation, transpiration (or evapotranspiration) and infiltration processes must first be measured or estimated to determine the balance.

- Physical methods use the principles of soil physics to estimate recharge. The direct physical methods are those that attempt to actually measure the volume of water passing below the root zone. Indirect physical methods rely on the measurement or estimation of soil physical parameters, which along with soil physical principles; can be used to estimate the potential or actual recharge. After months without rain the level of the rivers under humid climate is low and represents solely drained groundwater. Thus the recharge can be calculated from this base flow if the catchment area is known.

- Chemical methods utilize the presence of relatively inert water-soluble substances, such as an isotopic tracer or chloride, moving through the soil, as deep drainage occurs.

1.6. Afforestation on degraded land

Afforestation is planting of trees in areas that have not previously held by forests or is the planting of trees in deforested areas. It is the restocking of existing forests and woodlands which have been depleted, an effect of deforestation.

A sustainable and well-planned afforestation project helps improve soil conservation, management and water quality. Afforestation projects undertaken without a complete understanding of the surroundings can cause additional environmental damages. For instance, fast-growing trees commonly used in timber plantations consume huge amounts of water, hence depleting water resources around the area. There are also concerns about irreversible changes in the soil caused by exotic species. For example, pine trees are known to turn the soil acidic. The water from the soil eventually trickles down to local streams and water bodies, which, in turn, causes harm to both the water and land ecosystems. In areas of highly degraded land, afforestation is the main solution to retard soil erosion and it also enhances or improve soil water holding capacity or intake rate of the soil.

Trees may be planted:

- To provide timber and wood pulp;

- To provide firewood in countries where this is an energy source;

- To bind soil together and prevent soil erosion and to act as windbreaks.

Before Afforestation                                                  After Afforestation

Figure: 1.6 Areas before and after afforestation

| ### Self-check 1 |  |
| --- | --- |

Name……………………………………………   ID………………………… Date…….

Directions:  Answer all the questions listed below.

Test I: Choose the best answer from the given alternatives. (10 points)

- Which one of the following water source is a point where water flows out of the ground?

- River

- Spring

- wetland

- pond

- One of the following is planting of trees in areas that have not previously held by forests.

- Deforestation

- Afforestation

- Rehabilitation

- Transportation

- One of the following is not factors that affect soil moisture?

- Precipitation

- Temperature

- soil characteristics

- Availability of infrastructure

- One of the following is not true about appropriate methods to recharging underground water table.

- Process where water moves downward from surface water to groundwater.

- Rates of groundwater recharge are difficult to quantify.

- Rates of groundwater recharge are easy to quantify.

- Recharge may be  by human activities

- One of the following is odd.

- Gravimetric method

- Neutron probe

- Resistance block

- Tensiometer

Part II. Match column “B” with column “A”. 7.5 points.

| Answer |  |  |  |
| --- | --- | --- | --- |
| ----1 | All pore space in the soil is filled with water | A | Dry Bulk Density |
| ----2 | Soil water content when plants or crops cannot obtain water from the soil. | B | Field Capacity |
| -----3 | The oven-dried weight of soil in a known volume of field-extracted sample | C | Permanent Wilting Point |
| ----4 | Soil water content after water has drained by gravity | D | Available (Usable) Water |
| ----5 | The soil water content between field capacity and permanent wilting point. | E | Saturation |
|  |  | F | Bulk density |
|  |  | G | porosity |
|  |  | H | ground water recharge |

Test III: Short Answers. Explain briefly.  10 points

- Define the following terms. 3points

- Water source

- Surface water source

- Ground water source

- List at list three potential water source .2pts

- How do you utilize ground water for different purpose?2pts

- What is the difference between ground water source and surface water source?3pts

Note: Satisfactory rating – 25 points           Unsatisfactory - below 25 points

## Operation Sheet -1

  - Techniques of identifying potential Irrigation surface Water sources.

- Tools and equipment’s

- PPE

- Note book

- PEN

- Procedures

- Make a reconnaissance survey

- Observe  vegetation  covers, moisture content of the soil and topography

- Determine the amount  and pattern of rain fall

- Determine the quality of water that satisfy crop water requirement

- Determine the amount of water that gets from surface  sources

- Identify surface water sources, like river, lake, pond, reservoir etc.

| ### LAP TEST - 1 |  |
| --- | --- |

Name……………………………….           ID……………………………..                                                     Date…………………………………….

Time started: ________________________ Time finished:  ________________

Instructions: Given necessary templates, tools and materials you are required to perform the following tasks within 1 hour.  The project is expected from each student to do it.

Task 1. Identify potential Irrigation surface Water sources

| # LG #2 | # LO #2 - Water harvesting techniques |
| --- | --- |

## Instruction sheet - 2

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Water harvesting principles - Surface and ground water hydrology - Site selection for water harvesting - Identification of water harvesting techniques - Selection of shade & lining materials This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Understand water harvesting principles - Understand surface and ground water hydrology - Select appropriate site for water harvesting - Identify water harvesting techniques - Select shade & lining materials

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks - Perform Operation Sheets - Do the “LAP test”

## Information Sheet - 2

  - Water harvesting principles

The basic principle of water harvesting is to capture precipitation falling in one area and transfer it to another, thereby increasing the amount of water available in the latter. It can be define as is capturing potentially damaging rainfall runoff and translate this into plant growth or water supply. This makes clear sense where rainfall is limited, uneven or unreliable with pronounced dry spells. Yet despite these rainfall limitations, runoff occurs due to high intensity showers and the low water holding capacity of fields, pastures, and forests. With the impacts of climate change already with us, here is an approach to better use a local resource for livelihood sustenance.  Both large and small scale water harvesting techniques follow the same principles to create the most effective ways of capturing water.

The two main requirements for a roof to be used for Rooftop WH are:

- A roof must be easy to connect to gutters and there must be some method of attaching the gutters under the roof.

- The water that comes from the roof should be free of contaminants, particularly dissolved material.

- Procedures of rain water harvesting principles

- Begin with long and thoughtful observation. Use all of your senses to see where the water flows and how. What is working, what is not? Build on what works

- Start at the top or highpoint of your watershed and work your way down. Water travels downhill. Start at the top where there is less volume and velocity so it is easier to follow the next principle.

- Start small and simple. Work at the human scale so you can build and repair everything. One thousand small are far more effective than one big one when you are trying to infiltrate water into the soil.

- Spread and infiltrate the flow of water. Rather than having water run erosively off the land’s surface, encourage it to stick around, walk around, and infiltrate into the soil.

- Always plan for an overflow route, and manage that overflow water as a resource. Always have an overflow for the water in times of extra-heavy rains, and use that overflow as a resource.

- Maximize living, organic groundcover. Create a living sponge so the harvested water is used to create more resources, while the soil’s ability to infiltrate and hold water steadily improves.

- Maximize beneficial relationships and efficiency by “stacking functions.” Get your water-harvesting strategies to do more than hold water. Berms or swales can double as high and dry raised paths. Plantings can be placed to cool buildings. Trees can be selected to provide food.

- Continually reassess your system: the “feedback loop.” Learn from your work. We begin again with the first principle.

Figure 2.1 Principle of surface catchment area

2.1.1. Water harvesting for sustainable land management

Water harvesting must be seen as an integral part of sustainable land and water management (SLM).  SLM is the use of land resources, including soils, water, animals and plants, for the production of goods to meet changing human needs, while simultaneously ensuring the long-term productive potential of these resources and ensuring their environmental functions.

2.1.2. Water harvesting for water resource management

End users manage water according to different strategies and principles, depending on the amount of rainfall, potential evapotranspiration and the cropping system (or other use of water). Four different water management strategies can be recognized:

- Management of excess water from rainfall or seasonal flooding through controlled drainage and water storage for future use. Most suitable in humid and sub-humid conditions as well as semi-arid and arid conditions (floodwater harvesting).

- Increasing rainwater capture and availability, making use of surface runoff; suitable for dry sub-humid to arid conditions (rainwater harvesting).

- Reducing in situ water loss: improving direct water infiltration and reducing evaporation; soil water conservation practices that prevent surface runoff and keep rainwater in place (e.g. conservation agriculture, level bench terraces, mulching, dew harvesting); suitable for sub-humid to semi-arid conditions (in situ water conservation).

- Increasing water use efficiency (e.g. good agronomic practice, including use of best-suited planting material and fertility management).

2.1.3. Components of water harvesting

The basic components of a water harvesting system are a catchment or collection area, the runoff conveyance system, a storage component and an application area. In some cases the components are adjacent to each other, in other cases they are connected by a conveyance system.

Catchment or collection area: this is where rain in the form of runoff is harvested. The catchment may be as small as a few square meters or as large as several square kilometers. It may be a rooftop, a paved road, compacted surfaces, rocky areas or open rangelands, cultivated or uncultivated land and natural slopes.

Conveyance system: this is where runoff is conveyed through gutters, pipes (in case of rooftop WH) or overland, rill, gully or channel flow and either diverted onto cultivated fields (where water is stored in the soil) or into specifically designed storage facilities.

Storage component: this is where harvested runoff water is stored until it is used by people, animals or plants. Water may be stored in the soil profile as soil moisture, or above ground (jars, ponds or reservoirs), or underground (cisterns) or as groundwater (near-surface aquifers) (Oweis et al., 2012). There, where concentrated runoff is directly diverted to fields, the application area is identical to the storage area, as plants can directly use the accumulated soil water. A great variety of designed storage systems keep the water until it is used either adjacent to the storage facilities or further away.

Application area or target: this is where the harvested water is put into use either for domestic consumption (drinking and other household uses), for livestock consumption, or agricultural use (including supplementary irrigation).

2.3. Surface and ground water hydrology

Hydrology is defined as the science that deals with the depletion and replenishment of our water resources. It deals with surface as wells as ground waters, as far as their occurrence, circulation, distribution, chemical and physical properties, reactions to environment and living beings etc. are concerned. Most of the earth’s water sources, such as, rivers, lakes, oceans and underground sources, gets their supplies from the rains, while the rain water in self, is the evaporation from the sources.

Water is lost to the atmosphere as vapor from the earth, which is then precipitated back in the form of rain, snow, hail, dew, sleet or frost etc. This evaporation and precipitation continues for ever and thereby, a balance is maintained between the two this process is known as hydrological cycle.  Hydrologic cycle: the endless circulation of water between the atmosphere, the land surface and the ocean. The notion of the hydrologic cycle provides a framework for understanding the occurrence and distribution of water on the earth.

Surface water is the water occurs on the ground surface from rains, rivers, ponds, lakes etc. that is used for domestic and irrigation purposes. It is the main source of water and it also recharges the underground water through seepage, percolation etc.

Ground water is the underground water that occurs in the saturated zone of variable thickness and depth, below the earth’s surface. Cracks and pores in the existing rocks and unconsolidated crystal layers, make up a large underground reservoir, where part of the precipitation is stored.  The ground water is utilized through wells and tube wells. Various lifting devices, such as those animals, manual, wind, diesel or electric power can be used for lifting water from wells.

The use of open wells is a traditional method of tapping ground water in areas where ground water table is high. The use of tube wells, however, is a subsequent development in the techniques of tapping ground water, and certainly requires diesel or electric power.  The rainfall that percolates below the ground surface passes through the voids of the rocks and joins the water table. These voids are generally inter-connected, permitting the movement of the ground water. But in some rocks, they may be isolated and this preventing the movement of water between the interstices. The mode of occurrence of ground water therefore, depends largely upon the type of formation, and hence depends upon the geology of the area.

Figure 2.2 Hydrological cycle

2.4. Site selection for water harvesting

The site selected for a water-harvesting system will significantly affect the installation costs, performance, and utilization of the facility.  Prior to final site selection and design of the catchment and storage tanks, consideration should be given to all possibilities to make sure that lowest cost and most effective system components are not overlooked. Simultaneous consideration must be given to Construction site, the catchment construction materials, the type of storage tank, and all other elements involved in the system design.  Factors to consider when selecting a site are:-

- Alternate Water Sources

Permanent water sources will obviously be considered in deciding the approximate location of a water-harvesting site. Intermittent sources should also be considered.  Intermittent springs or streams or earthen stock ponds filled by runoff from high intensity storms may be able to supply some of the water requirements during some part of the year or grazing season. Utilizing these temporary sources of water can, in some places, allow installation of a smaller water-harvesting system. If the watering troughs connected to the water-harvesting systems can be turned off when alternate water sources are available, the catchment-supplied water can be saved for periods when the intermittent sources are dried up. This practice is of major importance during periods of extended drought. All possible water sources should be evaluated with respect to number, location, dependability, and potential quantity.

- Forage: The type and quantity of available forage, coupled with the grazing management plan, are major factors in site selection. The water-harvesting system is normally placed in areas inadequately used because of the prior absence of animal drinking water supplies to improve the distribution of the grazing animals and achieve more uniform utilization of forage. In some areas, remote watering troughs, gravity fed by pipelines from the storage, can be used to provide animal dispersion. This technique must be used with caution, because field experience has shown that extended pipelines to remote watering troughs are susceptible to mechanical failure, causing the loss of the stored water.

- Soils: Selection of the general area for the water-harvesting system is initially based on forage availability and alternate water-source factors. Frequently, a catchment treatment is desired that requires a specific soil type. The site is then selected using soil type as one of the main criteria. If the required soil type is not present within the general area, the catchment treatment must be changed to a type compatible to the soils at the site. The soil type and depth also affect the kind of water-storage structure that can be installed. For example, if insufficient soil depth and rocks prevent the installation of a partially buried tank near the catchment, an above-ground tank may have to be constructed at some distance from the catchment.

- Topography: In addition to specific soil types, some catchment materials have slope angle and length limitations. Premature treatment failure can occur if the treatment is installed on an unsuitable slope. If the slope requirements of a desired material cannot be met, the catchment site must be changed, or a different material must be used for catchment construction.

The effort required to prepare a catchment site can often be minimized by locating the catchment to take advantage of natural surface topography. Shallow natural depressions draining to a central location are one such topographic feature that is usually desirable. The catchment-to-storage distance can be minimized if the slope increases immediately below the catchment area. This allows the tank to be placed closer to the catchment, which reduces the length of catchment-to storage conveyance.

- Site Accessibility: The accessibility of the site for construction equipment is a factor of major economic importance. Sites that are difficult to reach, or that require a four-wheel drive vehicle increase the cost of equipment and labor required and may limit the types of materials that can be used in the installation. In very rough country, the movement of livestock will be restricted by poor site location.

- Precipitation: In some locations, local topographic features, such as hills or mountains, can affect precipitation patterns over relatively small distances. When possible, rain shadow areas should be avoided. If significant quantities of the precipitation occur as snow, locating the catchment apron where the snow will accumulate in drifts, or the installation of snow fences to promote drift accumulation on the catchment, may be desirable.

- Potential Expansion of System

The final consideration in site selection is to include the possibility of a future expansion of the system to provide additional quantities of water. The water requirement may be increased by changes in the grazing management plan to increase livestock numbers or by the installation of pipelines to remote watering troughs that increase the area served.

  - Identification of water harvesting techniques

In most cases the water harvesting techniques fall under four basic categories whose main characteristics are summarized as follows:

A. Rooftop WH

- Main characteristics of rooftop WH

- Consists of roof, gutters, ‘first flush’ device and above or below ground storage tank;

- Useful in areas with rainfall between 200 and 1000 mm.

- Especially good in areas with two separate rainy seasons (bimodal)

- High runoff coefficient (0.5 to 0.9);

- Mainly used for domestic purposes;

- May recharge groundwater if an infiltration well is built;

- Quality of water can be controlled by flushing away the

- First collection (from a dirty roof) filtration and simple disinfection techniques; collected water is normally acceptable both in terms of taste and appearance;

- Provides water next to homes;

- The user has control.

B. Micro catchments rainwater harvesting

- Main characteristics

- sheet and rill flow harvested from short catchment length;

- runoff stored in soil within field;

- catchment length usually between 1 and 30 meters;

- size of individual catchment 10 to 1000 m2;

- catchment: storage area ratio 1:1: 10:1;

- relatively high runoff coefficients, higher than macro-catchment systems;

- catchment area generally bear with sealed, crusted and compacted soils;

- system replicated many times with identical design within the same field;

- no water conveyance system needed;

- no provision for overflow;

- easily replicable and adaptable;

- suitable for most crops, planted in pits or strips within field;

- needs fertility management;

- Land user has control within his farm over catchment and the application area.

- Typical examples:

- Pits and basins

- micro-basins: e.g. negarims,

- small semi-circular bunds,

- eyebrow terraces,

- Contour Bunds (for trees)

C. Macro-catchment rainwater harvesting

- Main Characteristics:

- Overland flow or rill flow harvested

- Runoff stored in soil profile

- catchment usually 30 - 200 meters in length

- ratio catchment: cultivated area usually 2:1 to 10:1

- provision for overflow of excess water

- uneven plant growth unless land leveled

- Typical Examples:

- Trapezoidal Bunds (for crops)

- Contour Stone Bunds (for crops)

D. Floodwater farming (floodwater harvesting)

- Main Characteristics:

- turbulent channel flow harvested either (a) by diversion or (b) by spreading within  channel bed/valley floor

- referred to as "Water Spreading" and sometimes "Spate Irrigation"

- runoff stored in soil profile

- catchment long (may be several kilometers)

- ratio catchment: cultivated area above 10:1

- provision for overflow of excess water

- Typical Examples:

- Permeable Rock Dams (for crops)

- Water Spreading Bunds (for crops)

  - Selection of shade & lining materials

The approximate location of the construction site is determined by the area in which additional water is needed. Ordinarily, the lowest cost storage tank and catchment construction materials should be selected and a site located that permits their use. Frequently, a survey of the available sites will show that the actual choices are limited.

The main advantage of using shades and lining materials for water harvesting structures is to reduce evaporation and seepage loss respectively. Reducing evaporation is an important way to increase the supply of water. Therefore, the following measures should be taken:  The storage tank should be covered with appropriate roofing materials that inhibit vaporization. Surface area of the storage tank should be minimizing to reduce the cost of tank roofing.

    - Types of Lining

- Soil:

- Fly Ash

- Masonry (stone, rock, brick)

- Concrete (Portland cement)

- Non - reinforced concrete

- Reinforced concrete (with steel)

- Plastic

- Polyvinyl Chloride (PVC)

- Oil Resistant PVC

- Chlorinated Polyethylene (PE)

- Low Density

- Polyurethane foam with or without coatings

- High density

- Asphalt (bituminous)

- Sprayed (“blown”) asphalt

- Asphaltic Concrete

- Synthetic Rubber

- Butyl

- Neoprene Rubber

- Shot Crete over geosynthetic

- Concrete over geosynthetic

    - Advantages of Lining

- To save water (reduce Seepage)

- To stabilize channel bed And banks (reduce Erosion)

- To avoid piping through And under channel Banks

- To decrease hydraulic Roughness (flow Resistance)

- To promote movement, rather than deposition, of sediments

- To avoid waterlogging of adjacent land

- To control weed growth

- To decrease maintenance costs and facilitate cleaning

- To reduce excavation costs (when extant material is unsuitable)

- To reduce movement of contaminated groundwater plumes

| ### Self-Check – 2 |  |
| --- | --- |

Name……………………………………………   ID………………………… Date…….

Directions:  Answer all the questions listed below.

Test 2: Match column “A” with column “B”.  7.5 points

| Answer |  |  |
| --- | --- | --- |
| --------1. | Slope of a mountain or hill | - Rooftop WH |
| ---------2. | Turbulent channel flow harvested either by diversion or by spreading within channel bed/valley floor. | - Courtyard WH |
| -------3. | Method of collecting surface runoff/ sheet (and sometimes rill flow) from small catchments of short length. | C. Micro catchments rainwater |
| --------4. | Harvesting of rainwater can be from roofs of private, public or commercial buildings | - Macro-catchment rainwater |
| --------5. | Collected from compacted, paved surfaces or where plastic sheeting has been laid out. | - Flood water farming |
|  |  | - Irrigation method |
|  |  | - Water harvesting catchment area |
|  |  | - Principles of WH |

Test II: Multiple choice. 5 point

- One of the following is wrong about water harvesting.

- The principles of water harvesting is translating runoff in to plant growth.

- Water harvesting is significant for dry spells.

- Water harvesting is advantageous in an area where rain fall is uneven and unreliable.

- Water harvesting is advantageous in an area where rain fall is even and reliable.

- One of the following is an example of macro-catchment water harvesting.

- Eyebrow

- Semicircular bund

- Trapezoidal Bunds

- Rooftop WH

- Which one of the following is not the advantage of shading and lining materials for water harvesting structures?

- Reduce Evaporation

- Reduce Seepage Loss

- Increase Infiltration Rate

D. Increase the amount of water collected

- One of the following is the main characteristics of courtyard WH.

- consists of roof, gutters, ‘

- Useful in areas with rainfall between 200 and 1000 mm.

- mainly used for domestic purposes

- Mainly used for domestic and livestock consumption.

- Which one of the following is odd?

- Negarims

- Eye brow

- Planting pit

- Stone bund

Note: Satisfactory rating - 12points           Unsatisfactory - below 12 points

You can ask you teacher for the copy of the correct answers.

Note: Satisfactory rating - 12points           Unsatisfactory - below 12 points

You can ask you teacher for the copy of the correct answers.

You can ask you teacher for the copy of the correct answers.

## Operation Sheet -2

- Techniques/Procedures/Methods of  identifying  water harvesting techniques

- Tools and equipment’s

- Take on PPE

- Note book

- Pen

- Procedures/Steps/Techniques

- Examine the types of surface used.

- Examine area coverage of the  catchment

- Examine the slop of the area

- Examine types of storage

- Understand  availability and amount of rain fall in the area

- Based on the above information identify types of water harvesting techniques,   such as rooftop water harvesting, micro water harvesting, macro water harvesting and floodwater harvesting.

- Techniques/Procedures/Methods of  identifying proper site for water harvesting

- Tools and equipment’s / Materials

- Take on PPE

- Note book

- Pen

- Procedures/Steps/Techniques

- Observe the area

- Examine the slope of the area

- Select  smoothing or compacting the soil surface

- Select  area which is free from  rock surfaces

- Select impermeable surface

| ### LAP TEST-2 | Performance Test |
| --- | --- |

Name……………………………….           ID……………………………..                                                     Date…………………………………….

Time started: ________________________ Time finished:  ________________

Instructions: Given necessary templates, tools and materials you are required to perform the following tasks within 3 hour. The project is expected from each student to do it.

Task-2 Identify water harvesting techniques

Task -3 Identify proper site for water harvesting

| # LG #3 | # LO #3- Catchment area |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Characteristics of catchment area and climatic variables - Environmental issues, guidelines and legislation of water harvesting - OHS hazards This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Characteristics of catchment area and climatic variables - Environmental issues, guidelines and legislation of water harvesting - OHS hazards

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks - Perform Operation Sheets - Do the “LAP test”

## Information Sheet - 3

- Catchment Area

A catchment area is the area from which a particular river or lake receives both surface flow and drainage water originating from precipitation. It is an area where water is collected by the natural landscape. In a catchment, all rain and run-off water eventually flows to a creek, river, lake or ocean, or into the groundwater system. Natural and human systems such as rivers, bush land, farms, dams, homes, plants, animals and people can co-exist in a catchment.

Figure 3:1 Small water catchment area

  - Types of catchment area

- Roof catchment area: Roof water harvesting systems gather rainwater caught on the roof of a house or other areas using gutters and down-pipes (made of local wood, bamboo, galvanized iron or PVC) and lead it to one or more storage containers ranging from simple pots to large tanks.  Rainwater can be collected from most forms of roof. Tiled or sheeted roofs, of corrugated mild steel, are preferable being the easiest to use and give the cleanest water. Asbestos sheeting or lead painted surfaces should be avoided if possible due to health problems.

Roof top rain Water harvesting and conserving System, both for small and large areas, comprise of six basic components as described below:

- Catchment area/roof: surface upon which rain falls.

- Gutters and Downspouts: transport channels from catchment surface to storage.

- Leaf Screens and Roof Washers: systems that remove contaminations and floating debris.

- Storage Tanks: where collected rain water is stored for desired period.

- Water Treatment: filters and equipment and additives to settle flocks, filter and disinfect.  Conveying/transporting system for treated rain water, either by gravity or pump.

    - Identifying and characterizing catchment areas for climatic variable

- Selection of the catchment area  based on:-

- The catchment area must collect sufficient runoff water should have low infiltration

- The runoff water collected from the catchment area should be easily diverted to the tank.

- The catchment area should be located sufficiently away from pollution Sources

- The catchment must generate as little sediment as possible.

- Selection of the cropping area is also based on its infiltration characteristics, water storing capacity and its nutrients.

- Cropping areas should have: Selection for the cropping area is also based on its infiltration characteristics, water storing capacity and its nutrients. Cropping areas should have:

- high infiltration

- low runoff

- high water storing capacity

- good plant nutrition

- General Soil requirements in the catchment areas and the cropping areas

Catchment area:

- Macro catchments systems

- clayey / loamy soil with some clay and silt, sodic soils and rock surfaces

- compact structure

- shallow soils (<1m)

- Micro catchment systems

- Medium texture, deep soil, crust-forming capability

- Earth works

- Stable soil types not subject to piping

- No soils which crack on drying high proportion of montmorillonite clay

- No erodible soils (sandy soils, silty-sandy soils or soils with very poor structure)

- Cropping areas

- medium texture /infiltration/

- good structure (high organic matter) 1.0- 2.5 m deep

- Nutrient-rich, low salinity.

    - Climatic variability

Climate change is believed to have a large impact on water resources system both globally and regionally.

Rainfall variability affects RWH to a large extent because the variability can lead to deficiency or surplus in rainwater in a particular season or year, which will in turn affect people's domestic water needs. Scarcity of water is the most severe constraint for traditional agriculture in semi-arid areas of Ethiopia. Precipitation is extremely variable, thus water harvesting is crucial for ensuring improved crop production. Extreme climate events such as:

- aridity,

- drought,

- flood,

- cyclone and

- Stormy rainfall are expected to leave an impact on water harvesting technology.

3.2. Environmental issues, guidelines and legislation of water harvesting

The construction and operation of storm water harvest and use systems can pose potential risks from the pollutants and toxins found in storm water and harvest and use system materials. These risks are largely addressed via water quality standards, plumbing and building codes, storm water rules and regulations, required signage, and the engineering review process. Storm water harvesting is, however, an emerging practice in water resource management and existing regulations may not fully address the risks associated with harvest and use practices.

Risk assessment should be completed during the pre-design phase to ensure that potential risks are properly managed through system design, operation and maintenance. There are two types of risk assessments:

- Human health risk assessment, which is the process to estimate the nature and probability of adverse health effects in humans who may be exposed to chemicals in contaminated environmental media, now or in the future.

- An ecological risk assessment, which is the process for evaluating how likely it is that the environment may be impacted as a result of exposure to one or more environmental stressors such as chemicals, land change, disease, invasive species and climate change. The following factors should be considered when assessing human health and ecological risks of storm water harvesting and use systems.

- Geology and climate of the harvesting site

- Potential environmental hazards located within the rainwater/storm water source area

- Source water quality

- Potential exposure to harvested water (indoor/outdoor, restricted access, unrestricted access)

- End use of harvest water

3.2.2. Guidelines and legislation of water harvesting

Currently, rainwater harvesting is not regulated by the federal government but rather it is up to individual states to regulate the collection and use of rainwater. There is no centralized information on state-level regulations on rainwater harvesting maintained by a federal agency or outside organization. Information gathered through the research revealed interesting findings, such as:

- A number of states do not prohibit rainwater harvesting but do not have regulations protecting it, while some of these states have technical resources on rainwater harvesting to encourage the collection and use.

- Some states regulate size of storage capacity.

- Several states address rainwater harvesting in plumbing codes.

- Generally, states with rainwater harvesting regulations allow both commercial and residential application.

- There are local and county agencies that regulate rainwater harvesting that may impact potential project development, which needs to be carefully considered, especially for a large commercial operation.

- Designations of rainwater harvesting regulations at different state.

- Very Limited: The state has limited exemptions available for legal implementation of rainwater harvesting.

- No Regulations: No regulations are in place prohibiting rainwater harvesting

- No Regulations/Encouraged: No regulations are in place prohibiting rainwater harvesting and state specific rainwater harvesting technical resources are available, such as handbooks and guidelines on rainwater harvesting systems.

- State Regulations: Regulations are in place allowing rainwater harvesting; however, no specific technical resources or incentives are provided.

- State Regulations/Encouraged: Regulations are in place allowing rainwater harvesting and state specific technical resources are available to encourage the collection and use of rainwater.

- State Regulations/Incentives: Regulations are in place allowing rainwater harvesting, state incentive programs are in place, and state-specific technical resources are available.

3.3.   Identify OHS hazards in the working area

Contamination of collected rainwater with disease causing microorganisms (bacteria, viruses and protozoa), from birds and other animals, poses the biggest risk to health. There is always the possibility of inadvertent ingestion while washing hands or inhalation of the microorganisms found in harvested rainwater.  Leaching from roof materials, like lead, copper and other roof materials may also occur when rainwater harvesting. There are many potential health and safety hazards associated with WH.

- Biological hazards

- Risk of respiratory disorders, including lung disorders from inhaling moulds.

- Exposure to zoonotic diseases from various sources, including animals (e.g., toxoplasmosis, Q fever, rabies), poultry (e.g., histoplasmosis, psittacosis), insects, rodents (e.g., hantavirus), soil, manure, etc.

- Needlestick or sharps injuries.

- Chemical hazards

- Exposure to hazardous products including fuel, lubricants, antifreeze, fertilizers, pesticides, herbicides, insecticides, veterinary preparations, paints, varnishes etc.

- Exposure to hazardous gases in and around manure pits that may cause asphyxiation and/or explosions.

- Exposure to hazardous gases in silos during grain storage, as well as the risk of engulfment by flowing grain or loose materials such as gravel (see safety hazards and risks in confined spaces).

- Fire and explosion hazards, including combustible dusts and decaying manure.

- Ergonomics hazards

- Standing for long hours.

- Sitting for long hours, especially on moving or vibrating vehicles.

- Working in awkward postures, performing repetitive tasks, lifting.

- Fatigue from working long hours or shifts.

- Physical hazards

- Exposure to excessive noise.

- Work in extreme conditions, including temperature.

- UV radiation.

- Psychological hazards

- Stress.

- Working alone.

- Safety hazards

- Working at heights, such as falls from heights, down shafts, etc.

- Slips, trips and falls from wet, untidy and obstructed floors.

- Risks from animals including kicks, bites, steps, strikes, being squeezed against a wall or fence, etc.

- Various hazards from machinery and equipment (e.g., tractors, pickups, ATVs, front-end loaders, implements, attachments/power take-offs, chainsaws, etc.).

- Risk of accidental start up when working on equipment or machinery.

- Working with sharp blades or edges

- Working in or near confined spaces

| ### Self-Check – 3 |  |
| --- | --- |

Name……………………………………………   ID………………………… Date…….

Directions:  Answer all the questions listed below.

Test 3: Match column “A” with column “B”.  5 points

| Answer |  |  |
| --- | --- | --- |
| --------- | - Extreme climate events | - Rodents (e.g., hantavirus) |
| --------- | - Exposure to zoonotic diseases | - Drought |
| ---------- | - Stress | - Psychological hazards |
| -------- | - Allowing rainwater harvesting, | - State regulations/Incentives: |
| ---------- | - Prohibiting rainwater harvesting | - No regulations |
|  |  | - Catchment area |
|  |  | - Cropping area |
|  |  | - precipitation |

Note: Satisfactory rating - 5points           Unsatisfactory - below 5 points

You can ask you teacher for the copy of the correct answers.

Note: Satisfactory rating - 5points           Unsatisfactory - below 5 points

You can ask you teacher for the copy of the correct answers.

You can ask you teacher for the copy of the correct answers.

## Operation Sheet -3

3.1. Techniques/Procedures/Methods of identifying catchment area

- tools and materials

- PPE

- Note book

- Pen

- Procedures/Steps/Techniques

- Conduct field Assessment

- Assesse the boundary of the catchment

- Observe land cover of the catchment

- Examine the slope of the catchments

- Estimate  the area of the catchments

- Estimate the amount of runoff  generated from the area

- Decide the types of catchment area.

| ### LAP TEST-3 | ### Performance Test |
| --- | --- |

Name……………………………….           ID……………………………..                                                     Date…………………………………….

Time started: ________________________ Time finished:  ________________

Direction: - you have to make environmental scanning. This leads you to examine different types of   catchment area in your surroundings. Ask your instructor to help, if you need.

Instructions: Given necessary templates, tools and materials you are required to perform the following tasks within 1 hour. The project is expected from each student to do it.

Task -3 Identify catchment area.

### Reference Materials

Books:

African Development Bank. 2009. Rainwater Harvesting Handbook: Assessment of Best Practises and Experience in Water Harvesting. African Development Bank.

Alvord ED. ;( 1978); Development of native agriculture and land tenure in southern Rhodesia, Unpublished mos. Harare: Rhodes House, University of Zimbabwe.

Barrow C (2013); Design and construction of water harvesting schemes for plant production;Lowa State University, USA.

Bruins P.N (1986); Evaluation of the benefits Bratton M. (1987); Drought, food and the social organization of small farmers in Zimbabwe.

Chow V.T., 1964, (E.d) hand book of Applied Hydrology, MCGraw-Hill, New York,NY.

In: Glantz M, editor. Drought and hunger in Africa denying famine a future. New York: Cambridge of infiltration pits on soil moisture in semi-arid area; Michigan University.

Santosh Kumar garg. 2005. Irrigation engineering and hydraulic Structures, Naraina, Delhi .

Vander leeden. 1975. F,water Resources of the world, water information center, port Washington, N.Y, USA.

Web addresses

https://www.canr.msu.edu/uploads/235/67987/presentations/Water_Sources_for_Irrigation.pdf (accessed date, 28/08/2022).

https://www.fao.org/3/U5835E/u5835e03.htm (accessed date, 0822).

http://www.waterandnature.org/sites/default/files/national_irrigation_act2013.pdf (accessed date, 28/08/2022).

https://www.lawinsider.com/clause/suitability (accessed date, 28/08/2022).

https://pubs.usgs.gov/circ/circ1139/pdf/circ1139.pdf (accessed date, 29/08/2022).

https://repository.ruforum.org/documents/characterization-climate-variability-and-water-harvesting-system-crop-production-adulala (accessed date, 29/08/2022).

https://en.wikipedia.org/wiki/Irrigation (accessed date, 28/08/2022).

http://eagri.org/eagri50/AGRO103/lec04.pdf (accessed date, 29/08/2022).

- AKNOWLEDGEMENT

Ministry of Labor and Skills wishes to extend thanks and appreciation to the many representatives of ATVET instructors and respective industry experts who donated their time and expertise to the development of this Teaching, Training and Learning Materials (TTLM).

The experts who developed the learning guide

| No | Name | Qualification | Organization/ College | Mobile number | E-mail |
| --- | --- | --- | --- | --- | --- |
| 1 | Serawit Gensa | Msc. Water Resource Engineering | W/Sodo AVET | 0916740916 | serawitgen@gmail.com |
| 2 | Edao Hassen | Msc.Irrigation Engineering | Alage AVET | 0911098097 | hassedao@gmail.com |
| 3 | Mekete Agizew | Msc. Water Resource Engineering | Amahara. Kombolcha ATVET | 0925221192 | gen.mam09@gmail.com |
| 4 | Wondu Alemayehu | Msc.Irrigation Engineering | Oromia Kombolcha ATVET | 0910-28-99-61 | woldualem@gmail.com |
| 5 | Ademe Ayalew | Msc.Irrigation Engineering | Agrafa ATVET | 0912720547 | ademe2004@gmail.com |
| 6 | Seid Mohammed | Msc.Irrigation& drainage Engineering | Alage ATVET | 09-17-18-01-81 | Siyamsdmhmmd@gmail.com |
| 7 | Molalign Asfaw | Bsc.Water Resource &Irrigation Engineering | Alage ATVET | 0921431096 | Mollalign410ass@gmail.com |
| 8 | Yonas Hailu | Bsc.Water Resource &Irrigation Engineering | Agrafa AVET | 0934715578 | yonashailuw@gmail.com |
| 9 | Lemessa Mulata | Msc.Irrigation Engineering | Agrafa AVET | 0913266845 | Lamimulle2022@gmail.com |
| 10 | Misganew Yimer | Bsc. Soil and Water Engineering | Woreta ATVET |  | Misge1976@gmail.com |
| 11 | Daniel Derese | Bsc. Soil and Water Engineering | W/sodo AVET | 0912-79-28-85 | danielderesse7@gmail.com |
| 12 | Teshome Getachew | Msc.Irrigation & drainage Engineering | Alage ATVET | 0925-50-13-99 | teshomegetachew131@yahoo.com |