Irrigation and Drainage

Level-I

Based on March 2022, Version 3 Occupational standard

Module Title: - Manual Excavation

LG Code: AGR IRD1 M10 LO (1-6) LG (39-44)

TTLM Code: AGR IRD1 TTLM 0822v1

August, 2022

Addis Ababa, Ethiopia

Table of Contents

Contents

Introduction to the Module	1

LO #1-Perform Manual excavation	2

Instruction sheet	2

Information Sheet 1	3

Self-check 1	16

LO #2-Work Site Preparation	18

Instruction sheet	18

Information Sheet 2	19

Self-check 2	33

LO #3- Small Excavations by Hand	35

Instruction sheet	35

Information Sheet 3	36

Self-check 3	52

Operation Sheet -3	54

LAP TEST-3	55

LO #4- Excavation site isolate	56

Instruction sheet	56

Information Sheet 4	57

Self-check 4	61

LO #5- Restoring work site	62

Instruction sheet	62

Information Sheet 5	63

Self-check 5	66

Operation Sheet -5	67

LAP TEST-5	68

LO #6- Record and Reporting activity	69

Instruction sheet	69

Information Sheet 6	69

Self-check 6	74

Operation Sheet -6	75

LAP TEST-6	76

Reference Materials	77

# Introduction to the Module

This unit covers the knowledge, skills and attitudes required to plan and prepare for work, prepare work sites, perform small excavations by hand, complete and isolate the excavation and clean up prior to work and restore them on completion of work.

Excavation work generally means work involving the removale of soil or rock from a site to form an open face, hole or cavity using tools, machinery or explosive. A person conducting a business or undertaking must manage risk associated with all kind of excavation at the work place, no matter how deep. Specific duties apply in relation to the higher-risk excavation, such as trenches, shafts and tunnels.

| LG #39 | # LO #1-Perform Manual excavation |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Tools, equipment and materials - Personal protective equipment - Site preparation - Public and environmental risk identification - Legislative and organisational requirement This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - select tools, equipment and materials - Select personal protective equipment selection - prepare site - Identify Public and environmental risks - Identify legislative and organisational requirements

Learning Instructions:

- Read the specific content of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks

## Information Sheet 1

Introduction

- Material

- Is anything used to making something else.

- It may include construction and backfill materials.

- Equipment

- Is the thing needed for particular purpose

- It includes pumps, motors, flow meters, etc.

- Tools

- Is any physical item that is used to achieve a goal

- Is a device intended to make a task easier

- Is something which is handled to perform operation

- Is multi-purposes

- Personal protective equipment (PPE)

- Is used to protect an individual from hazards associated with their work tasks or environment.

- Site preparation

- site preparation is also defined as a set of activities and tasks before engineers commence work

  - Identifying the required tools, equipment and materials

Different kinds of tools and equipment are required to carry out manual excavation:

Table 1.1 lists of tools and equipment

| Name of tools, equipment and function | Picture |
| --- | --- |
| - Pick axe: - a tool for breaking hard surfaces, with a long wooden handle and a curved metal bar with a sharp point. | Fig 1.1 Different types of Pick axe |
| - Spade: is a tool which consists metal plate having sharp edges, the plate is attached to long handle which is generally made up of wood | - Fig 1.2 spade |

| - Shovel: is tool which is used for the purpose of lifting of excavated soil. It is also similar to spade the difference between spade and shovel is the difference in leading edge. The curvature of metal plate of shovel is generally higher when compared to spade so we can hold the soil easily and lifted it. Shovel can also be used for digging purpose in case of soft soils, sand etc. | Fig 1.3 Different types of Shovels |
| --- | --- |
| - Pegs: are used to make fast a rope or string on, to stop a hole, or to mark some point. | Figure 1.4 Different kinds of pegs |
| String line: is one of the oldest and most basic of hand tools, and it is commonly used in masonry and carpentry to help the user create a straight line between two reference points. | Fig 1.5 String lines |
| - Spirit Levels A spirit level, bubble level or simply a level is an instrument designed to indicate whether a surface is horizontal (level) or vertical (plumb). | - Fig 1.6 Different types and size of spirit levels |
| Hammers are used nails or hooks into walls or wood (small hammers). Rubber hammers are used to level tiles. If you want to break down walls and concrete structures you need a sledge hammers. | Fig 1.7 Hammers |
| Tap meter: used two measure distance between two points (eg. distance between point A and point B). | - Fig 1.8 Tape measures |
| Wheelbarrow: for transporting all kinds of materials concrete, stone, sand, soil and other, etc | Fig 1.9 Wheelbarrows |
| Crowbars: are commonly used to open nailed wooden crates. Common uses for larger crowbars are: removing nails, prying apart boards, and generally breaking things. | - Fig 1.10 Different types Leveling equipment |
| Jackhammers:- are used to demolish old concrete, remove pavement, and demolish many other surfaces in projects. The jackhammer itself is heavy, so, only appropriate personnel should handle the tools to reduce the risk of accidents | Fig 1.11 Survey Leveling Rod |

1.2. Selecting Suitable personal protective equipment (PPE)

Personal protective equipment’s (PPE): are devices worn, put on, tied on, or inserted in such as hearing protectors inserted in the ears of workers for protection against industrial hazard health hazards and accidents.

Personal protective equipment’s are used:

- As supplementary means of protection, and

- As direct means of protection

Table 1.1   list of personal protective equipment

| - Name of Personal protective equipment | Picture |
| --- | --- |
| Foot protection:- Potential hazards which may lead to foot and leg injuries include falling or rolling objects, crushing or penetrating materials, hot, corrosive or poisonous substances, electrical hazards, static electricity, or slippery surfaces. | Fig 1.12 foot protection |
| Safety Helmet: Used to protect the head from injury | Figure 1.13 helmet |
| Eye protection:-Used to protect the eye from injury | Fig 1.14 eye protection |
| Hand protection:-Used to protect the hand from injury | Fig 1.15 hand protection |
| Overalls: Hazards that affect the entire body can include temperature extremes, chemical exposure, radioactive materials, arc flash, infectious materials, sparks or flames, falls and sharp objects. | Fig 1.16 overalls |
| - Respiratory protection - Wearing respirators to protect themselves against the inhalation of harmful air contaminants in the work place. | - Fig 1.17 respiratory protection |
| Ear muff:- It is required to protect the ear from dangerous sound and dust. | Fig 1.18 ear muff |

1.3. Site Preparation

Site preparation is an activity that should be done accordingly for other construction activities to commence. The failure and success of site preparation set the condition and the tone for other construction activities executed on-site. The definition of site preparation is already enough to demonstrate the value of this activity to irrigation structure construction. https://www.youtube.com/watch?v=aJ9zekjgY2w  (video of site preparation for excavation).

Furthermore, site preparation is also defined as a set of activities and tasks before engineers commence work.

- Steps Involved in Site Preparation

When site preparation is done correctly, all activities are straightforward. If constructing a irrigation canal needs to be done in a specific place with tons of underground pipes, preparing the site is necessary. By having the site ready, contractors are helped in knowing where to dig and where not to dig. It will prevent any issues related to underground pipe bursting. Implementing project management for construction is equally crucial because it lessens possible mistakes and delays that can lead to costly reworks.

Below are the five necessary steps involved in implementing a site preparation for construction projects.

- Site Clearing

Clearing the site is the very first step in site preparation. The entire site must be in a graded and cleared condition. The case involves the demolishing of buildings, the removal of trees, and eliminating any underground infrastructures. These obstacles should not hinder in the first place because it might affect the future building process. Completing the project might also at risk if clearing the site is not successful. Hence, it must be prioritized once and for all.

- Site Surveying

You might not be sure if you are building on the right block if survey pegs do not determine the building block. A surveyor is responsible for surveying the site and lining out the exact area where the road project or structures to be built. The process for surveying land might not be an option but is mostly a requirement zoning and permitting processes. Besides, surveying is the translation of the contractor’s set of plans into an actual representation of the project site.

- Soil testing

Testing the soil is one of the most critical tasks that have to be done before the site is procured. The soil’s main composition should be determined to test the ability to absorb water and examine the ability to withstand structure. The site engineer must insist on doing all necessary soil testing procedures before commencing any structural tasks.

If the soil at the site isn’t suitable enough for future projects, there might be no other choice but to look for another site that has soil appropriate for the projects. A chart showcasing the different classes of soil is outlined below:

- Class A (Acceptable): mostly the rocks and sand have no little or no ground movement

- Class S (Satisfactory): slightly reactive clay sites that only have little ground movement

- Class M (Moderate): Moderately slit sites or reactive clay that can experience moderate ground movement

- Class H1 and H2 (Highly reactive): Clay sites are highly reactive that it can experience higher ground movement

- Class E (Extreme): Extremely reactive sites that is prone to experiencing extreme ground movement

- Class P (Problem): Sites that include softer soils like a slit or soft clay. Soil is subject to erosion, which cannot be classified otherwise.

- Site Plan Design

After doing the soil testing, all necessary canal and drainage should be installed. The next step will be modifying the design to indicate the placement of partial flume and division box. Additionally, there should also be a permanent record of the areas located underground. A construction site is considered a living and breathing thing. Hence, it changes every single day as the water tank’s placements slightly change. Underground rock formations often cause changes. The use of a crew scheduler superior is proven to be a helpful tool during site plan design.

Moreover, the site plan shows all newly developed access roads for temporary storage areas and construction vehicles. The site plan also depicts where the building must be placed after it is constructed. Unlike any other steps taken during site preparation, revisions are carried out in the office, and the site plan is always updated in the field.

- Site Investigation

Geotechnical site investigation is performed to characterized rock, soil, and groundwater condition of any proposed sites. A geotechnical site investigation is defined as evaluating site conditions and collecting data to construct and design the foundation of a structure. Parking lot, bridges, roads, and buildings are some of the typical samples.

Well-planned management and strategy of a geotechnical site investigation is the primary key in acquiring correct site information. This data is generally used for structure designing with minimum surprises for effort and estimates need. The preparation of reports and collection of geotechnical data must be considered in the following three stages. Take a look below:

- The project definition is usually prepared by the respective owner in conjunction with the architect. Thus, the project definition is expected to consist of engineering/architectural foundation criteria such as settlement and loading.

- The chosen geotechnical consultant must conduct project evaluation and preliminary site for the site investigations. It often consists of initial site reviews of past geotechnical investigations. This consensus by the owners and initial assessment is used to develop the proposed geotechnical site investigations’ details.

1.4. Public and environmental risk identification

The first step in the risk management process is to identify risk which could injure or harm anyone. A good risk identification process is the key to risk management. Identify risk and controls before the work starts. It may not be possible to control all risks before work starts so identify the controls and implement them when required. For example, consider the risk to a person:

- Falling into an excavation

- Being trapped by the collapse of an excavation

- Working in an excavation being struck by a falling object

- Working in an excavation being exposed to contaminants that take oxygen from the environment.

- To manage the risks, consider all relevant matters including the:

- Nature of the excavation

- Nature of the excavation work, including the range of possible methods of carrying out the work means of entry into and exit from the excavation.

- Complete and monitor hazard identification regularly to make sure controls are working and that no new hazards have been introduced.

- Risk Identification Methods

- identify risk by:

- Physical inspections:

- Inspect the workplace and assess where someone could get injured by excavation activities

- Consider hazards that may be created by other site users, or if the excavation activities could create hazards for others (eg traffic management)

- Task analysis: identify the hazards involved in each task. Some workplaces use job safety analysis (JSA) or task analysis (TA) to do this

- Engaging with workers

- Process analysis: identify hazards at each stage of the work plan

- Consulting guidance and standards

- Hazard and operability analysis (HAZOP)

- Accident investigation analysis – identify hazards and causes of harm from investigations involving similar types of work.

- Public risk of excavation

Excavation is one of the preliminary processes for any construction project. The hazards and risks associated with it include the collapse of the earth, injuries to workers due to the failure of the trench, machines falling into improperly excavated areas, etc. These mishaps could prove to be dangerous and life-threatening.

- Environmental risk of manual excavation

The first thing organizations involved in construction need to do is determine the environmental risks that they have to manage. Environmental risks are those risks that are associated directly and indirectly with the construction activities. In the construction sector, the environmental risks can be one or more of the types described in 16.2 above. These can happen during the course of the construction, during the operation, or at the end of operation, as well as in abnormal or emergency situations such as during heavy rain or landslides. These environmental risks should therefore be taken into consideration at every stage of the project implementation, from conception to completion.

- Risk of Land Degradation

Large projects usually entail extensive land disturbance involving the removal of vegetation and reshaping of topography. Such activities make the soil vulnerable to erosion. Soil removed by erosion may become airborne and create dust problem or be carried by water into natural waterways, thereby polluting them. Due to the soil erosion of the exposed and loose earth, there will be a deterioration of water quality in the surrounding water bodies due to siltation. This can result in mud floods and flash floods in immediate or downstream areas during heavy downpours. Landslides and slope failure can occur on unstable slopes or when the soil is saturated with water during heavy rainfalls.

- Risk to Flora and Fauna

The biological environment includes various species of animal and plant life, and their habitats. Unfortunately, however, loss of flora and fauna is likely in any development. Planning is essential to ensure minimal losses during the implementation stages and steps must also be taken later to ensure that the losses are “replenished.” This is crucial especially when development is in the vicinity of a protected or animal sanctuary, a forested area, or a catchment area. Ecological losses should be minimized and suitable protection put in place for the conservation of flora and fauna.

- Risk of Water Pollution

Water quality is important for economic, ecological, aesthetic, and recreational purposes. Changes in water quality may affect its aesthetic value or even prevent some uses of the water. During construction, the potential for soil erosion and risk to water quality is greatest when removal of vegetation for initial clearing and grading activities exposes soil and makes it susceptible to erosion. The impacts and risks are greatest during the rainy season where extensive land clearing can increase sediment load into the rivers from erosion of the exposed soil.

- Risk of Air Pollution

Activities of major concern for air quality are the burning of waste, the emission of fumes and smoke, and the release of chemical impurities such as heavy metals, acid and other toxic bases. Air quality impacts from construction include increased dust particulates in the atmosphere caused by grading, filling, removals, and other construction activities. Air quality may also be impacted by emissions from construction equipment and vehicles.

- Risk from Noise and Vibration

Noise and vibration can be generated by various activities and equipment used in construction projects. Noise and vibration levels due to construction activities in the project area vary depending on the types of equipment used, the location of the equipment, and the operating mode. Adverse impacts resulting from construction noise and vibration are generally limited to areas adjacent to the project, and are temporary in nature.

1.5 Legislative and organizational requirement

- Legislative requirement of work site

Legislation is an important instrument in the control of environmental hazards to health, and has been used from the earliest days of public health, for example in the control of water quality and sanitation in the 19th century, and in various forms of air quality legislation in the 20th century.  Legislation may take many forms, including regulation of emissions that may cause environmental pollution, taxation of environment- and health-damaging activities, and establishing the legal framework for trading schemes (e.g. for carbon emissions).  Other actions may rely on voluntary agreements.  Among major current legislative frameworks are those relating to environmental permitting, and those mandating environment and health impact assessments.

Regulation has a key role in controlling environmental pollution and the associated risks to human health.  Among its different forms are:

- Direct regulation – which allows control of emissions or abstractions that may cause pollution.  Primarily based on use of licences/permits;

- Environmental taxation – an alternative to conventional regulatory instruments aimed at influencing behavior through pricing.  Examples include the Landfill Tax and the Climate Change Levy

- Organizational requirement of work site

Every organization has plans and requirements that determine how it will progress into the future. The strategic planners and management of the organization, in consultation with staff, determine most of these requirements. Others are externally imposed, such as the legislation the organization is required to comply with.

Examples of organizational requirements

- the organization’s vision, goals, objectives and priorities

- business and performance plans

- systems, processes and requirements for quality assurance

- specific change initiatives

- legal requirements, for example, occupational health and safety and anti-discrimination legislation

- standards (such as for ethical behavior) and protocols

- confidentiality and security requirements

- Defined resource parameters.

| ### Self-check 1 | Written test |
| --- | --- |

Name: ----------------------------------------------------- Date: ------------------------

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about set of activities and tasks before engineers commence work  (5 pts)

A. Personal protective equipment   B. Site preparation C. Tools and equipment

- Which one of the following is true is a tool which consists metal plate having sharp edges, the plate is attached to long handle which is generally made up of wood? (5pts)

A. Shovel     B. Spade      C. leveling   D. Measuring tape

- Which one of the following is true is used to protect the head from injury? (5pts)

A. Overall     B. Safety Helmet       C. Ear Muff   D. Crowbars

- Which one of the following is true are commonly used to open nailed wooden crates. Common uses for larger crowbars are: removing nails, prying apart boards, and generally breaking things. (5pts)

A. Overall     B. Safety Helmet       C. Ear Muff   D. Crowbars

- Which one of the following is true Hazards that affect the entire body can include temperature extremes, chemical exposure, radioactive materials, arc flash, infectious materials, sparks or flames, falls and sharp objects?  (5pts)

A. Overall           B. Safety Helmet                  C. Ear Muff           D. Crowbars

- Which one of the following is not true about steps in site preparation?(5pts)

A. Site Surveying       B. Site clearing        C. soil testing           D. concrete work

- Which one of the following is true about demolishing of buildings, the removal of trees, and eliminating any underground infrastructures?(5pts)

A. Site Surveying       B. Site clearing        C. soil testing           D. concrete work

- Which one of the following is true environmental risk of manual excavation? (5pts)

A. Risk of land degradation        B. Risk of flora and fauna

C. Risk of water pollution           D. Risk of air pollution    D. All

Note: Satisfactory rating - 30 points and above          Unsatisfactory - below 30 points

You can ask your teacher for the copy of the correct answer

| LG #40 | # LO #2-Work Site Preparation |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Safety equipment and materials - Securing safety materials and equipment - Excavation method - Environmental damage - Environmental protection - Compliance documentation - Traffic management This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Sect and use safety equipment and materials - Secure safety materials and equipment - Identify excavation methods - Identify environment damages - Identify environmental protection requirements - Apply compliance documentation - Manage traffic

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks

## Information Sheet 2

2.1. Safety of equipment and material

Material handling is a source of safety issues and injuries in many workplaces. Let’s find out why that’s the case – and how to come up with an effective safety plan for materials handling.

- Scope of Material Handling

Materials handling is a form of logistics movement that includes any process that involves the movement between vehicles, conveyors, store rooms, other forms of logistics support where employees are involved to some extent.

The extent of employee involvement will vary by operation and may include manual, automated, and semi-automated material handling events. The safety keys outlined below offer general guidance that would require tailoring for specific materials and logistics support scenarios.

Figure: 2.1. Safety of material handling

The foundation for a safe workplace starts with policies and procedures that senior management must put in place. A comprehensive health and safety policy would include the appropriate level of guidelines on material handling and the associated employee training program.

A key portion of the policy would be the safety compliance activity that is essential for the business to operate. Most importantly, the policy would inform on the shared responsibilities of each employee in keeping the workplace safe.

These policies need to follow both the federal guidelines for occupational health and safety, as well as all provincial guidelines. Health and safety policies with respect to material handling need to address specific hazards including, but not limited to:

- Poorly maintained lifting devices

- Exceeding load limits on lifting devices

- Damaged racking and storage units

- Repetitive or overly heavy manual lifting, lowering, pushing, pulling or carrying loads

- Handling materials while on a ladder or step stool

- Being stuck between materials

- Falling materials that are improperly stored

- Incorrectly using securing devices on materials

- Collisions with moving equipment

Since the materials that are handled in the workplace on a daily basis can vary widely in size, shape, weight, and quantity, a safety policy for material handling will look very different from business to business. Each one needs to have a custom tailored policy to properly protect their employees who handle materials for their job.

- Safety Training for Material Handling

There are employee safety risks involved with material handling in distribution centers, warehouses, shop floors and loading docks. The mitigation of safety risks starts with proper training in awareness, procedures and safety equipment. Safety training is mandated for any new employees involved with lifting, carrying, loading or transporting materials in any form. In addition to new hire training, the safety risks and potential safety compliance rules may require re-training on an annual basis or some other regular time interval.

- Material Handling Safety Procedures

The day to day handing of materials continues free of accident or injury when safety procedures are consistently followed. Procedures start with awareness of the specific material handling risks involved and the proper safety equipment for the situation. Procedures would cover the types of material, and the conditions of the work area.

Here’s a list for safely lifting and moving materials:

- Maintain the correct posture: avoid bending over and keep lifts close to the body

- Lift in a careful, deliberate manner and avoid any sudden lift movements

- Never lift materials from a sitting position, or twist to pick up a heavy object

- Use the correct grip on objects; lifts should be shoulder high and with full grasp of the hands

- Get assistance from co-workers to avoid successive lifts of heavy objects

- Always make use of conveyors, slides or other devices, to avoid unnecessary lifting or pushing of objects

- Always go around a blocked pathway, never step over an obstacle while carrying material

- Maintain a clear line of site; objects should not block vision; lift only where there is sufficient lighting

- Whenever possible; reduce load sizes, adjust bulky objects to ease movement

- Regularly stretch back and leg muscles during the day and keep in good physical shape

These are useful if posted and/or included in safety training manuals and modules. Safety for material handling gear means the proper fit and use of personal protection, to include, eye-protection, steel toed boots, other personal safety equipment.

Material handling safety equipment included the proper tools should be used in moving material. This would include the proper use of loaders, conveyers, forklifts and other aids to safely move material.

2.2. Securing of equipment and material

Securing material means all dunn age, lashing and other equipment used to block, brace, and secure packed cargo items in a container. Securing the materials using the following methods:-

- Attaching Equipment to Walls or Supports

Heavy or hazardous items that could collapse and create a danger or block emergency exits must be secured to the walls or floor by operation and technical Services. These items include shelving units, equipment racks, and tall file cabinets, distillation units, gas cylinders (attach at two heights, approximately one third and two thirds of the cylinder height). Any new apparatus should be constructed robustly and secured to supporting fixtures. If you need to route gas lines between apparatus mounted to different supports, the lines should either be made of a compatible material that is flexible, or have flexing joints if made of solid lines.

- Modifying Shelves and Cabinets

Shelves holding chemical containers must have protective restraint devices to prevent chemical containers from being eluded the shelf. Shelves above the floor level should have anti-earthquake matting or protective restraint device if used to hold heavy manuals, books, or equipment. Cabinets used to store chemical containers should have a closure device to prevent the door from being shaken open.

- Protective Procedures

Users should be:

- Ensuring there is clear access to exits, fire extinguishers, eyewash stations, and safety showers.

- Closing containers when not in use.

- Closing fume hood sashes when not in use.

- Maintaining good housekeeping, including not placing chemical containers on the floor.

- Not storing boxes or large items within 40 cm of the ceiling if the room has fire sprinklers.

2.3. Excavation method

youtube.com/watch?v=kD8QUfEMWLQ ((Accessed date 27/08/2022) showed that excavation method)). The nature of the excavation work being undertaken will affect the selection of an excavation
method and a safe system of work. Careful consideration should be given to health and
safety issues when planning the work where the excavation involves anything other than shallow trenching and small quantity of material.

A person conducting a business or undertaking who proposes to excavate a trench at
least 1.5m deep, must minimize the risk to any person arising from the collapse of the
trench by ensuring that all sides of the trench are adequately supported by one or more of the  following:-

- shoring by shielding or other comparable means (for example, boxing) „

- benching „

- battering

- Methods of excavation

- Full open cut method

- Bracing excavation method

- Anchored excavation methods

- Island excavation methods

- Zoned excavation methods

- Top down excavation methods

- Full open cut method

It divided into two major types including sloped full open cut as shown in Figure 2.2 and cantilever full open cut. The former is assumed to be economical since the side of excavation would be sloped and does not need any support to held foundation wall. However, if the slope is considerably gentle or the excavation is largely deep, it will costly.

Figure: 2.2 Full open cut excavation with side slopes

- Bracing excavation method

Bracing excavation as shown in (Figure 2.3) is the placement of horizontal struts in front of retaining wall to held excavation wall material pressure. Bracing system consist of wale, strut, center posts, end braces, and corner braces. Earth pressure transfer to horizontal struts through wale, and the purpose of corner and end braces is to reduce wale span without increasing strut number. Center posts prevent the failure of struts due to their own weight.

Figure: 2.3 Bracing excavation method

- Anchored excavation methods

In this technique, anchors as shown in (Figure 2.4) are installed to counter act against earth pressure. Configuration and components of anchored excavation technique are illustrated. Bonded portion of the anchor provides anchoring force that works against earth pressure whereas unbonded part of the anchored transfer pressure to the anchor head. Anchor head transfer loads to the retaining wall. The anchoring force is greatly based on the soil strength. The higher the soil strength the greater the anchoring forces. This technique is not suitable for clay and granular soil with high ground water table. Lastly, it require short time to complete excavation with great efficiency and suitable for large areas and shallow depth.

Figure: 2.4 Anchored excavation methods

- Island excavation methods

In this method, the center of excavation area is dug and excavated material placed close to the retaining wall to create a slope. After that, the major part of the structure would be constructed at the center of the excavation. Then, the sloped soil will be excavated and struts will be placed between retaining wall and the main structure. Finally, the struts will be removed and remaining parts of the structure will be constructed. Sometimes, it might be required to use anchored or braced technique to removed slopes soil material, specifically when the excavation is too deep.

Figure: 2.5.  Island excavation method

- Zoned excavation methods

Diaphragm walls are used as a retaining wall in the zoned excavation method. Deformation of the longer span wall would greater than short span wall as explained Figure 2.6.

Figure: 2.6 deformation of longer span wall compared to shorter span ones

So, the deflections of longer span walls are declined by dividing excavation area into small area to decrease wall deformation and settlement as shown in Figure 2.7.

Figure: 2.7 dividing total excavation area into two smaller areas

The excavation will begin in area B while area A would be left to support the wall of area B. then struts in area B would be installed and excavation starts in area A. This process will continue in stages till the whole excavation is completed. It can be clearly observed that the load on diaphragm wall would be considerably large and hence deflection would great if the area had not been divided into smaller area.

- Top down excavation methods

In this method, construction begins from the top to the bottom of excavation and superstructure construction starts after the construction of the first slab is completed. So, slabs are constructed after each stage of excavation is finished. The slabs play the same role as struts in holding earth pressure. Construction process order include retaining wall construction, pile construction under column of superstructure, placing columns on piles, and installing formwork for the first slab at the top then other slabs of the would be constructed after each excavation. This technique would need short construction time, but the cost is higher compare to other methods. Another advantage is that, construction area safer since slabs are stronger than struts.

2.4. Use proper drainage and diversion system at work site

- The diversion drains are used in the work site:

- To reduce the length of slope.

- To divert the runoff away from the protected area.

- To divert water away from the work site.

- To protect excavated land from overflow.

- To intercept the coming water from the top terraced area, into un-terraced area.

Apart from above, the following points should be considered for proper use of diversion drains.

They are:

- The catchment area of diversion drain should be covered by grasses or kept under cultivation to check the soil erosion or sediment yield. Otherwise, the section of diversion drain gets filled by the eroded soil (sediment).

- They should not be constructed wider than the width of three strips in strip cropped field.

- They should not be used as permanent control measure for the fields requiring terracing practice.

2.5. Environmental protection requirement from work site

- Environmental Requirements

Environmental Requirements:- means all applicable present and future laws, statutes, regulations, rules, ordinances, codes, licenses, permits, orders, approvals, plans, authorizations, judicial, administrative and regulatory decrees, directives and judgments of all governmental agencies, departments, commissions and boards, relating to the protection of human health or safety, or regulating or relating to industrial hygiene or environmental conditions, or the protection of the environment, or pollution or contamination of the air, soil, surface water or groundwater, including, without limitation, all requirements and regulations pertaining to reporting, licensing, permitting, investigating and remediating emissions, discharges, releases or threatened releases of Hazardous Substances, whether solid, liquid or gaseous in nature, into the air, surface water, or land, or relating to the manufacture, processing, distribution, use, treatment, storage, disposal, transport or handling of Hazardous Substances, whether solid, liquid or gaseous in nature.

- How to Protect the Environment

Environmental friendly business practices are a hot topic and the strong focus of businesses in nearly every industry, including construction. The great news is that often these solutions are not only better for the environment, but they’re also better for your bottom line. Get started with the following guide:

- Minimize Waste

Construction produces a large amount of waste materials, period. There’s no way around it, but you can and should look for ways to minimize the waste you’re producing. Increasing the efficiency of your operations, optimizing the use of supplies and materials, and choosing products and methods that reduce waste are all great ways to reduce the production of waste materials for all your projects. Every little bit really does add up.

- Industrial Recycling

Explore recycling initiatives specifically aimed at the construction industry like the EPA’s Industrial Recycling Program. This program focuses on recycling construction and demolition debris in other construction applications on site.

Figure 2.7 West recycling machinery

- Efficient Energy Use

Use tools, products, and materials that promote efficient energy use. Look for ways to improve the energy efficiency of your operations, materials used, and final products. Use the Energy Star Program as a guide.

- Choose Green Solutions

Look for products and materials specifically designed to be more environmentally friendly. Consider products that reduce the environmental impact of specific parts of your operations such as using an inflatable bladder dam for all your dewatering applications.

- Protection of Ecological Resources

It’s important to protect the water, plant life, and animal species in the area of your project. Not only will the general public take note, but the government has regulations in place to protect the biodiversity of local ecologies. Your end goal is to complete your project with the highest quality, most efficient timeline, and the least environmental impact.

Figure 2.8 Waste collections from the environment

2.6. Compliance documentation at work site

Compliance is the act of complying with a command, desire, wish, order, or rule. It can also mean adhering to requirements, standards, or regulations.

Compliance Documentation: The proposed tool is a set of draft documents that dynamically will change in each of the stages and for each of the components. It is necessary that the documentation be aligned with the principles of quality managed by the organization.  Rules and regulations lead to concrete requirements for companies. In order to meet these requirements, organizations must

- Define processes and operations,

- Guarantee the availability of information,

- Monitor processes and operations,

- Establish an effective communications policy

- Implement software tool supporting compliance measures.

Compliance and flexibility – no contradiction: - these measures apply to organizations as well as small organizational units, departments or projects. The best way is to establish a so-called Compliance Management System and set up a workflow that helps you to meet standards, one that is independent from individual persons while at the same time supporting the work of individuals. This will help you to master audits, implement changes to your processes and operations and take advantage of the opportunities compliance offers

2.7. Traffic Management

The work site is the length of road which includes the area where the work is being undertaken and any additional length of road used for traffic control including signs, tapers, traffic lights and other devices. A work area is a component of a work site and is occupied by workers, plant and materials. Work areas must be:

- Designed so that the minimum length and width of a road is closed at each stage to minimize disruption and inconvenience to road users while maintaining work site safety and efficiency;

- Staged to ensure minimum disruption to traffic especially at peak times, nights, weekends, holiday periods and during special events; and

- Monitored with action taken if lengths of traffic queues or delays occur which are greater than those predicted and allowed for.

Movement of vehicles and accommodation of farm vehicles can affect the safety and amenity of the public and neighbor’s. The problems of odor, noise, dust, and vehicle lights can be associated with traffic movement. Signage must be installed in accordance with Signs and devices. When using signs they must be:

- Placed before the roadwork begins and be removed as soon as they are no longer required;

- Regularly checked to ensure they are still relevant, in good mechanical condition, have not moved, rotated or blown over, are clean, not faded and have good night-time visibility as necessary; and

- Inspected to ensure they remain clearly visible and command attention to road users and are not obscured by vegetation, vehicles, plant or other signs and devices, and are displayed in the correct sequence.

Signs mounted on posts used for long-term operation must be located as follows:

- In open road areas or roads without kerbing – signs should be placed clear of the outer edge of the shoulder and, where possible, at least 2 m clear of the travelled way, whichever is the greater clearance.

- In built up areas or roads with kerbing – signs should be placed at a minimum of 300 mm clear distance behind the kerb.

Figure 2.9 sample signs in the work site

| ### Self-check 2 | Written test |
| --- | --- |

Name: ----------------------------------------------------- Date: ------------------------

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about protective procedure of equipment and material (5 pts)

A. Closing containers when not in use         B. Closing fume hood sashes when not in use

C. Ensuring there is clear access to exits       D. All

- Which one of the following is true placement of horizontal struts in front of retaining wall to held excavation wall material pressure? (5pts)

A. Bracing excavation method                 B. Full open cut method

C. Island excavation methods                    D. Top down excavation methods

- Which one of the following is true center of excavation area is dug and excavated material placed close to the retaining wall to create a slope? (5pts)

A. Bracing excavation method                           B. Full open cut method

C. Island excavation methods                    D. Top down excavation methods

- Which one of the following is true diaphragm walls are used as a retaining (5pts)

A. Bracing excavation method                           B. Full open cut method

C. zone excavation methods                    D. Top down excavation methods

- Which one of the following is true construction begins from the top to the bottom of excavation and superstructure construction starts after the construction of the first slab is completed?  (5pts)

A. Bracing excavation method                 B. Full open cut method

C. zone excavation methods                     D. Top down excavation methods

Test II: Mache column A to B

|  | A | B |
| --- | --- | --- |
|  | --------Diaphragm | A. minimize waste |
|  | --------Protect the environment | B. Shelves and Cabinets |
|  | --------Protect equipment | C. zone excavation |
|  | --------Diversion drains | D. complying with a command |
|  | --------Compliance | E. Divert water away from work site |

Note: Satisfactory rating - 30 points and above          Unsatisfactory - below 30 points

You can ask your teacher for the copy of the correct answer

| LG #41 | # LO #3- Small Excavations by Hand |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Location and specification confirmation - Location of underground services and taped area - Excavation dimension - Unstable ground collapse prevention - Barricades placing This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Confirm location and specification - Identify location of underground services and taped areas - Use excavation dimensioning tools - Prevent unstable ground collapse - Place Barricades

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks - Perform Operation Sheets - Do the “LAP test”

## Information Sheet 3

3.1. Location and specification of excavation

Specification and Tenders - Scope of Work: The scope for work covered under this specifications pertain to excavation of foundations, trenches, pits and over areas, in all sorts of soil, soft and hard rock, correct to dimensions given in the drawing including shoring, protections of existing underground utilities of any, such as water lines, electric cables etc. dewatering and shoring if necessary, stacking the useful materials as directed within the lead specified, refilling around the foundation and into the plinth with selected useful excavated earth and disposing off the surplus earth / materials within specified lead and finishing the surface to proper levels, slopes and camber etc. all complete.

- Scope of Work

The scope for work covered under this specifications pertain to excavation of foundations, trenches, pits and over areas, in all sorts of soil, soft and hard rock, correct to dimensions given in the drawing including shoring, protections of existing underground utilities of any, such as water lines, electric cables etc. dewatering and shoring if necessary, stacking the useful materials as directed within the lead specified, refilling around the foundation and into the plinth with selected useful excavated earth and disposing off the surplus earth / materials within specified lead and finishing the surface to proper levels, slopes and camber etc. all complete.

- Site Clearance:

Before the earth work is started the area coming under cutting and filling shall be cleared of all obstruction, loose stones, shrubs, rank vegetation, grass, bushes and rubbish removed up to a distance of 150 meters outside the periphery of the area under clearance. This work is deemed to be included in the earthwork item rate and no separate payment will be admissible.

- Roots and Vegetation clearance:

The roots of trees if any shall be removed to a minimum depth of 60 cm below ground level or a minimum of 30 cm below formation level whichever is lower and the hollows filled up with earth leveled and rammed. This work is deemed to be included in the earthwork items and no separate payment will be admissible for the work. Any material obtained from the site will be the property of the Government of India and the useful materials as decided by the Engineer-in-charge will be conveyed and properly stacked as directed within the lead specified.

- Setting out and making profiles:

Masonry or concrete pillars will be erected at suitable points in the area to serve as benchmarks for the execution of the work. Necessary profiles with pegs, bamboos and strings shall be made to show the correct formation levels before the work is started. The contractor shall supply labor and materials for setting out and making profiles for the work at his own cost and the same shall be maintained during the excavation work. The Department will show grid co-ordinate or other reference points. It shall be the responsibility of the contractor to set out center lines correctly with reference to the drawings and install substantial reference marks. Checking of such alignment by the Department will not absolve the contractor from his responsibility to execute the work strictly in accordance with the drawings.

3.2. Location of underground services before excavation

It outlines the potential dangers of working near underground services and gives advice on how to reduce any direct risks to people's health and safety, as well as the indirect risks arising through damage to services. It explains the three basic elements of a safe system of work during excavation:

- Planning the work

- Detect locating and identifying buried services

- Safe excavation

Plan work to avoid underground services:- where this is not possible, develop plans to minimize the risk of damage to those services in the  work area.  Identify clearly the extent of the work area and find out what underground services are within the area before considering whether they are likely to be disturbed:

- Obtain service drawings from utilities companies and other organizations with relevant information about the site.

- Survey the site to identify the services and other underground structures. Record the location of any services.

- Review/assess the planned work to avoid disturbing services where possible.

- Allow sufficient time and provide sufficient resource to do the work safely.

- Emergency work still requires planning and assessment of the risks arising from the work. A precautionary approach must be taken when breaking ground.

- Obtaining information on services

- Obtain plans or other suitable information about all underground services in the area when the work is being planned. Wherever possible, you must consult owners/operators. Remember that for some services there may be more than one owner/operator.

- There are a number of ‘one-call’ services available that can simplify the process of identifying who may have underground services in the work area and arranging for copies of plans and service records to be provided. Some of these services are free to use while others may charge.

- Where it is not possible for those undertaking the work to obtain information, as may be the case when emergency work has to be undertaken, the work must be carried out as though there are underground services in the area.

- Electricity

- Most electrical service cables belong to the regional distribution network operator (DNO). However, some cables belong to other bodies, such as: the highway or roads authority; the street lighting authority; electricity generating companies; National Grid; Ministry of  Defence; railway operator (usually Network Rail); Independent Distribution Network Operators (IDNOs); or other companies.

- Gas

- Most underground gas pipes are operated by public gas transporters (PGTs). The main exceptions are new developments, often supplied by independent gas transporters (IGTs) and properties fed from bulk-stored LPG, where the pipes may be owned by the property owners or other private individuals. In the latter case, owners/managers should be able to provide information but on certain estates owners and managers may not be available round the clock. You can obtain further information from the LPG supplier, whose name and telephone number (manned 24 hours) should be displayed at the bulk storage vessel compound or, for underground LPG tanks, at the segregated area above the tanks. Where the presence of gas pipes which operate at pressures of 2 bar (30 psig) and above is indicated, consult the owner/operator before work begins.

- Other services

- Other underground pipes may be found on or around: hospitals; airports; Ministry of Defence sites; universities; research parks; petrochemical, nuclear and other industrial sites. Where work is planned on such sites, make contact with the owner/occupier of the sites to request specific information about underground services.

- Because of the difficulty in detecting some telecommunication cables and the cost of damage to them, telecommunication companies may prefer to visit the site to locate the cables.

Protect underground service and people on your site by following these essential health and safety tips.

- Before excavation starts, consult the utilities suppliers/site owner about buried gas pipes or electrical cables. Get hold of service plans.

- To locate buried services use cable avoidance tools (CATs) and service plans. Mark their route with paint or wooden pegs.

- Don’t use mechanical excavators or power tools within 0.5m of the suspected route.

- Take care using power tools to break through paved surfaces above gas pipes and cable routes.

- Work well away from any overhead power lines when handling long items like scaffold poles, or using lifting equipment and MEWPs.

- Or, arrange with the electricity supplier for overhead power lines to be shrouded (covered), or made dead or rerouted.

- Plant working near overhead power lines should not approach closer than:

- 15m (plus length of jib) if the line is suspended from steel towers; or

- 9m (plus length of jib) if the line is supported on wooden poles.

- If unsure where concealed cables are, turn the electricity supply off before drilling into walls etc.

Figure 3.1 good and bad practice of cable avoidance

3.3. Dimension of trench excavation

The Contractor shall excavate all trenches to the dimensions and depths indicated on the drawings and described in this specification. The Contractor to allow for all bedding, fittings, cables and valve box assemblies. Trench bases shall be flat, smooth and free from rocks or other debris that could be injurious to pipes and control cables. No trenches are to be left open overnight or while the site is unoccupied under any circumstance. The Contractor shall provide all necessary equipment for the back-filling of trenches, compacting, leveling and shall be responsible for top dressing trenches. This shall include trenches that have subsided after completion of works. No variations shall be approved and paid for excavations except where material cannot be removed by reasonable methods – rock. The definition of rock for the purpose of this specification is any material that in the Project Manager’s opinion cannot be removed by hand tools or a 76kW backhoe with 300mm wide bucket. The Contractor shall advise the Project Manager before attempting to remove the rock by other means. The Contractor and Project Manager shall agree upon a method and price for removal. The price will reflect the type of rock and the methodology required for its removal.

- Layout

Trenches shall be pegged out using a Total Survey System by the Contractor and checked by the Irrigation Technical Officer, on behalf of the council, before trenching is undertaken. Trench layout shall only be altered where necessary and with the written approval of the Irrigation Technical Officer.

- Trench Alignment

The Contractor shall peg all mainline and lateral paths and use string lines for all straight-line trenching. Trenches shall follow the alignments indicated on the drawings, unless noted otherwise. Trenches shall align with fixed items such as buildings, kerbs, paths, sprinklers or valve locations. Trenches shall be straight and less than a trench width deviation from true, unless sweeping arcs are required to avoid obstacles. When there is a requirement to divert around an obstacle, the trench shall be offset parallel to the first trench and 45 degree angles used to join the pipe work. Pipe deflections will only be allowed where the deflected radius is within pipe manufacturer’s tolerances or is greater than 130 times the nominal pipe diameter.

Where existing trees are to be retained, any works adjacent to trees shall be performed without causing damage to roots or limbs. Council shall be notified prior to any trenching or excavation suspected to involve cutting of roots >50mm in diameter. The roots must be clean cut with no tearing behind the cut.

Visit the site and inspect the tree roots and determine the best approach, which may require a combination of: -

- Hydrovac excavation around roots

- Hand digging where appropriate

- Tree roots being cut back Contractor shall provide a schedule of rates

- Trench Width

Trenches for all pipe work up to and including 50mm in diameter shall be installed in a minimum 75mm wide trench, unless noted otherwise. 63 ~100mm diameter pipe work shall be installed within a minimum 300mm wide trench. If 2off pipes (up to and including 50mm diameter) are laid in the same trench, a minimum 300mm wide trench shall be provided. If 2off pipes greater than 50mm are laid in the same trench, the trench shall be 450mm wide. Where multiple pipes are laid in 1off trench, pipes must be apart by at least 1off diameter of the largest pipe and trench width adjusted to suit.

- Trench Depth

The Contractor shall allow for trench depths, including allowances for fittings, valve assemblies, valve boxes to ensure the resultant cover is achieved.

- Trench Adjustments

Trench adjustment changes, where required, shall be increased or decreased by gradual changes to the satisfaction of the Irrigation Technical Officer.

3.4. Preventing unstable ground collapse

Ground collapse is one of the primary risks to be controlled in excavation work. Ground
collapse can occur quickly and without warning, giving a worker virtually no time to escape,
especially if the collapse is extensive. A buried worker is likely to die from suffocation before
help arrives (e.g. either the head is buried, or the chest is so restricted by the weight of
ground that the worker can no longer breathe). Figure 8 shows a typical example of ground failure where material collapses onto a worker pinning them against the wall of a trench. Trench collapses of this nature can cause fatal injuries.

Figure 3.2. Start sliding	Figure 3.3.  Shear plan failur

Figure 3.4. Crush against trench         Figure 3.5. Badly injured

- Figure 3.2. showed that

This is a very dangerous situation, requiring ground support. No worker should be in the trench unless support has been installed.

- Area of tension, as wall starts to collapse.

- Slipping plane.

- Seepage along the slipping plane further reduces the stability of the wall. Water seeping into  the excavation, tension cracks on the surface and bulging side walls are all signs of imminent collapse.

- Figure 3.3. showed that

- Shear plane failure along the seepage (slippage) plane

- Figure 3.4. showed that

- Worker trapped and crushed against the trench wall by the quick collapse

- Figure 3.5. showed that

- Worker badly injured and probably smothered after being crushed against the opposite wall by the collapsing ground. The weight of a wedge of sand over a one meter length of trench two meters deep is about three tons

When planning the work and selecting appropriate excavation methods and control measures, it is important to consider:

- the type and strength of the material to be excavated (e.g. whether the ground is natural and self-supporting or has been previously backfilled)

- the moisture content of  the soil „

- if the ground is level or sloping if groundwater is present

- if there are any discontinuities or faults in the strata

- if there are any other nearby water courses, drains or run-off that might affect the stability of the excavation

- the work area and any access or operational limitations

- the planned height of the excavated face

- if vehicular traffic and/or powered mobile plant will operate near the excavation

- if there will be other construction activity nearby that may cause vibration

- any other loads adjacent to the planned excavation (e.g. buildings, tanks, retaining
walls, trees)

- if the need for persons to enter the excavation can be eliminated any underground essential services.

The ground conditions will have a significant impact on the selection of an excavation
method and the control measures implemented

- Ground conditions

In their natural condition, soils have varying degrees of cohesive strength and frictional
resistance. Examples of materials with virtually no cohesive strength are dry sand, saturated
sand and gravels with minimum clay content. Ground encountered in excavations can generally be categorized as one of three types:

- hard, compact soil

- soil liable to crack or crumble

- Loose or running material.

Of these materials, hard compact soil is the type that can cause the most trouble because the face ‘looks good’ and this often leads to risks being taken. Loose or running material is often the safest, because the need for safety precautions is obvious from the start.
Soil liable to crack or crumble is doubtful and should be given careful consideration before
the treatment to be given is determined. Useful information can often be obtained from local authorities.

Non-cohesive faces can be very hazardous. With the right amount of moisture they can
look safe and solid. A little loss of water by evaporation from the face or an increase in water
content from rain or other causes can make the soil crumble.
The stability of any excavated face depends on the strength of the soil in the face being
greater at all times than the stresses it is subjected to. The following situations all increase
soil stresses in an excavated face and may lead to failure under adverse weather conditions,
additional load or vibration:

- deep cuts and steep slopes, by removal of the natural side support of the excavated material

- loads on the ground surface near the top of the face, such as excavated material, digging equipment or other construction plant and material „

- shock and vibration, which could be caused by pile-driving, blasting, passing loads or vibration producing plant „

- water pressure from ground water flow, which fills cracks in the soil, increases horizontal stresses and the possibility of undermining„

- Saturation of soil, which increases the weight and in some cases the volume of the soil.

There are three main types of ground collapse control measures that can be used where ground collapse may occur:

- benching and battering

- positive ground support for example shoring

- Shielding – shields do not ensure ground stability but they protect workers inside the shield from ground collapse by preventing the collapsing material from falling onto them.

- Benching and battering

One fairly simple way of controlling the risk of ground collapse is to bench or batter the
excavation walls. An excavated slope is safe when the ground is stable. That is, the slope
does not flatten when left for a considerable period, there is no movement of material down the slope and the toe of the slope remains in the same place. If excavation work is planned to be carried out without positive ground support (that is, shoring), the continuing safety of the excavation will depend on the conditions arising during construction. If the conditions during construction are not as expected, or if conditions change during the course of the work (e.g. different soils, heavy rain/flooding) action should be taken immediately to protect workers, other persons and property. Implement appropriate control measures such as temporarily suspending work until the ground is stable or, if necessary, providing positive ground support. Benching is the creation of a series of steps in the vertical wall of an excavation to reduce
the wall height and ensure stability (see Figure 3.6). Benching is a method of preventing
collapse by excavating the sides of an excavation to form one or more horizontal levels
or steps with vertical surfaces between levels.

Figure 3.6 Benching

Battering is where the wall of an excavation is sloped back to a predetermined angle to
ensure stability (see Figure 7). Battering prevents ground collapse by cutting the excavated
face back to a safe slope. Battering should commence from the bottom of the excavation
and in some circumstances it may be appropriate to use a combination of the two methods
on an excavation (see Figure 8).

Figure 3.7 Battering

Figure 3.8 combinations of benching and battering controls

Benching and battering of excavation walls can minimize the risk of soil or rock slipping
onto the excavation. Control measures should be designed by a competent person (e.g.
geotechnical engineer) and be relative to the soil type, the moisture content of the soil, the
planned height of the excavated face and any surcharge loads acting on the excavated face.

It is not necessary to bench or batter the face of excavations which a competent person
determines are in stable rock or has assessed that there is no risk of collapse. When
benching or battering the walls of an excavation, an angle of repose of 45 degrees should
not be exceeded unless designed by a competent person and certified in writing.

- Shoring
Shoring is a positive ground support system that can be used when the location or depth of an excavation makes battering and/or benching impracticable. It should always be designed for the specific workplace conditions by a competent person (e.g. an engineer). Shoring is the provision of support for excavated face(s) to prevent the movement of soil and therefore ground collapse. It is a common method of ground support in trench excavation where unstable ground conditions, such as soft ground or ground liable to be wet during excavation such as sand, silt or soft moist clay are often encountered. Where ground is not self-supporting and benching or battering are not practical or effective control measures, shoring should be used. Shoring should also be used when there is a risk of a person being buried, struck or trapped by dislodged or falling material which forms the side of, or is adjacent to, the excavation work. Where such a risk also exists for those installing shoring, other appropriate control measures must be in place to ensure the health and safety of persons entering the excavation. Shoring the face of an excavation should progress as the excavation work progresses. Where earthmoving machinery is used risk assessment should be used to determine whether any part of the trench may be left unsupported. They should not work ahead of the shoring protection if it is being progressively installed.

The basic types of shoring are hydraulically operated metal shoring and timber shoring.
The most common shoring used consists of hydraulic jacks and steel struts, walls and sheeting. Sometimes aluminum or timber components are used. The use of metal shoring has largely replaced timber shoring because of its ability to ensure even distribution of pressure along a trench line and it is easily adapted to various depths and trench widths. Some of the common types of shoring are: „

- hydraulic systems:- systems are commonly used to provide temporary or mobile ground
support while other ground supports are being installed  „

- Steel sheet piling:- Steel sheet piling is generally used on major excavations such as large building foundations or where large embankments are to be held back and can be installed prior to excavation work commencing. „

- Steel trench sheeting:- Other methods of excavation may require the use of steel trench sheeting or shoring. It is positioned and pneumatically driven in to final depth.

- Timber soldier sets:- The soldier set is a simple form of trench support set which can be  formed with steel or timber. This system is mostly used in rock, stiff clays and in other  oil types with similar self-supporting properties

3.5. Placing barricades

Barricades act as warning devices that alert others of the hazards created by construction
activities and should be used to control traffic, both vehicular and pedestrian, safely through
or around construction work sites. It is the Contractor responsibility is to maintain a safe and accessible path for all pedestrians including persons with disabilities, around and/or through construction sites.

Barricades are required around all construction sites and all excavations, holes, openings in
floors or roofs, raised platforms, for certain types of overhead work, restriction of access
areas and wherever it is necessary to warn people against the potential of falling. Barricades
must be suitable for each area of use. Examples of barricades are plastic safety fencing,
temporary cyclone fencing and portable manhole barricades. Yellow caution tape and/or
cones are not considered acceptable barricades and should be used only temporary until
suitable barricades are erected. Signs and illumination should be used appropriately.

The following are examples of activities where barricades may be required:

- Wherever construction debris is dropped without the use of an enclosed chute.

- Areas with temporary wiring operating at more than 600 volts.

- Work areas for electrical equipment with exposed energized parts.

- The swing radius of the rotating superstructure of cranes or other equipment.

- Wherever equipment is left unattended near a roadway at night.

- Excavations. Areas used for the preparation of explosive charges or blasting operations.

- Street openings – manholes.

- Construction areas in energized electrical substations.

Contractors are responsible for providing all barrier materials for both interior and exterior application including but not limited to appropriate street closing barricades and signage that meet requirements, and all local, state and federal laws.

- The Contractor shall ensure the general construction area is protected; barricades must be erected before any excavation, extended as the excavation progresses and maintained until the project is completed.

- The Contractor shall furnish, erect, and maintain all the necessary signs, barricades, lighting, fencing, bridging, and flaggers that conform to the requirements.

- Barricaded areas which contain an opening or hole for access must be protected during working hours and must be secured at the end of each day.

- All holes or openings through floors or decking at all elevations must be immediately covered or barricaded. Material or equipment must never be stored in an excavation cover or inside an excavation area.

- Barricades shall not create a trip hazard.

- The type of barricading system, whether it is fencing, caution tape, or some other means, the Contractor must discuss the barricades system to be used with the office to ensure protection for the campus community.

- Warning signs should be placed on barricades/fences for the duration of the construction project.

- Upon completion of the project, barricades shall be removed promptly when no longer needed.

| ### Self-check 3 | Written test |
| --- | --- |

Name: ----------------------------------------------------- Date: ------------------------

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about  Before the earth work is started the area coming under cutting and filling shall be cleared of all obstruction (5 pts)

A. scope of work                                    B. site clearance

C. setting out of concrete                          D. none

- Which one of the following is true masonry or concrete pillars will be erected at suitable points in the area to serve as benchmarks for the execution of the work? (5pts)

A. scope of work                                    B. site clearance

C. setting out of concrete                          D. Top down excavation methods

- Which one of the following is true one of the primary risks to be controlled in excavation work? (5pts)

A. scope of work                                     B. site clearance

C. ground collapse                                   D. Top down excavation methods

- Which one of the following is true way of controlling the risk of ground collapse (5pts)

A. benching                                           B. battering

C. zone excavation methods                    D. A and B

- Which one of the following is true a positive ground support system that can be used when the location or depth of an excavation makes battering and/or benching impracticable?  (5pts)

A. benching                                           B. battering

C. Shoring                                              D. All

Test II:       Mache column A to B

|  | A | B |
| --- | --- | --- |
|  | -------Barricade | A. develop plans to minimize the risk |
|  | -------Hydraulic systems | B. positive ground support |
|  | --------Battering | C. mobile ground support |
|  | --------Shoring | D. warning devices |
|  | --------Plan work | E. cutting the excavated face back to a safe slope |

Note: Satisfactory rating - 30 points and above          Unsatisfactory - below 30 points

You can ask your teacher for the copy of the correct answer

## Operation Sheet -3

3.1. Excavate Trench

    - Tools and equipment

- Tools used for trench excavation:- Pick axe, spade, shovel,  rope, hoe ,tape meter, line level, leveling, hammer, peg,  ranging pole, survey leveling instruments,

- Personal protective equipment: Safety shoes, Sun hat, Gloves, Overall,

    - Steps of trench excavation

- Makes recon sense survey by transact wake

- Select a suitable site

- Prepare the site

- Site clearing

- Site surveying

- Soil testing

- Sketch and layout trench location

- Excavate trench width 0.8 meter, depth 0.6 meter, length 5 meter

- Prepare benching, battering, shoring----to protect ground collapse

- Benching:-the creation of a series of steps in the vertical wall of an excavation to reduce
the wall height and ensure stability

- Battering:- preventing collapse by excavating the sides of an excavation to form one or more horizontal levels or steps with vertical surfaces between levels

- Shoring:- a positive ground support system that can be used when the location or depth of an excavation makes battering and/or benching impracticable

- Removing shoring support after excavation

- Perform backfilling

- Add Porous material will be placed to protect the drain from sedimentation and other filtering materials if it necessary and refill it

- Perform compaction of backfield material

- Place a signage (singe) of vehicle protecting to cross the trench

| ### LAP TEST-3 | Performance Test |
| --- | --- |

Name……………………………….           ID……………………………..                                                     Date…………………………………….

Time started: ________________________ Time finished:  ________________

Instructions: Given necessary templates, tools and materials you are required to perform the following tasks within 12 hour. The project is expected from each student to do it.

Task-3.1. Perform trench excavation

| LG #42 | # LO #4- Excavation site isolate |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Loose material cleaning - Work area and material clearing - Confirmation and checking excavation This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Clean loose materials - Clear work area and materials - Confirm and check excavation

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks

## Information Sheet 4

4.1. Cleaning loose material of excavation

Clearing and grubbing foundation preparation usually begins with clearing, grubbing, stripping, and cleaning of the areas upon which a structure will be built. These operations are designed to remove materials having undesirable engineering qualities, such as low shear strength, high compressibility, undesirable permeability, or other characteristics, that could interfere with compaction operations and/or provide a poor surface for a structure foundation. Clearing and grubbing entails removal of trees, brush, large roots, and other woody growth to expose the soil prior to earthwork operations. Clearing consists of removal of aboveground obstructions, including trees, vegetation, felled timber, brush, and debris. Grubbing includes removal of objectionable belowground obstructions or material including stumps, roots, logs, and debris. The plans and specifications for any project should delineate the clearing and grubbing limits, the depth of removal, and the method of disposal. The inspector must know the specified limits of the work, see that they are marked on the ground, and assure the work is conducted within these limits as to not damage adjacent property. The inspector must also verify surveys are made prior to clearing and grubbing when necessary for computing payment quantities. The inspector may be required to verify that machinery does not operate within the drip line of specific trees and that the drip line of those trees are marked or fenced to protect the area within the drip line. The timing and rate of clearing and grubbing must comply with requirements in the specifications related to sediment and erosion control.

The inspector’s responsibilities related to clearing and grubbing includes verifying:

- the limits for clearing and grubbing are clearly marked

- surveys are completed prior to clearing and grubbing when necessary for computing quantities  all materials are removed and disposed of as specified

- the contractor’s operation does not damage adjacent property

- the contractor’s operation does not damage trees that are not to be removed

- burning is performed according to local ordinances and job specifications

- the timing and rate of clearing conforms to specifications

4.2. Work area and material clearing of excavated

Work area means a workplace, whether outdoors or inside a structure, where substances are stored, used, or manufactured, (and includes, but is not limited to, areas where substances are handled, mixed, processed, packaged, or re-packaged); and where employees, licensees, invitees, or other persons may be present. The term "work area" does not include principal and secondary residences and also does not include lawns or fields upon which fertilizers, pesticides, or herbicides are applied.

All excess excavated material which has been stockpiled at the work site, and which will not be used for backfill or other fill purposes, shall be removed from the project area within 48 hours of the excavation. In all cases, stockpiles of all excavated material and all construction materials shall be of limited size and shall be neatly maintained in such a manner that they will not block existing drainage or be hazardous to pedestrian or vehicular traffic in any way. The limitation relative to the stockpiling of all excavated material and all construction materials shall be controlled by the Owner and the City. In the event the Contractor fails to remove excess excavated material as required above, or fails to satisfactorily modify his operations relative to the stockpiling of excavated or construction material upon order of the Owner or the City, all work except cleanup operations will be stopped, and remain stopped, until the order of the Owner or the City has been complied.

The removal and disposal of all surplus excavated material shall be the responsibility of the Contractor. The contractor shall be provided with any surplus material desired, to be delivered at a location selected by City or Owner within City corporation limits. The City or Owner shall require the selected location to be leveled or contoured by Contractor daily at no expense to the City.

Silt barriers shall be placed within 24 hours around long term stockpiles of soil (unless temporary seeding is used) as well as along the down slope, or drainage course, and portions of staging sites. Long term stockpiles shall be defined as stockpiles remaining after 48 hours of the excavation. Also, excavated materials and stored materials shall not be placed next to trees within that area encircled by drawing a circle having a diameter in feet equal to the tree diameter in inches about the center of the tree.

4.3. Confirmation and checking of trench excavation

Checking excavation of trench is done according to the standard of  system installed in the area of work by giving the result if performed well say “yes” if not justify the reason.

The activities for those saying “yes” observe and make confirmation

Project Name:____________________________________

Contractor:_____________________________

Date:___________ Time_________

Table 4.1 checklist of excavation in the work site

| Section | Description | Yes | If no why? |
| --- | --- | --- | --- |
|  | General Site Information |  |  |
|  | Is the contractor's competent person onsite during excavation & trenching operations? |  |  |
|  | Does the manual excavation is done according to depth |  |  |
|  | Does the manual excavation is done according to width |  |  |
|  | Does the manual excavation is done according to length |  |  |
|  | Workers are cleared and away from overhead hazards? |  |  |
|  | Does the excavation is use ground collapse protection system |  |  |
|  | Engineering designs for sheeting & or manufacturers data on trench box available on site? |  |  |
|  | Adequate signs posted and barricades provided? |  |  |
|  | Has a PPE assessment been completed and are workers wearing the appropriate PPE? |  |  |
|  | Is the contractor competent person performing daily inspections on the work area and protective systems? |  |  |
|  | Utilities: |  |  |
|  | Has the Utility Locate Request Permit been completed? |  |  |
|  | Have the location of utilities been marked? |  |  |
|  | Overhead lines located, noted and reviewed with operator. |  |  |
|  | Underground installations protected, supported, or removed when excavation is open. |  |  |
|  | Wet Conditions |  |  |
|  | Precautions taken to protect workers from water accumulation if applicable (continuous dewatering). |  |  |
|  | Surface water or runoff diverted or controlled to prevent accumulation in the excavation. |  |  |
|  | Means of Access and Egress: |  |  |
|  | Lateral travel to means of egress no greater than 25 feet in excavations four feet or more in depth. |  |  |
|  | Ladders use in excavations are secured and extended three feet above the edge of the trench? |  |  |
|  | Are structural ramps designed and inspected by the competent person? |  |  |
|  | Environmental Hazards |  |  |
|  | There is no evidence of odors, oil or contaminated soil in the excavation. |  |  |
|  | Excavation greater than 4 feet in depth does not contain potential hazardous atmosphere. |  |  |
|  | Loose material are cleaned from the site |  |  |
|  | Materials in the work site are cleared |  |  |

| ### Self-check 4 | Written test |
| --- | --- |

Name: ----------------------------------------------------- Date: ------------------------

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about Clearing and grubbing foundation preparation usually begins with? (5 pts)

- Stripping             B. clearing               C. Grubbing        D. All

- Which one of the following is true workplace, whether outdoors or inside a structure, where substances are stored, used, or manufactured? (5pts)

- Stripping          B. clearing          C.  Grubbing                 D. work area

- Which one of the following is true one of the excavation of trench is done according to the standard of  system installed in the area of work by giving the result if performed well say “yes” if not justify the reason is called? (5pts)

- scope of work                                      C. checking

- confirmation                                        D. Top down excavation methods

- Which one of the following is not true about general site information  (5pts)

- Does the manual excavation is done according to depth

- Overhead lines located, noted and reviewed with operator

- Precautions taken to protect workers from water accumulation if applicable   D. All

- Which one of the following is true a positive ground support system that can be used when the location or depth of an excavation makes battering and/or benching impracticable?  (5pts)

- Excavation greater than 4 feet in depth does not contain potential hazard

- Does the manual excavation is done according to width

- Does the manual excavation is done according to width               D. All

Note: Satisfactory rating - 15 points and above          Unsatisfactory - below 15 points

You can ask your teacher for the copy of the correct answer

| LG #43 | # LO #5- Restoring work site |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Backfill excavation - Compaction - Planting or replanting vegetation - Reinstating site This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Backfill - Compact - Plant or replant vegetation - Reinstate site

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks - Perform Operation Sheets - Do the “LAP test”

## Information Sheet 5

5.1. Backfilling excavation

Backfilling was performed as soon as possible after the remediation excavations were complete to mitigate risks associated with excavation collapse, slips and falls, and recontamination.

Backfill materials were subject to validation procedures as detailed in the Remedial Strategy and the Auditor-approved Remedial Works and Validation Plan. This included historical checks on the source of the material and chemical analysis to confirm that no contamination was brought back onto the site.

The Contractor shall generally apply bedding, backfill and top finishes to trenches and excavations. The base of trenches shall be laid & prepared with 60mm deep bedding sand complying with. Contractor to allow for imported bedding sand as part of the tender and include as a separate price (supply & delivery only). At the time of excavation, if it is agreed with the Project Manager and CoA Irrigation Technical Officer that the excavated spoil is suitable for bedding sand, it shall be used in lieu of the tendered bedding sand. Adjustments to the Contract Sum shall be made through a variation order, as required by the Contractor and assessed by the Project Manager. Unless noted otherwise, the first backfill to 200mm over any pipe shall be selected bedding material from excavated spoil or bedding sand complying with AS3500.1. The remaining trench depth can be filled with selected site spoil excavated from site. Backfilling of trenches shall occur in layers of not more than 150mm thereafter. Each layer shall be compacted prior to the application of the next layer. Trench compaction shall be by the use of manual tamping for up to the first 200mm above the top of pipe and mechanical tamping thereafter.

5.2. Compaction after excavation

There are six different methods used to compact substances. Some are more suitable for soil while others are used to compact non-soil materials such as asphalt. The most effective techniques are generally the ones that apply the most compressive stress.

- Static – a large amount stress is slowly applied to the soil and then is released

- Impact – a large mass is dropped onto the soil to apply the stress

- Vibrating – a mechanically driven plate or hammer is used repeatedly to apply stress

- Gyrating – soil is subjected to a gyratory motion while the stress is maintained in one direction

- Rolling – stress is applied via a large cylinder rolling over the surface of the soil

- Kneading – shear is applied by alternating movement in adjacent positions

To determine whether or not the natural soil is ready to be filled, there is a technique called proof rolling. Proof rolling is when heavy construction/compacting equipment is rolled over the area, looking for deflections all over the fill site.

5.3. Planting and replanting vegetation

Forests play a critical part to keep our environment healthy. But our forests need us more than ever – more than 1 million acres of National Forests are in need of reforestation. We are excited to share some facts about reforestation. Tree planting improves wildlife habitat connectivity and supports biodiversity.

Trees remove harmful pollutants from the air and act as our natural air filter.

- The seedlings we plant are collected from local seeds next to the planting site.

- Wildfire accounts for a staggering 80 percent of reforestation needs.

- More than one million acres of National Forests are in need of reforestation.

- Trees would not naturally regenerate, tree planting helps re-establish forest cover and improve forest health.

- Reforestation helps make our forests more resilient to future challenges like climate change and wildfire.

Figure 4.1.  Planting tree in the work site

5.4. Reinstatement of the Excavated Area with Clean Material

The excavated area was reinstated with clean material when it had been demonstrated that the site had been remediated in accordance with the relevant approvals and criteria.

Reinstatement activities included:

- Importation of fill, and backfilling and compaction of the excavated area to the designated levels in accordance with the detailed earthworks specification;

- Importation and placement of sub-grade materials;

- Revegetation of the site’s surface and maintenance of the new vegetation;

- Fencing works;

- Completion of the noise wall on the southern site boundary;

- Installation of heritage interpretation signage;

- Repairs to the shallow drain on the western boundary; and

- Repairs to the site access road and footpath area

| ### Self-check 5 | Written test |
| --- | --- |

Name: ----------------------------------------------------- Date: ------------------------

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about was performed as soon as possible after the remediation excavations were complete to mitigate risks associated with excavation collapse? (5 pts)

- Stripping                                               C. Compacting

- Grubbing                                               D. Backfilling

- Which one of the following is true a large mass is dropped onto the soil to apply the stress? (5pts)

- stripping                                                  C. impact

- Static                                                      D. work area

- Which one of the following is true a mechanically driven plate or hammer is used repeatedly to apply stress? (5pts)

- Vibrating                                             B. checking

- confirmation                                        D. Gyrating

- Which one of the following is true a mechanically driven plate or hammer is used repeatedly to apply stress? (5pts)

- Vibrating                                             C. Kneading

- Rolling                                                 D. Gyrating

- Which one of the following is true Importation and placement of sub-grade materials?  (5pts)

- Vibrating                                             C. Kneading

- Rolling                                                 D. Gyrating

Note: Satisfactory rating - 15 points and above          Unsatisfactory - below 15 points

You can ask your teacher for the copy of the correct answer

## Operation Sheet -5

  - Planting tree

- Tools and equipment

- Tools used for trench excavation:- Pick axe, spade, shovel,  rope, hoe ,tape meter, line level, leveling, hammer.

- Personal protective equipment: Safety shoes, Sun hat, Gloves, Overall,

- Steps of planting tree

- Dig the planting hole

- Loosen the roots and remove the nursery stake

- Place the tree in the center of the hole

- Backfill the tope soil to the hole

- Build a soil berm

- Add mulch

| ### LAP TEST-5 | Performance Test |
| --- | --- |

Name……………………………….   ID……………            Date…………………………………….

Time started: ________________________ Time finished:  ________________

Instructions: Given necessary templates, tools and materials you are required to perform the following tasks within 7 hour. The project is expected from each student to do it.

Task-5.1. Perform trench plantation

| LG #44 | # LO #6- Record and Reporting activity |
| --- | --- |

## Instruction sheet

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Argument of workplace record - Reporting activities This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Argue of workplace record - Report activities

Learning Instructions:

- Read the specific objectives of this Learning Guide. - Follow the instructions described below. - Read the information written in the information Sheets - Accomplish the Self-checks

## Information Sheet 6

6.1. Argue work place record

There are a few reasons why an employee may want to record a conversation, including:

To remember what was said for a later date For legal evidence For improper reasons, e.g. to use as blackmail. The employee should ask for your permission before recording, and you can choose whether to allow them to. If you choose not to let your employees record a meeting or conversation, make this clear to them beforehand and ask them to confirm that they’ll not be doing so. It’s best practice to outline your stance on recording meetings within your employee handbook so that it’s clear what will be considered misconduct. Many employers choose to include a line in their Disciplinary Policy, stating that the consent of all parties must be given before making a recording.

Typically, if an individual makes a recording in secret without asking, or after you’ve denied them permission, this will be seen as misconduct, and could even amount to gross misconduct.

However, it’s particularly important to seek professional advice on whether or not their behavior amounts to gross misconduct and before making any snap decisions to dismiss them, especially with the law around recording conversations in the workplace being unclear.

As a general rule, you should go into each meeting prepared for the possibility that an employee may secretly be recording it, and that the recording could still be permitted as evidence in a tribunal, even if you’ve asked the employee not to record the conversation.

6.2. Report completed work

If you are the person that is tasked with doing a site visit as well as coming up with the report, then know that there are things that you will have to do to acquire what the report needs. With that in mind, here are the steps that will help you create a proper site visit report: https://www.youtube.com/watch?v=hJuLo-A9-wg (video of report format).

Step 1: Start With the General Information

The first thing that you will need to do is to come up with an introduction. Start by pointing out the location of the site that you visited, followed by the date on which you visited it. Also, provide information on the person you contacted that permitted the visit. If extensive travel was needed, then provide a brief explanation in regards to how you made it to the site.

Step 2: Define the Site and Its Purpose

Next is for you to describe the type of site that you visited. Was it a factory, construction site, business, or school? You will want to include details such as the size of the site, its layout, and the different materials and equipment that were being used. State the number of people working on the site and its owner.

Step 3: Explain the Sequence of Events

This is where you will be talking about everything that you have noticed during your visit. Start at the very beginning of your visit. Who were the people you met? What it is that you did? Describe the events of the day going on until you left the site. Include details such as what you managed to see, the events that took place, demonstrations conducted, etc.

Step 4: Summarize the Site Operations

This is the summary report of all the processes and procedures of the site that you visited. Provide detailed information such as the special techniques that were used or the site’s entire manufacturing process. You can get more information by looking into the site’s many different departments; doing this also helps you understand the corporate structure.

Step 5: Identify the Site’s Strengths and Weaknesses

It is here where you will need to do something similar to a SWOT analysis as you’ll want to take note of which processes, policies, and practices work well at the site. If there are any flaws or glaring weaknesses, then you will need to point them out in your report. When doing this, make sure to be as specific as possible.

Step 6: Make Your Conclusion and Provide Suggestions

Once you are done with the entire report, the last thing that you will need to do is to provide your feedback and your suggestions; only give the latter if you are required to do so. Make sure that you come up with a logical conclusion based on all of the information you’ve gathered from the visit.

If you are asked to write a reflection of your visit, try to:

- Make links between theory and practice, i.e. what you’ve been doing in your subject, what you’ve read, any previous professional experience you have in the field and the practices you observed at the site.

- Demonstrate in your reflection that you understood the most important features of the site.

- Evaluate and discuss the relative strengths and weaknesses of the processes and procedures you observed (e.g. technology, efficiency).

6.2.1. Excavation Site Checklist and Daily Field Report

Competent Person: ------------------------------ Date: -------------------------

Project Name: -----------------------------------Time: -----------------------

Project Location: ------------------------------- Job No: ----------------------

Weather Conditions: --------------------------- Rainfall (Inches):------------------

| Description of Inspection Item: | Yes | No | Comments |
| --- | --- | --- | --- |
| 1. Have all utilities marked their locations? |  |  |  |
| 2. Have all affected parties been notified? |  |  |  |
| 3. Is proper traffic control in place? |  |  |  |
| 4. Has the soil been classified? |  |  |  |
| 5. Has a protective system been selected by the competent person? |  |  |  |
| 6. Has the competent person inspected the excavation/trench prior to start of each work period? |  |  |  |
| 7. Has the work plan been discussed with all employees? |  |  |  |
| 8. Are all employees protected from cave-ins when entering and exiting the excavation? |  |  |  |
| 9. Have hazardous objects around the excavation been removed or supported? |  |  |  |
| 10. Is all spoil maintained at least 2 feet back from the edge of the excavation? |  |  |  |
| 11. Are ladders used for access and egress? If so, are they installed correctly? |  |  |  |
| 12. Are employees protected from loose materials or tools which could fall into the trench? |  |  |  |
| 13. Are employees wearing the proper safety equipment? |  |  |  |
| 14. Is the excavation/trench free of standing or seeping water? |  |  |  |
| 15. Are there evidences of shrinkage cracks in the face of the trench wall? |  |  |  |
| 16. Were there evidences of sloughing of soil from the trench face since the last inspection? |  |  |  |
| 17. If a support system has been installed, was it installed in accordance with recommendations? |  |  |  |
| 18. Is heavy equipment kept away from the edge of the excavation? |  |  |  |
| 19. Are any changed conditions properly noted? |  |  |  |
| 20. Additional comments on safety. |  |  |  |

Instructions: Complete the items in the checklist by circling the appropriate response and noting descriptive conditions in the comments column. Sign the form to attest that the indicated items were reviewed during the inspection.

Table 6.1. Report format of trench excavation

| ### Self-check 6 | Written test |
| --- | --- |

Name……………………………………………   ID………………………… Date…….

Directions:  Answer all the questions listed below.

Test I: Choose the best answer

- Which one of the following is true about Start by pointing out the location of the site that you visited, followed by the date on which you visited it? (5 pts)

- Start With the General Information       C. Define the Site and Its Purpose

- Explain the Sequence of Events             D. Summarize the Site Operations

- Which one of the following is true You will want to include details such as the size of the site, its layout, and the different materials and equipment that were being used? (5pts)

- Start With the General Information       B. Define the Site and Its Purpose

- Explain the Sequence of Events             D. Summarize the Site Operations

- Which one of the following is true Start at preparing report at the very beginning of your visit? (5pts)

- Start With the General Information       C. Define the Site and Its Purpose

- Explain the Sequence of Events             D. Summarize the Site Operations

- Which one of the following is true you will need to do something similar to a SWOT analysis as you’ll want to take note of which processes, policies, and practices work well at the site? (5pts)

- Start With the General Information       C. Define the Site and Its Purpose

- Explain the Sequence of Events             D. Summarize the Site Operations

Test II: Short Answer Questions

- Prepare a daily report format of trench excavation follow-up checklist?(10)

Note: Satisfactory rating - 20 points and above          Unsatisfactory - below 20 points

You can ask your teacher for the copy of the correct answer

## Operation Sheet -6

6.1. Field excavation activities used to make a report of site actives of excavation.

- materials and equipment

- Materials used for preparing report format: - square paper, or paper, pen, ruler.

- Personal protective equipment: hand protection, eye protection,

- Procedures of field report

- prepare the format of report

- on the format list out the activities to be reported

- systematically observe and accurately record the situation

- analyze your observation

- forward the report to the concerned body

| ### LAP TEST-6 | Performance Test |
| --- | --- |

Name……………………………….   ID……………            Date…………………………………….

Time started: ________________________ Time finished:  ________________

Instructions: Given necessary templates, materials you are required to perform the following tasks within 2 hour. The project is expected from each student to do it.

Task-1 Conduct report of field excavation

### Reference Materials

Books:

Abusharar, S. and Han, J. (2011). Two-dimensional Deep-seated Slope Stability Analysis of Embankments over Stone Columns. Engineering Geology, 120: pp. 103-110.

Baez, J.I. and Martin, G.R. (1992). Quantitative Evaluation of Stone Column Techniques for Earthquake Liquefaction Mitigation. Proc. Tenth World Conference on Earthquake Engineering, A.A. Balkema, Brookfield, VT, pp. 1477-1483.

Han, J. (2015). Principles and Practice of Ground Improvement. John Wiley & Sons, Hoboken, NJ, 418p.

Hickie D, Wade M (1997). The development of environmental action plans: turning statements into actions. J Environ Plan Manage 40(6):789–801

Põder T (2006) Evaluation of environmental aspects significance in ISO 14001. Environ Manage 37(5):732–743

Web addresses:

https://www.safeworkaustralia.gov.au/system/files/documents/1705/mcop-excavation-work-v1.pdf (Accessed August, 26, 2022)

https://www.ehs.gatech.edu/sites/default/files/excavation_and_trenching_procedure.pdf (Accessed August, 26, 2022)

https://www.worksafe.govt.nz/topic-and-industry/excavation/excavation-safety-gpg/ (Accessed August, 25, 2022)

https://www.worksafe.govt.nz/assets/dmsassets/1/1442WKS-2-excavations-safety-with-underground-services.pdf (Accessed August, 25, 2022)

https://www.youtube.com/watch?v=hJuLo-A9-wg (Accessed August, 24, 2022)

https://www.youtube.com/watch?v=kD8QUfEMWLQ ((Accessed date 24/08/2022)

https://www.youtube.com/watch?v=aJ9zekjgY2w ((Accessed date 23/08/2022)

AKNOWLEDGEMENT

Ministry of Labor and Skills wish to extend thanks and appreciation to the many representatives of TVET instructors and respective industry experts who donated their time and expertise to the development of this Teaching, Training and Learning Materials (TTLM).

The experts who developed the learning guide

| No | Name | Qualification | Organization/ College | Mobile number | E-mail |
| --- | --- | --- | --- | --- | --- |
| 1 | Serawit Gensa | Msc. Water Resource Engineering | W/Sodo AVET | 0916740916 | serawitgen@gmail.com |
| 2 | Edao Hassen | Msc.Irrigation Engineering | Alage AVET | 0911098097 | hassedao@gmail.com |
| 3 | Mekete Agizew | Msc. Water Resource Engineering | Amhara. Kombolcha ATVET | 0925221192 | gen.mam09@gmail.com |
| 4 | Wondu Alemayehu | Msc.Irrigation Engineering | Oromia Kombolcha ATVET | 0910-28-99-61 | woldualem@gmail.com |
| 5 | Ademe Ayalew | Msc.Irrigation Engineering | Agrafa AVET | 0912720547 | Ademe2004@gmail.com |
| 6 | Said Mohammed | Msc.Irrigation& drainage Engineering | Alage ATVET | 0917180181 | Siyamsdmhmmd@gmail.com/ |
| 7 | Molalign Asfaw | Bsc.Water Resource &Irrigation Engineering | Alage ATVET | 0921431096 | Mollalign410ass@gmail.com |
| 8 | Yonas Hailu | Bsc.Water Resource &Irrigation Engineering | Agrafa AVET | 0934715578 | yonashailuw@gmail.com |
| 9 | Lemessa Mulata | Msc.Irrigation Engineering | Agrafa AVET | 0913266845 | Lamimulle2022@gmail.com |
| 10 | Misganew Yimer | Bsc. Soil and Water Engineering | Woreta ATVET |  | Misge1976@gmail.com |
| 11 | Daniel Derese | Bsc. Soil and Water Engineering | W/sodo AVET | 0912-79-28-85 | danielderesse7@gmail.com |
| 12 | Teshome Getachew | Msc.Irrigation & drainage Engineering | Alage ATVET | 0925-50-13-99 | teshomegetachew131@yahoo.com |