GARMENT PRODUCTION

LEVEL - IV

Based on 9/2023 Curriculum Version I,

Module Title: Apply statistics to operational processes

Module Code: IND GAP4 M07 0322

Nominal duration: 28 hr

Prepared By: Addis Ababa TVET Bureau

September /2023

Addis Ababa, Ethiopia

# Acknowledgement

Addis Ababa TVET Bureaus wishes to thank and appreciation for the trainers who donated their effort and time to develop this Curriculum based TTLM for the TVET Program garment production Level IV. We also thank all Addis Ababa TVET College, Addis Ababa TVET Bureaus and coordinators, all instructors who developed this TTLM for active facilitation of this TTLM development.

# Table of Contents

Acknowledgement	2

Table of Contents	3

Acronyms	5

Module units	6

Module Learning Instructions:	6

INTRODUCTION	7

Unit one: Collect process data	8

1.	Collect process data	8

1.1	Sampling techniques	9

1.1.1	Sample size	11

1.1.2	Sampling timing	12

1.1.3	Sampling points	13

1.1.4	Sample type	14

1.1.5	Batch sheets	16

1.2	Standard measurements	17

1.2.1	Standard operating procedures	19

1.2.2	Type of measurements	20

1.3	Required data	22

Self-check-1	23

Unit Two: Interpret data	25

2.	Interpret data	25

2.1	Control chart	27

2.1.1 Identify statistical data	28

2.1.2 Calculating statistical data	30

2.2	Random and non-random patterns	32

2.3	Control limits	33

2.4	Recognize situations for taking corrective action	35

2.5	Determine cost of non-conformance	36

Self-check-2	38

Unit three: Calculate control limits	40

3.	Calculate control limits	40

3.1.	Control limit	41

3.2.	Relevant stakeholders	42

3.3.	Control limit revise	43

3.4.	Plot limits on control chart	44

3.5.	Plot chart	46

Self-check-3	47

REFERENCE	49

Developers Profile	Error! Bookmark not defined.

# Acronyms

SOP …………………….Standard operating procedures

ULC……………………. Upper limit control

LLC…..............................lower limit control

# Module units

Unit 1. Collect process data

Unit 2. Interpret data

Unit 3. Calculate control limits

Learning objectives of the Module

At the end of this session, the students will able to:

- Collecting process data

- Interpreting data

- Calculating  control limits

# Module Learning Instructions:

- Read the specific objectives of this Learning Guide.

- Follow the instructions described below.

- Read the information written in the information Sheets

- Accomplish the Self-checks

# INTRODUCTION

Statistics can be applied to operational processes in garment production in a number of ways. Some common examples include: Statistical process control (SPC): SPC is a set of tools and techniques that can be used to monitor and control the quality of manufacturing processes. SPC charts can be used to identify and eliminate sources of variation, which can lead to improved product quality and reduced costs. Design of experiments (DOE): DOE is a statistical method that can be used to identify the optimal settings for process variables. This can be useful for improving the quality of products, reducing costs, or increasing efficiency.

Data mining: Data mining is a process of extracting knowledge from large datasets. It can be used to identify patterns and trends in garment production data, such as which styles are most popular, which sizes sell the best, and which manufacturing processes are most efficient.

Machine learning: Machine learning is a type of artificial intelligence that can be used to build models that can predict outcomes based on historical data. Machine learning models can be used in garment production to predict things like demand for products, the likelihood of defects, and the time it will take to produce a particular garment.

SPC can be used to monitor the quality of cutting operations. For example, SPC charts can be used to track the accuracy of the cutting process and to identify any sources of variation. This information can then be used to make corrective actions to improve the quality of the cutting process.

DOE can be used to optimize the sewing process. For example, DOE can be used to identify the optimal settings for stitch length, stitch tension, and presser foot pressure. This can help to improve the quality of the sewing process and to reduce the number of defects.

Data mining can be used to identify patterns and trends in garment production data. For example, data mining can be used to identify which styles are most popular, which sizes sell the best, and which manufacturing processes are most efficient. This information can then be used to make better decisions about product design, production planning, and inventory management.

Machine learning can be used to predict demand for products. For example, a machine learning model can be trained on historical data to predict how many units of a particular product will be sold in a given period of time. This information can then be used to plan production and inventory levels more effectively.

# Unit one: Collect process data

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Interpret sampling techniques - Handling required data - Obtain standard measurements This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Interpreting sample scheme - Obtaining measurements in accordance with standard procedures - Handling data

# Collect process data

Collecting process data statistics to operational processes means in garment production is the process of gathering information about how the production process is performing. This information can be used to identify areas for improvement and make necessary changes to improve efficiency, quality, and profitability.

There are a number of different ways to collect process data statistics in garment production. Some common methods include:

- Manual data collection: This involves manually collecting data from production records, such as time sheets, production logs, and quality control records.

- Automated data collection: This involves using automated systems to collect data from production equipment, such as sewing machines and cutting machines.

- Sensor-based data collection: This involves using sensors to collect data about the production process, such as machine temperature and cycle time.

Once the data has been collected, it can be analyzed using statistical methods to identify trends and patterns. This information can then be used to make informed decisions about how to improve the production process.

Here are some examples of how process data statistics can be used to improve operational processes in garment production:

- Identifying production bottlenecks: By tracking production data, garment manufacturers can identify areas where the production process is slowing down. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improving quality control: By tracking quality control data, garment manufacturers can identify areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reducing costs: By tracking production data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

By collecting and analyzing process data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, and profitability of their production processes.

Here are some additional tips for collecting and analyzing process data statistics in garment production:

- Identify the specific processes that you want to improve. This will help you to focus your data collection efforts.

- Identify the key performance indicators (KPIs) that you want to track. KPIs are metrics that measure the performance of a specific process.

- Collect data regularly and consistently. This will allow you to track trends over time and identify areas where improvement is needed.

- Use statistical methods to analyze the data. This will help you to identify trends and patterns that are not easily visible to the naked eye.

- Use the data to make informed decisions about how to improve the production process.

By following these tips, you can collect and analyze process data statistics to improve the operational processes in your garment production operation.

## Sampling techniques

Sampling techniques are used to collect data from a subset of a population to make inferences about the entire population. This can be useful in garment production because it can be difficult to collect data from every single garment that is produced.

There are a number of different sampling techniques that can be used in garment production, depending on the specific needs of the manufacturer. Some common techniques include:

- Random sampling: This involves randomly selecting a sample of garments from the population. This is the most common sampling technique and it is considered to be the most representative of the population.

- Stratified sampling: This involves dividing the population into different groups (strata), such as garment style or size, and then randomly selecting a sample from each stratum. This technique is useful when the population is not homogeneous and it ensures that all groups are represented in the sample.

- Cluster sampling: This involves dividing the population into groups (clusters) and then randomly selecting a sample of clusters. This technique is useful when it is difficult or expensive to collect data from individual garments.

Once the sample has been collected, the data can be analyzed using statistical methods to make inferences about the entire population. For example, the data could be used to estimate the average quality of the garments in the population or to identify any areas where there is a problem with the production process.

Here are some examples of how sampling techniques can be used to process data statistics to operational processes in garment production:

- Quality control: Sampling techniques can be used to randomly select a sample of garments from a production batch to inspect for quality defects. This information can then be used to estimate the overall quality of the batch and to identify any areas where there is a problem with the production process.

- Process improvement: Sampling techniques can be used to collect data from a sample of garments to identify areas where the production process can be improved. For example, the data could be used to identify bottlenecks in the production process or to identify areas where there is a lot of waste.

- Production planning: Sampling techniques can be used to collect data from a sample of garments to estimate the average time it takes to produce a garment. This information can then be used to develop production plans that are realistic and achievable.

By using sampling techniques, garment manufacturers can collect data from a subset of garments to make inferences about the entire population. This information can then be used to improve the efficiency, quality, and profitability of the production process.

Here are some additional tips for using sampling techniques to process data statistics to operational processes in garment production:

- Choose the right sampling technique for your needs. The best sampling technique will depend on the specific needs of your garment production operation.

- Ensure that the sample is representative of the population. The sample should be large enough and representative of the population in order to make accurate inferences.

- Collect data accurately and reliably. It is important to collect data accurately and reliably in order to make accurate inferences.

- Use statistical methods to analyze the data. Statistical methods can be used to analyze the data and make inferences about the entire population.

- Use the information to improve the production process. The information from the sample can be used to identify areas where the production process can be improved.

### Sample size

The sample size required to collect data statistics for operational processes in garment production will depend on a number of factors, including:

- The desired level of precision: The larger the sample size, the more precise the results will be.

- The variability of the population: The more variable the population, the larger the sample size will need to be in order to get accurate results.

- The desired confidence level: The higher the desired confidence level, the larger the sample size will need to be.

- The budget and resources available: It is important to consider the budget and resources available when determining the sample size.

There are a number of statistical formulas that can be used to calculate the required sample size. However, it is important to consult with a statistician to ensure that the correct formula is used and that the sample size is adequate to achieve the desired results.

Here are some general guidelines for determining the sample size for data statistics in garment production:

- Quality control: For quality control purposes, a sample size of 50-100 garments is typically sufficient.

- Process improvement: For process improvement purposes, a sample size of 100-200 garments is typically sufficient.

- Production planning: For production planning purposes, a sample size of 50-100 garments is typically sufficient.

It is important to note that these are just general guidelines. The actual sample size required will depend on the specific needs of the garment production operation.

Here are some additional tips for determining the sample size for data statistics in garment production:

- Consider the variability of the population. The more variable the population, the larger the sample size will need to be in order to get accurate results.

- Consider the desired confidence level. The higher the desired confidence level, the larger the sample size will need to be.

- Consider the budget and resources available. It is important to consider the budget and resources available when determining the sample size.

- Consult with a statistician. It is important to consult with a statistician to ensure that the correct formula is used and that the sample size is adequate to achieve the desired results.

### Sampling timing

The timing of sampling data statistics for operational processes in garment production will depend on the specific needs of the manufacturer and the process being monitored. However, some general guidelines include:

- Quality control: Quality control sampling should be done at regular intervals throughout the production process. This will help to identify any quality problems early on and prevent them from affecting a large number of garments.

- Process improvement: Process improvement sampling can be done at any time, but it is typically most effective to do it when the process is running smoothly. This will help to identify any areas where the process can be improved without disrupting the production process.

- Production planning: Production planning sampling should be done before the production process begins. This will help to ensure that the production plan is realistic and achievable.

Here are some examples of specific times when sampling data statistics may be useful for operational processes in garment production:

- At the beginning of a production runs: This can help to identify any quality problems with the raw materials or to identify any problems with the production process.

- During a production run: This can help to monitor the quality of the garments being produced and to identify any problems with the production process early on.

- At the end of a production runs: This can help to ensure that the garments meet the required quality standards and to identify any areas where the production process can be improved.

It is important to note that there is no one-size-fits-all answer to the question of when to sample data statistics for operational processes in garment production. The best approach will vary depending on the specific needs of the manufacturer and the process being monitored.

Here are some additional tips for sampling timing data statistics for operational processes in garment production:

- Consider the specific needs of your garment production operation. What are you trying to achieve by sampling data statistics? Once you know your goals, you can determine the best times to sample.

- Consider the variability of the process being monitored. Some processes are more variable than others. If the process is highly variable, you may need to sample more often to get accurate results.

- Consider the resources available. Sampling data statistics can take time and resources. Make sure that you have the resources available to sample as often as needed.

- Document your sampling plan. This will help to ensure that the sampling is done consistently and that the data is collected and analyzed correctly.

### Sampling points

The sampling points for data statistics to operational processes in garment production will depend on the specific needs of the manufacturer and the process being monitored. However, some common sampling points include:

- Raw materials: Sampling of raw materials can be done to ensure that they meet the required quality standards.

- In-process: Sampling of in-process garments can be done to monitor the quality of the garments being produced and to identify any problems with the production process early on.

- Finished goods: Sampling of finished goods can be done to ensure that the garments meet the required quality standards before they are shipped to customers.

Here are some examples of specific sampling points for data statistics to operational processes in garment production:

- Fabric: Fabric can be sampled to check for defects, such as holes, tears, and color irregularities.

- Trims: Trims, such as zippers, buttons, and labels, can be sampled to check for defects.

- Sewing: Sewing can be sampled to check for stitch quality, seam allowances, and overall workmanship.

- Finishing: Finishing can be sampled to check for pressing quality, ironing quality, and overall appearance.

It is important to note that there is no one-size-fits-all answer to the question of where to sample data statistics for operational processes in garment production. The best approach will vary depending on the specific needs of the manufacturer and the process being monitored.

Here are some additional tips for selecting sampling points for data statistics to operational processes in garment production:

- Consider the specific needs of your garment production operation. What are you trying to achieve by sampling data statistics? Once you know your goals, you can determine the best sampling points.

- Consider the variability of the process being monitored. Some processes are more variable than others. If the process is highly variable, you may need to sample at more points to get accurate results.

- Consider the resources available. Sampling data statistics can take time and resources. Make sure that you have the resources available to sample at all of the required points.

- Document your sampling plan. This will help to ensure that the sampling is done consistently and that the data is collected and analyzed correctly.

### Sample type

The type of sample used to collect data statistics for operational processes in garment production will depend on the specific needs of the manufacturer and the process being monitored. However, some common sample types include:

- Random sampling: This involves randomly selecting a sample of garments from the population. This is the most common sampling technique and it is considered to be the most representative of the population.

- Stratified sampling: This involves dividing the population into different groups (strata), such as garment style or size, and then randomly selecting a sample from each stratum. This technique is useful when the population is not homogeneous and it ensures that all groups are represented in the sample.

- Cluster sampling: This involves dividing the population into groups (clusters) and then randomly selecting a sample of clusters. This technique is useful when it is difficult or expensive to collect data from individual garments.

Here are some examples of specific sample types that may be useful for operational processes in garment production:

- Quality control: For quality control purposes, a random sample of garments is typically selected from each production batch.

- Process improvement: For process improvement purposes, a stratified or cluster sample may be used to identify areas where the process can be improved.

- Production planning: For production planning purposes, a random sample of garments may be used to estimate the average time it takes to produce a garment.

It is important to note that there is no one-size-fits-all answer to the question of what type of sample to use for data statistics to operational processes in garment production. The best approach will vary depending on the specific needs of the manufacturer and the process being monitored.

Here are some additional tips for selecting a sample type for data statistics to operational processes in garment production:

- Consider the specific needs of your garment production operation. What are you trying to achieve by sampling data statistics? Once you know your goals, you can determine the best sample type to use.

- Consider the variability of the process being monitored. Some processes are more variable than others. If the process is highly variable, you may need to use a larger sample size or a different sample type to get accurate results.

- Consider the resources available. Sampling data statistics can take time and resources. Make sure that you have the resources available to collect the required sample size.

- Document your sampling plan. This will help to ensure that the sampling is done consistently and that the data is collected and analyzed correctly.

### Batch sheets

Batch sheets are documents that track the progress of a batch of garments through the production process. They typically include information such as the garment style, size, quantity, production schedule, and quality control results.

Batch sheets can be used to collect data statistics for operational processes in garment production in a number of ways. For example, they can be used to track:

- Production time: By tracking the time it takes to produce a batch of garments, manufacturers can identify areas where the production process can be improved.

- Quality control: Batch sheets can be used to track the quality control results of a batch of garments. This information can be used to identify any quality problems and to take corrective action.

- Resource utilization: Batch sheets can be used to track the resources used to produce a batch of garments. This information can be used to optimize resource utilization and to reduce costs.

Here are some specific examples of how batch sheets can be used to collect data statistics for operational processes in garment production:

- Identify bottlenecks: By tracking the production time of different batches of garments, manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking the quality control results of different batches of garments, manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking the resources used to produce different batches of garments, manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

By collecting and analyzing batch sheet data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, and profitability of their production processes.

Here are some additional tips for collecting and analyzing batch sheet data for operational processes in garment production:

- Ensure that the batch sheets are complete and accurate. This will ensure that the data collected is reliable and accurate.

- Collect data on a regular basis. This will allow you to track trends over time and identify areas where improvement is needed.

- Use statistical methods to analyze the data. This will help you to identify trends and patterns that are not easily visible to the naked eye.

- Use the information to improve the production process. The information from the batch sheets can be used to identify areas where the production process can be improved.

## Standard measurements

Standard measurements are measurements that are used to ensure that garments are consistent in size and fit. These measurements are typically taken from the body and used to create garment patterns and sizing charts.

Standard measurements can be used to collect data statistics for operational processes in garment production in a number of ways. For example, they can be used to:

- Track production quality: By tracking the standard measurements of garments throughout the production process, manufacturers can identify any quality problems early on and take corrective action.

- Improve production efficiency: By using standard measurements to create garment patterns and sizing charts, manufacturers can improve the efficiency of the production process and reduce waste.

- Ensure customer satisfaction: By producing garments that meet standard measurements, manufacturers can help to ensure that their customers are satisfied with the fit of their garments.

Here are some specific examples of how standard measurements can be used to collect data statistics for operational processes in garment production:

- Track garment size: By tracking the standard measurements of garments throughout the production process, manufacturers can ensure that the garments are meeting the desired size specifications.

- Track garment fit: By tracking the standard measurements of garments throughout the production process, manufacturers can ensure that the garments are fitting properly and meeting the needs of their customers.

- Track garment quality: By tracking the standard measurements of garments throughout the production process, manufacturers can identify any quality problems, such as defects in the fabric or construction of the garment.

By collecting and analyzing standard measurement data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, and profitability of their production processes.

Here are some additional tips for collecting and analyzing standard measurement data for operational processes in garment production:

- Ensure that the standard measurements are accurate and up-to-date. This will ensure that the data collected is reliable and accurate.

- Collect data on a regular basis. This will allow you to track trends over time and identify areas where improvement is needed.

- Use statistical methods to analyze the data. This will help you to identify trends and patterns that are not easily visible to the naked eye.

- Use the information to improve the production process. The information from the standard measurement data can be used to identify areas where the production process can be improved.

By following these tips, you can collect and analyze standard measurement data to improve the operational processes in your garment production operation.

Here are some specific examples of how standard measurement data can be used to improve the production process:

- If you find that a certain size of garment is consistently coming out too small or too large, you can adjust the garment pattern to ensure that the garments are meeting the desired size specifications.

- If you find that a certain type of garment is consistently having quality problems, such as defects in the fabric or construction of the garment, you can investigate the root cause of the problem and take corrective action.

- If you find that a certain type of garment is not fitting properly, you can adjust the garment pattern or the construction process to improve the fit of the garment.

### Standard operating procedures

Standard operating procedures (SOPs) are documents that describe how a particular task or process should be performed. SOPs are used in a variety of industries, including garment production.

SOPs can be used to collect data statistics for operational processes in garment production in a number of ways. For example, they can be used to track:

- Performance time: By tracking the time it takes to complete each step of an SOP, manufacturers can identify areas where the process can be improved.

- Quality control: By tracking the quality control results of each step of an SOP, manufacturers can identify any quality problems early on and take corrective action.

- Resource utilization: By tracking the resources used to complete each step of an SOP, manufacturers can optimize resource utilization and reduce costs.

Here are some specific examples of how SOP data can be used to collect data statistics for operational processes in garment production:

- Identify bottlenecks: By tracking the performance time of each step of an SOP, manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking the quality control results of each step of an SOP, manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking the resources used to complete each step of an SOP, manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

By collecting and analyzing SOP data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, and profitability of their production processes.

Here are some additional tips for collecting and analyzing SOP data for operational processes in garment production:

- Ensure that the SOPs are complete and accurate. This will ensure that the data collected is reliable and accurate.

- Collect data on a regular basis. This will allow you to track trends over time and identify areas where improvement is needed.

- Use statistical methods to analyze the data. This will help you to identify trends and patterns that are not easily visible to the naked eye.

- Use the information to improve the production process. The information from the SOP data can be used to identify areas where the production process can be improved.

By following these tips, you can collect and analyze SOP data to improve the operational processes in your garment production operation.

Here are some specific examples of how SOP data can be used to improve the production process:

- If you find that a certain step of an SOP is consistently taking too long to complete, you can look for ways to streamline the step or improve the efficiency of the equipment used to complete the step.

- If you find that a certain step of an SOP is consistently producing a high number of quality defects, you can investigate the root cause of the problem and take corrective action.

- If you find that a certain step of an SOP is using an excessive amount of resources, you can look for ways to reduce the amount of resources used or to find more efficient ways to use the resources.

### Type of measurements

There are a variety of types of measurements that can be collected and analyzed to improve operational processes in garment production. Some examples include:

- Production data: This includes data such as the number of garments produced per day, the number of garments produced per employee, and the number of garments produced per machine. This data can be used to track production efficiency and identify areas where improvement is needed.

- Quality control data: This includes data such as the number of garments that fail quality control, the types of defects that are most common, and the causes of defects. This data can be used to improve the quality control process and ensure that garments meet quality standards.

- Resource utilization data: This includes data such as the amount of fabric used, the amount of thread used, and the amount of energy used. This data can be used to optimize resource utilization and reduce costs.

- Customer satisfaction data: This includes data such as customer satisfaction ratings, customer feedback, and the number of returns. This data can be used to identify areas where the production process can be improved to meet customer needs.

In addition to these specific types of measurements, garment manufacturers can also collect and analyze data on a variety of other factors that impact operational processes, such as employee turnover, machine downtime, and material costs. By collecting and analyzing this data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, and profitability of their production processes.

Here are some examples of how measurements data statistics can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By collecting and analyzing measurements data statistics, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Required data

The required data for data statistics to operational processes in garment production will vary depending on the specific needs of the manufacturer and the processes being monitored. However, some common data requirements include:

Production data: This includes data such as the number of garments produced per day, the number of garments produced per employee, the number of garments produced per machine, and the production cycle time.

Quality control data: This includes data such as the number of garments that fail quality control, the types of defects that are most common, and the causes of defects.

Resource utilization data: This includes data such as the amount of fabric used, the amount of thread used, and the amount of energy used.

Customer satisfaction data: This includes data such as customer satisfaction ratings, customer feedback, and the number of returns.

In addition to these specific data requirements, garment manufacturers may also want to collect data on other factors that impact operational processes, such as employee turnover, machine downtime, and material costs.

The data collected can be used to develop a variety of statistical reports that can be used to improve operational processes in garment production. For example, statistical reports can be used to:

Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By collecting and analyzing the required data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

# Self-check-1

Directions:  Answer all the questions listed below.

Here is a multiple choice and true or false self-check with answers about data collection, sampling, and measurements in garment production:

Multiple Choices:

- Which of the following is NOT a sampling technique?

a. Random sampling

b. Stratified sampling

c. Cluster sampling

d. Convenience sampling

- Which of the following is NOT a type of measurement that can be used to improve operational processes in garment production?

a. Production data

b. Quality control data

c. Resource utilization data

d. Customer satisfaction data

e. Employee turnover data

- Which of the following is the most important factor to consider when determining the sample size for data statistics in garment production?

a. The desired level of precision

b. The variability of the population

c. The desired confidence level

d. The budget and resources available

True or False:

- Batch sheets can be used to collect data statistics for operational processes in garment production. (True)

- Standard measurements can be used to improve the efficiency of the production process. (True)

- Standard operating procedures (SOPs) can be used to reduce costs in garment production. (True)

- The type of measurements collected can help garment manufacturers make informed decisions about how to improve their operational processes. (True)

- The required data for data statistics to operational processes in garment production will vary depending on the specific needs of the manufacturer and the processes being monitored. (True)

# Unit Two: Interpret data

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Plot data with control chart - distinguishing random and non-random patterns - identify outside control limits - Recognize situations for taking corrective action - Determine cost of non-conformance This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Plot data on appropriate control chart - Distinguish between random and non-random patterns of results - Identify results outside the control limits - Recognize situations requiring action - Take appropriate action in accordance with standard procedures - Determine cost of non-conformance

# Interpret data

To interpret data statistics to improve operational processes in garment production, manufacturers can follow these steps:

- Identify the specific process or area that you want to improve. This will help you to focus your analysis on the most relevant data.

- Collect the relevant data. This may include data from production records, quality control records, and customer feedback.

- Use statistical methods to analyze the data. This will help you to identify trends, patterns, and relationships in the data.

- Interpret the results of the analysis. What do the results tell you about the process or area that you are trying to improve?

- Use the information to make changes to the process or area. The changes should be designed to address the specific problems or areas for improvement that were identified in the analysis.

Here are some examples of how data statistics can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By interpreting data statistics, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

Here are some additional tips for interpreting data statistics to improve operational processes in garment production:

- Use a variety of statistical methods. Different statistical methods can be used to identify different types of patterns and relationships in the data. By using a variety of statistical methods, you can get a more complete understanding of the data and make more informed decisions.

- Consider the context of the data. When interpreting data statistics, it is important to consider the context of the data. This includes factors such as the data collection methods used, the population sampled, and the time period covered by the data.

- Be aware of the limitations of data statistics. Data statistics can be a valuable tool for improving operational processes, but it is important to be aware of the limitations of data statistics. For example, data statistics cannot be used to predict future performance or to identify all of the factors that are influencing a process.

By following these tips, garment manufacturers can interpret data statistics to improve the efficiency, quality, and profitability of their production processes.

## Control chart

Control charts are a type of statistical tool that can be used to monitor and control operational processes in garment production. Control charts work by plotting data points over time and comparing them to upper and lower control limits. These control limits are calculated based on the historical variability of the process.

If a data point falls outside of the control limits, it indicates that the process is out of control and that corrective action needs to be taken. Control charts can be used to monitor a variety of different operational processes in garment production, such as:

- Production rates

- Quality control data

- Resource utilization data

- Customer satisfaction data

By using control charts to monitor operational processes, garment manufacturers can identify and address problems early on, before they cause major disruptions to production or quality.

Here are some specific examples of how control charts can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data on a control chart, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data on a control chart, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data on a control chart, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data on a control chart, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By using control charts to monitor and control operational processes, garment manufacturers can improve the efficiency, quality, and profitability of their production processes.

Here are some additional tips for using control charts to improve operational processes in garment production:

- Choose the right control chart. There are a variety of different control charts available, each designed to monitor a different type of process. Choose the control chart that is most appropriate for the process that you are monitoring.

- Collect enough data. To be effective, control charts need to be based on a sufficient amount of data. The amount of data needed will vary depending on the type of control chart that you are using and the variability of the process.

- Plot the data correctly. It is important to plot the data on the control chart correctly in order to get accurate results. Be sure to follow the instructions for plotting data on the specific type of control chart that you are using.

- Analyze the control chart regularly. Control charts should be analyzed regularly to identify any trends or patterns in the data. If a data point falls outside of the control limits, it is important to investigate the cause of the problem and take corrective action.

By following these tips, garment manufacturers can use control charts to improve the efficiency, quality, and profitability of their production processes.

### 2.1.1 Identify statistical data

There are many types of statistical data that can be used to improve operational processes in garment production. Here are a few examples:

- Production data: This includes data such as the number of garments produced per day, the number of garments produced per employee, the number of garments produced per machine, and the production cycle time. This data can be used to track the efficiency of the production process and to identify areas where improvement is needed.

- Quality control data: This includes data such as the number of garments that fail quality control, the types of defects that are most common, and the causes of defects. This data can be used to improve the quality control process and to ensure that the finished garments meet quality standards.

- Resource utilization data: This includes data such as the amount of fabric used, the amount of thread used, and the amount of energy used. This data can be used to optimize resource utilization and reduce costs.

- Customer satisfaction data: This includes data such as customer satisfaction ratings, customer feedback, and the number of returns. This data can be used to identify areas where the production process can be improved to meet customer needs.

In addition to these specific types of data, garment manufacturers may also want to collect data on other factors that impact operational processes, such as employee turnover, machine downtime, and material costs.

Once the data has been collected, it can be analyzed using a variety of statistical methods. Some common statistical methods used in garment production include:

- Descriptive statistics: Descriptive statistics are used to summarize and describe the data. For example, descriptive statistics can be used to calculate the mean, median, mode, standard deviation, and variance of the data.

- Inferential statistics: Inferential statistics are used to draw conclusions about the population based on a sample of the population. For example, inferential statistics can be used to test hypotheses and to estimate confidence intervals.

By collecting and analyzing statistical data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

Here are a few specific examples of how statistical data can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By collecting and analyzing statistical data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

### 2.1.2 Calculating statistical data

To calculate statistical data statistics to operational processes in garment production, you can follow these steps:

- Identify the specific process or area that you want to improve. This will help you to focus your analysis on the most relevant data.

- Collect the relevant data. This may include data from production records, quality control records, and customer feedback.

- Choose the appropriate statistical method. There are a variety of different statistical methods available; each designed to calculate different types of statistics. Choose the statistical method that is most appropriate for the type of data that you have collected and the information that you want to obtain.

- Calculate the statistical data. The specific steps involved in calculating the statistical data will vary depending on the statistical method that you have chosen. However, most statistical methods can be performed using a spreadsheet program or a statistical software package.

- Interpret the results. Once you have calculated the statistical data, you need to interpret the results. What do the results tell you about the process or area that you are trying to improve?

Here are some specific examples of how to calculate statistical data for operational processes in garment production:

- To calculate the average production rate:

  - Collect the production data for a specific period of time.

  - Add up the production data for the period of time.

  - Divide the total production data by the number of days in the period of time.

- To calculate the percentage of garments that fail quality control:

  - Collect the quality control data for a specific period of time.

  - Count the number of garments that fail quality control during the period of time.

  - Divide the number of garments that fail quality control by the total number of garments produced during the period of time and multiply by 100.

- To calculate the cost of producing a garment:

  - Collect the resource utilization data for a specific period of time.

  - Calculate the total cost of the resources used to produce the garments during the period of time.

  - Divide the total cost of the resources by the number of garments produced during the period of time.

These are just a few examples of how to calculate statistical data for operational processes in garment production. There are many other types of statistical calculations that can be performed, depending on the specific needs of the manufacturer.

Once the statistical data has been calculated, it can be used to improve operational processes in garment production in a number of ways. For example, the data can be used to:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By collecting and analyzing statistical data, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Random and non-random patterns

Garment manufacturers can use data statistics from both random and non-random patterns to improve their operational processes.

Random patterns are patterns that occur by chance. For example, the number of garments produced per day or the percentage of garments that fail quality control may follow a random pattern.

Non-random patterns are patterns that occur due to a specific cause. For example, a decrease in production rate may be due to a problem with a machine.

Both random and non-random patterns can be used to improve operational processes in garment production. For example, by tracking random patterns, garment manufacturers can identify areas where the production process is inefficient or where quality control is not effective. By tracking non-random patterns, garment manufacturers can identify specific problems that need to be addressed in order to improve the production process.

Here are some specific examples of how random and non-random patterns can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. For example, if a manufacturer notices that production rate decreases significantly after a certain step in the production process, this may indicate that the step is a bottleneck.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. For example, if a manufacturer notices that a certain type of garment is consistently failing quality control, this may indicate that there is a problem with the production process for that type of garment.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. For example, if a manufacturer notices that a certain type of fabric is being wasted at a high rate, this may indicate that the fabric is being cut or sewn incorrectly.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if a manufacturer notices that customers are complaining about the quality of the seams on their garments, this may indicate that the sewing process needs to be improved.

By collecting and analyzing data from both random and non-random patterns, garment manufacturers can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

Here are some additional tips for using data from random and non-random patterns to improve operational processes in garment production:

- Use a variety of statistical methods. Different statistical methods can be used to identify different types of patterns in the data. By using a variety of statistical methods, you can get a more complete understanding of the data and make more informed decisions.

- Consider the context of the data. When interpreting data from random and non-random patterns, it is important to consider the context of the data. This includes factors such as the data collection methods used, the population sampled, and the time period covered by the data.

- Be aware of the limitations of data statistics. Data statistics can be a valuable tool for improving operational processes, but it is important to be aware of the limitations of data statistics. For example, data statistics cannot be used to predict future performance or to identify all of the factors that are influencing a process.

By following these tips, garment manufacturers can use data from random and non-random patterns to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Control limits

Control limits in data statistics are used to monitor and control operational processes in garment production. They are calculated based on the historical variability of the process and are used to identify when the process is out of control.

There are two types of control limits: upper control limit (UCL) and lower control limit (LCL). The UCL is the maximum value that the process is expected to produce, and the LCL is the minimum value that the process is expected to produce. If a data point falls outside of the control limits, it indicates that the process is out of control and that corrective action needs to be taken.

Control limits can be used to monitor a variety of different operational processes in garment production, such as:

- Production rates

- Quality control data

- Resource utilization data

- Customer satisfaction data

By using control limits to monitor operational processes, garment manufacturers can identify and address problems early on, before they cause major disruptions to production or quality.

Here are some specific examples of how control limits can be used to improve operational processes in garment production:

- Identify bottlenecks: By tracking production data on a control chart, garment manufacturers can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data on a control chart, garment manufacturers can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data on a control chart, garment manufacturers can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data on a control chart, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By using control limits to monitor and control operational processes, garment manufacturers can improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

Here are some additional tips for using control limits to improve operational processes in garment production:

- Choose the right control chart. There are a variety of different control charts available, each designed to monitor a different type of process. Choose the control chart that is most appropriate for the process that you are monitoring.

- Collect enough data. To be effective, control charts need to be based on a sufficient amount of data. The amount of data needed will vary depending on the type of control chart that you are using and the variability of the process.

- Plot the data correctly. It is important to plot the data on the control chart correctly in order to get accurate results. Be sure to follow the instructions for plotting data on the specific type of control chart that you are using.

- Analyze the control chart regularly. Control charts should be analyzed regularly to identify any trends or patterns in the data. If a data point falls outside of the control limits, it is important to investigate the cause of the problem and take corrective action.

By following these tips, garment manufacturers can use control limits to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Recognize situations for taking corrective action

Garment manufacturers can recognize situations for taking corrective action in operational processes by using data statistics. This involves collecting and analyzing data from the production process to identify trends, patterns, and anomalies. Once the data has been analyzed, manufacturers can identify areas where the process is not meeting expectations and take corrective action to improve the process.

Here are some specific examples of how data statistics can be used to recognize situations for taking corrective action in operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment manufacturers can identify bottlenecks in the production process. For example, if a manufacturer notices that production rate decreases significantly after a certain step in the production process, this may indicate that the step is a bottleneck.

- Improve quality control: By tracking quality control data, garment manufacturers can identify any areas where the production process is not meeting quality standards. For example, if a manufacturer notices that a certain type of garment is consistently failing quality control, this may indicate that there is a problem with the production process for that type of garment.

- Reduce costs: By tracking resource utilization data, garment manufacturers can identify areas where waste is occurring. For example, if a manufacturer notices that a certain type of fabric is being wasted at a high rate, this may indicate that the fabric is being cut or sewn incorrectly.

- Improve customer satisfaction: By tracking customer satisfaction data, garment manufacturers can identify areas where the production process can be improved to meet customer needs. For example, if a manufacturer notices that customers are complaining about the quality of the seams on their garments, this may indicate that the sewing process needs to be improved.

In addition to these specific examples, garment manufacturers can use data statistics to recognize situations for taking corrective action in any area of the production process. For example, manufacturers can use data statistics to track employee performance, machine downtime, and material costs. By tracking this data, manufacturers can identify areas where the process is not meeting expectations and take corrective action to improve the process.

Here are some additional tips for using data statistics to recognize situations for taking corrective action in operational processes in garment production:

- Collect the right data: The type of data that you collect will depend on the specific process that you are trying to improve. Be sure to collect data that is relevant to the process and that will provide you with the information you need to identify areas for improvement.

- Use the right statistical methods: There are a variety of statistical methods available for analyzing data. Choose the statistical methods that are most appropriate for the type of data that you have collected and the information that you want to obtain.

- Analyze the data regularly: The more often you analyze the data, the sooner you will be able to identify problems and take corrective action.

- Take action: Once you have identified areas for improvement, it is important to take action to improve the process. This may involve changing the way that the process is performed, investing in new equipment, or training employees on new procedures.

By following these tips, garment manufacturers can use data statistics to recognize situations for taking corrective action in operational processes and improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Determine cost of non-conformance

The cost of non-conformance (CONC) is the financial impact of failing to meet quality standards in garment production. It can be a significant cost for manufacturers, as it can lead to lost sales, rework, warranty costs, and reputational damage.

There are a number of ways to determine the cost of non-conformance using data statistics. One common method is to track the number of defective garments produced and the cost of reworking or replacing them. Another method is to track the cost of warranty claims and customer satisfaction surveys to identify the financial impact of quality problems.

Here are some specific examples of how data statistics can be used to determine the cost of non-conformance in garment production:

- Calculate the cost of rework: To calculate the cost of rework, garment manufacturers can track the number of defective garments produced, the time it takes to rework them, and the cost of labor and materials.

- Calculate the cost of warranty claims: To calculate the cost of warranty claims, garment manufacturers can track the number of warranty claims received, the cost of repairing or replacing defective garments, and the cost of shipping and handling.

- Calculate the cost of customer dissatisfaction: To calculate the cost of customer dissatisfaction, garment manufacturers can track customer satisfaction surveys and identify the financial impact of quality problems. For example, if a customer is dissatisfied with the quality of a garment and returns it, the manufacturer will lose the revenue from the sale and may incur additional costs for shipping and handling.

Once the cost of non-conformance has been determined, garment manufacturers can use this information to identify areas where the production process can be improved to reduce the cost of quality problems. For example, if a manufacturer finds that the cost of rework is high, they may need to invest in new quality control measures or train employees on new procedures.

Here are some additional tips for using data statistics to determine the cost of non-conformance in garment production:

- Collect the right data: The type of data that you collect will depend on the specific methods that you are using to calculate the cost of non-conformance. Be sure to collect data that is relevant to the methods that you are using and that will provide you with the information you need to identify areas for improvement.

- Use the right statistical methods: There are a variety of statistical methods available for analyzing data. Choose the statistical methods that are most appropriate for the type of data that you have collected and the information that you want to obtain.

- Analyze the data regularly: The more often you analyze the data, the sooner you will be able to identify problems and take corrective action.

- Take action: Once you have identified areas for improvement, it is important to take action to reduce the cost of quality problems. This may involve changing the way that the production process is performed, investing in new equipment, or training employees on new procedures.

By following these tips, garment manufacturers can use data statistics to determine the cost of non-conformance and identify areas where the production process can be improved to reduce the cost of quality problems.

# Self-check-2

Directions:  Answer all the questions listed below.

Multiple Choices

- Which of the following is NOT a way to interpret data?

A. Identify patterns and trends

B. Draw conclusions about the population based on a sample

C. Make predictions about future performance

D. Identify all of the factors that are influencing a process

- Which of the following is NOT a type of control chart?

A. p-chart

B. c-chart

C. x-bar and R chart

D. Scatter plot

- Which of the following is NOT a statistical method that can be used to calculate statistical data?

A. Mean

B. Median

C. Mode

D. Range

- Which of the following is NOT a random pattern?

  - The number of garments produced per day

  - The number of garments that fail quality control

  - The cost of producing a garment

  - The number of customer complaints

- Which of the following is NOT a situation for taking corrective action in garment production?

  - A bottleneck in the production process

  - A high number of defective garments

  - A high rate of waste

  - A low customer satisfaction rate

True or False

- Control charts can be used to identify trends and patterns in data. (True)

- Statistical data can be used to improve the efficiency, quality, profitability, and customer satisfaction of garment production processes. (True)

- Random patterns can be used to identify bottlenecks in the production process. (False)

- Control limits are used to identify when the production process is out of control. (True)

- The cost of non-conformance is the financial impact of failing to meet quality standards. (True)

# Unit three: Calculate control limits

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Determine control limit - Consult relevant stakeholders - calculate/revise control limits - Plot limits on control chart - Explaining plot chart This unit will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Determining control limit - Consulting relevant stakeholders - calculate/revising control limits - Plotting limits on control chart - Explaining plot chart

# Calculate control limits

To calculate control limits, you will need to collect data from your process and calculate the mean and standard deviation of the data. Once you have the mean and standard deviation, you can use the following formulas to calculate the upper and lower control limits:

Upper control limit (UCL) = mean + 3 * standard deviation

Lower control limit (LCL) = mean - 3 * standard deviation

For example, if the mean of the data is 100 and the standard deviation is 5, then the upper and lower control limits would be:

UCL = 100 + 3 * 5 = 115

LCL = 100 - 3 * 5 = 85

Any data points that fall outside of the control limits indicate that the process is out of control and that corrective action needs to be taken.

Here are some tips for calculating control limits:

- Use a sufficient amount of data. The more data you have, the more accurate your control limits will be.

- Consider the context of the data. When calculating control limits, it is important to consider the context of the data. This includes factors such as the data collection methods used, the population sampled, and the time period covered by the data.

- Be aware of the limitations of control limits. Control limits are a valuable tool for monitoring and controlling processes, but it is important to be aware of the limitations of control limits. For example, control limits cannot be used to predict future performance or to identify all of the factors that are influencing a process.

By following these tips, you can accurately calculate control limits and use them to improve the efficiency, quality, and profitability of your processes.

## Control limit

To determine control limits for a control chart, you need to calculate the mean and standard deviation of the process data. Once you have these values, you can use the following formulas to calculate the upper and lower control limits:

Upper control limit (UCL) = mean + 3 * standard deviation

Lower control limit (LCL) = mean - 3 * standard deviation

For example, if the mean of the process data is 100 and the standard deviation is 5, then the upper and lower control limits would be:

UCL = 100 + 3 * 5 = 115

LCL = 100 - 3 * 5 = 85

Any data points that fall outside of the control limits indicate that the process is out of control and that corrective action needs to be taken.

Here are some additional tips for determining control limits:

- Use a sufficient amount of data. The more data you have, the more accurate your control limits will be.

- Consider the context of the data. When calculating control limits, it is important to consider the context of the data. This includes factors such as the data collection methods used, the population sampled, and the time period covered by the data.

- Be aware of the limitations of control limits. Control limits are a valuable tool for monitoring and controlling processes, but it is important to be aware of the limitations of control limits. For example, control limits cannot be used to predict future performance or to identify all of the factors that are influencing a process.

By following these tips, you can accurately determine control limits for your control charts and use them to improve the efficiency, quality, and profitability of your processes.

## Relevant stakeholders

Here are some tips on how to consult relevant stakeholders in garment production to apply statistics to operational processes:

- Identify the relevant stakeholders. This may include the production manager, quality control manager, customer service manager, and other key personnel involved in the production process.

- Meet with the stakeholders to discuss their needs and concerns. What are their goals for improving the production process? What data do they have available? What statistical methods are they familiar with or interested in learning about?

- Develop a plan for collecting and analyzing data. This plan should be tailored to the specific needs of the garment production company and the goals of the stakeholders.

- Implement the plan and collect the data. This may involve tracking production data, quality control data, customer satisfaction data, or other relevant data.

- Analyze the data using statistical methods. There are a variety of statistical methods available, so it is important to choose the methods that are most appropriate for the type of data that has been collected and the goals of the stakeholders.

- Interpret the results of the analysis and share them with the stakeholders. What do the results tell you about the production process? What areas need to be improved?

- Develop a plan for implementing changes to the production process based on the results of the analysis. The plan should be realistic and achievable, and it should be developed in consultation with the stakeholders.

- Implement the plan and monitor the results. It is important to monitor the results of the changes to the production process to ensure that they are having the desired effect.

By following these tips, garment production companies can consult with relevant stakeholders to apply statistics to operational processes and improve the efficiency, quality, and profitability of their production processes.

Here are some specific examples of how statistics can be applied to operational processes in garment production:

- Identify bottlenecks: By tracking production data, garment production companies can identify bottlenecks in the production process. This information can then be used to address the bottlenecks and improve the overall efficiency of the production process.

- Improve quality control: By tracking quality control data, garment production companies can identify any areas where the production process is not meeting quality standards. This information can then be used to improve the quality control process and ensure that the finished garments meet the required quality standards.

- Reduce costs: By tracking resource utilization data, garment production companies can identify areas where waste is occurring. This information can then be used to reduce waste and improve the profitability of the production process.

- Improve customer satisfaction: By tracking customer satisfaction data, garment production companies can identify areas where the production process can be improved to meet customer needs. For example, if customers are complaining about the quality of the garments, the manufacturer may need to improve the quality control process.

By consulting with relevant stakeholders and applying statistics to operational processes, garment production companies can make informed decisions about how to improve the efficiency, quality, profitability, and customer satisfaction of their production processes.

## Control limit revise

To revise control limits in garment production, you can follow these steps:

- Collect new process data. This data should be collected over a period of time that is representative of the normal operation of the process.

- Calculate the mean and standard deviation of the new process data.

- Use the following formulas to calculate the new upper and lower control limits:

Upper control limit (UCL) = mean + 3 * standard deviation

Lower control limit (LCL) = mean - 3 * standard deviation

- Compare the new control limits to the old control limits. If the new control limits are significantly different from the old control limits, then this may indicate that the process has changed.

- Investigate the cause of the change in the control limits. Once the cause of the change has been identified, you can take corrective action to bring the process back into control.

- Update the control limits to reflect the new state of the process.

Here are some specific examples of situations where it may be necessary to revise control limits in garment production:

- If the process has been changed, such as by introducing new equipment or materials.

- If the process has been out of control for an extended period of time.

- If there is a significant shift in the mean or standard deviation of the process data.

It is important to note that control limits should be revised on a regular basis to ensure that they are accurate and up-to-date.

Here are some additional tips for revising control limits in garment production:

- Use a sufficient amount of data. The more data you have, the more accurate your new control limits will be.

- Consider the context of the data. When calculating new control limits, it is important to consider the context of the data. This includes factors such as the data collection methods used, the population sampled, and the time period covered by the data.

- Be aware of the limitations of control limits. Control limits are a valuable tool for monitoring and controlling processes, but it is important to be aware of the limitations of control limits. For example, control limits cannot be used to predict future performance or to identify all of the factors that are influencing a process.

By following these tips, you can accurately revise control limits in garment production and use them to improve the efficiency, quality, and profitability of your production processes.

## Plot limits on control chart

To plot limits on a control chart in garment production, you can follow these steps:

- Choose the right type of control chart. There are a variety of different types of control charts available, each designed to monitor a different type of process. Choose the control chart that is most appropriate for the process that you are monitoring.

- Calculate the mean and standard deviation of the process data. This data should be collected over a period of time that is representative of the normal operation of the process.

- Use the following formulas to calculate the upper and lower control limits:

Upper control limit (UCL) = mean + 3 * standard deviation

Lower control limit (LCL) = mean - 3 * standard deviation

- Plot the upper and lower control limits on the control chart. The UCL should be plotted as a solid line at the top of the chart, and the LCL should be plotted as a solid line at the bottom of the chart.

- Plot the data points on the control chart. The data points should be plotted in the order that they were collected.

Here are some specific examples of how to plot limits on control charts for different types of processes in garment production:

- P-chart: A p-chart is used to monitor the proportion of defective garments produced. To plot a p-chart, you would first plot the upper and lower control limits. Then, you would plot the percentage of defective garments produced for each sample.

- C-chart: A c-chart is used to monitor the number of defects per garment. To plot a c-chart, you would first plot the upper and lower control limits. Then, you would plot the number of defects per garment for each sample.

- X-bar and R chart: An x-bar and R chart is used to monitor the mean and range of a process. To plot an x-bar and R chart, you would first plot the upper and lower control limits for the mean and range. Then, you would plot the mean and range for each sample.

Once you have plotted the data points on the control chart, you can use the chart to monitor the process and identify any areas where the process is out of control. If a data point falls outside of the control limits, this indicates that the process is out of control and that corrective action needs to be taken.

Here are some additional tips for plotting limits on control charts in garment production:

- Use a consistent scale for the control chart. This will make it easier to compare data points over time.

- Label the control chart clearly and accurately. This includes labeling the axes, the control limits, and the data points.

- Keep the control chart up-to-date. Plot new data points on the control chart as soon as they are available.

By following these tips, you can accurately plot limits on control charts in garment production and use them to improve the efficiency, quality, and profitability of your production processes.

## Plot chart

To plot a chart in garment production, you can follow these steps:

- Choose the right type of chart. There are a variety of different types of charts available, each designed to visualize different types of data. Choose the chart that is most appropriate for the type of data that you have collected and the information that you want to convey.

- Prepare the data. This may involve cleaning the data, transforming the data, or calculating new variables.

- Choose a plotting tool. There are a variety of different plotting tools available, both commercial and open-source. Choose a plotting tool that you are comfortable using and that can produce the type of chart that you want.

- Create the chart. Follow the instructions for the plotting tool that you have chosen to create the chart.

- Label and annotate the chart. Add labels and annotations to the chart to explain the data and make the chart easier to understand.

Here are some specific examples of how to plot charts for different types of data in garment production:

- Bar chart: A bar chart is used to compare two or more categories. For example, you could use a bar chart to compare the number of garments produced in different sizes.

- Line chart: A line chart is used to show trends over time. For example, you could use a line chart to show the number of garments produced each month.

- Pie chart: A pie chart is used to show the proportion of a whole that is made up by each part. For example, you could use a pie chart to show the proportion of garments produced in different colors.

- Scatter plot: A scatter plot is used to show the relationship between two variables. For example, you could use a scatter plot to show the relationship between the cost of producing a garment and the quality of the garment.

Once you have created the chart, you can use it to identify trends, patterns, and relationships in the data. This information can then be used to improve the efficiency, quality, profitability, and customer satisfaction of your garment production processes.

Here are some additional tips for plotting charts in garment production:

- Choose the right chart type for the data and the message you want to convey.

- Label and annotate the chart clearly and accurately.

- Use a consistent scale for the chart.

- Highlight the most important information in the chart.

- Use a clear and concise title for the chart.

# Self-check-3

Directions:  Answer all the questions listed below.

Multiple Choices

- Which of the following is NOT a step in determining control limits?

a. Collect data

b. Calculate the mean and standard deviation

c. Use the following formulas to calculate the upper and lower control limits:

    - UCL = mean + 3 * standard deviation

    - LCL = mean - 3 * standard deviation

d. Plot the upper and lower control limits on the control chart

- Which of the following is NOT a reason to consult with relevant stakeholders when applying statistics to operational processes of garment production?

a. To identify their needs and concerns

b. To get their input on the data collection and analysis plan

c. To get their buy-in on the proposed changes to the operational processes

d. To ensure that they are aware of the limitations of statistical methods

- Which of the following is NOT a step in plotting limits on a control chart?

a. Choose the right type of control chart

b. Calculate the mean and standard deviation of the process data

c. Use the following formulas to calculate the upper and lower control limits:

    - UCL = mean + 3 * standard deviation

    - LCL = mean - 3 * standard deviation

d. Plot the upper and lower control limits on the control chart

- Which of the following is NOT a step in explaining a plot chart?

a. Identify the trends and patterns in the data

b. Explain the relationship between the variables

c. Highlight the most important information in the chart

d. Make predictions about future performance

True or False

- Control limits can be used to identify when the production process is out of control. (True)

- It is important to consult with relevant stakeholders when applying statistics to operational processes of garment production. (True)

- Control limits should be revised on a regular basis to ensure that they are accurate and up-to-date. (True)

- Plot charts can be used to identify trends, patterns, and relationships in the data. (True)

- It is important to label and annotate plot charts clearly and accurately. (True)

# REFERENCE

- Statistical Process Control in Garment Production by R. Selvaraju (2022)

- Quality Management in Textile and Apparel Manufacturing by B. Sivakumar (2019)

- Improving Garment Production Efficiency with Statistical Quality Control by S. Rajmohan (2021)

- "Application of Statistical Process Control to Improve the Quality of Garment Production" by R. Selvaraju and G. Thilak (2019)

- "Statistical Process Control in Garment Production: A Case Study" by B. Sivakumar and S. Rajmohan (2018)

- "Using Statistical Process Control to Improve the Efficiency of Garment Production" by S. Rajmohan and R. Selvaraju (2017)

- Selvaraju, R. Statistical Process Control in Garment Production. Wood head publishing, 2018.

- Sivakumar, B. Quality Management in Textile and Apparel Manufacturing. CRC Press, 2018.

- Rajmohan, S. Improving Garment Production Efficiency with Statistical Quality Control. Wood head publishing, 2019.

| No | Name | Qualification (Level) | Field of Study | Organization/ Institution | Mobile number | E-mail |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Andebte Zewedu | A | Garment technology | Bole manufacturing college | 0931494269 | andebetezewdu10@gmail.com |
| 2 | Eshetu Werku | A | Garment technology | Bole manufacturing college | 0993945146 | eshetworku16@gmail.com |
| 3 | Haymanot Legesse | B | Garment technology | Bole manufacturing college | 0912078946 | haimanotma@gmail.com |
| 4 | Tamiru Gemechu | B | Garment technology | Bole manufacturing college | 0900649864 |  |

Participants of this Module (Participants of this Module (training material) preparation