Footwear Production

Level-II

Based on November, 2021 Curriculum Version 1

Module Title: Performing Footwear Finishing and Packaging

Module Code: IND FWP2 M08 1221

Nominal duration: 30 hours

Prepared By: Ministry of Lobar and Skill

August, 2022

Addis Ababa, Ethiopia

# Acknowledgment

Ministry of Labor and Skills wish to extend thanks and appreciation to the many representatives of TVET instructors and respective industry experts who donated their time and expertise to the development of this Teaching, Training and Learning Materials (TTLM).

TABLE OF CONTENTS

Acknowledgment	ii

Introduction to the Module	iv

Unit one: Preparation of Tools, Materials Equipment’s and Machines	5

1.1.Materials and components for finishing	6

1.2.Tools and equipment	17

1.3.Common problems and faults of materials	23

1.4. Personal protective equipment	25

1.5. Safety of operator and work place	26

Self chek-1	30

Unit Two: Preparation of  workstation and set up the machine	32

2.1.Setting up and arranging work station	33

2.2.Collecting, sorting, laying out and preparing footwear for finishing operation	34

2.3.Setting-up, adjusting, cleaning and maintaining finishing machineries	36

Self chek-2	48

Unit Three: Perform Finishing and packaging of the work	49

3.1.Types of leather finishes	50

3.2.Finishing processes	55

3.3.Finishing operations	57

3.4.Packaging and labeling materials	72

3.5.Checking products	75

3.6.Packaging operations	76

3.7.Prevent accidents and to eliminate risks to personal safety	82

3.8.Completing production and other records	84

Self chek-3	86

Operation sheet 3.1: Finishing and packaging	88

LAP Test	5

Unit Four: Checking final quality of the finished pair and dispatch	6

4.1. Identifying quality check points	7

4.2. Checking footwear	10

4.3. Identify rectify and return faults	14

4.4. Quality records and documentation	19

Self chek-4	24

REFERENCES	25

# Introduction to the Module

The process, which enhances the appearance and restores the inherent characteristics of the Upper leather, is called Shoe Finishing. Finishing is done to upgrade the quality and the aesthetic appeal of the product or to give some special appearance, e.g. Shadow effect or Antique effect. It also imparts the desired color and level appearance to the material.

This module is designed to meet the industry requirement under the footwear production occupational standard, particularly for the unit of competency: Performing Footwear Finishing and Packaging

This module covers the units:

- tools, materials, equipment and machine preparation

- workstation preparation and machine set up

- Finishing and packaging of the work

- final quality of the finished pair and dispatch

Learning Objective of the Module

At the end of this session, the students will able to:

- Prepare tools, materials, equipment and machine

- Prepare workstation and set up the machine

- Perform Finishing and packaging of the work

- Check final quality of the finished pair and dispatch

Module Instruction

For effective use this modules trainees are expected to follow the following module instruction:

- Read the information written in each unit

- Accomplish the Self-checks at the end of each unit

- Perform Operation Sheets which were provided at the end of units

- Do the “LAP test” if it is provided at the end of units and

- Read the identified reference book for Examples and exercise

# Unit one: Preparation of Tools, Materials Equipment’s and Machines

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Materials and components for finishing. - Tools and equipment. - Common problems and faults - Personal protective equipment - Safety of operator and work place This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify and select Materials and components used for footwear finishing as per the required specification. - Identify and made ready Tools and equipment as per the required work specification for operation. - Identify and rectify Common problems and faults of materials - Identify Suitable personal protective equipment for safety with regards to tools, equipment and machines. - Describe Safety of operator and work place for finishing operation.

## Materials and components for finishing

1.1.1 Introduction

Shoe making is a rough and tough process.  Most  of  the  operations  during  its  manufacture  are  meant  for  giving  it  a  proper  shape.  During  process,  the  upper  material  is  subjected  to  a  wide  variety  of  abrasive  and  stretching  operations,  which  can  adversely  affect  its  appearance  and  sometimes  even  in  heart  its  characteristics.

The  process  which  enhance  the  appearance  and  restores  the  inherent  characteristics  of  the  upper  leather  is  called  “ shoe finishing”.  Finishing  is  done  to  upgrade  the  quality  and  the  aesthetic  appeal  of  the  product  or  to  give  some  special  appearance  like  shadow  effect  or  Antique  effect.  It  also  impart  the  desired  color  and  level  appearance  to  the  material. Hence  good  shoe  finishing  will  enhance  the  image  of  our  enterprise  and  product  brand  image

1.1.2 Common materials and components used in finishing operation

Waxes

Waxes are used for burnishing, repairing and polishing. They are of different types as shown below:

Figure 1. 1 Types of wax

Abrasive Wax: Used for cutting or smoothening of grain surface, and for filling and special effects, for example brush-off and burnish effect. These are available in three colors: black, brown and natural. This is to be applied on cotton roller.
Repair Wax: these are gap-filling crayons and do not shrink on flexing.
Burnishing Wax: It is a medium-hard dyed wax, which enhances the burnishing effect on burnished leather. This is to be applied on cotton roller.
Polishing Wax: Used as the final operation to polish and protect the finish that is done in the shoe finishing room. It also enhances the gloss of the shoe. This is to be applied on woolen roller.
Two types of polishing wax are used:

- Synthetic and

- Carnauba.

Available in three colors Black, Brown and Natural. White Polishing wax is synthetic carnauba wax whereas natural carnauba wax is yellowish in colour.

Spray gun and spray booth for spraying operation;

Figure 1. 2 Spray gun and spray booth for spraying operation

Spraying Technique
The technique of spraying is defined in the above figure

- Spray must be applied evenly;

- The spray gun is always started before applying the spray which
prevents heavy application of dressing;

- Spray in smooth strokes; and

- The airflow and dressing flow should be adjusted which ensures neither dry dressing nor flooded wet dressing. Flooding will result in increase of drying times and tear marks.

Figure 1. 3 Spraying Technique

Cream

Shoe cream is a type of shoe polish that contains dye. In addition to restoring shine and suppleness to your leather shoes, shoe cream can brighten up the color of your shoes and cover faded spots and scratches. Once you’ve picked the right cream polish, take a little time to clean and prep your shoes. Then you’re ready to apply the shoe cream and get your shoes looking brand new again!

Creams could be soft, medium or hard. Soft and medium creams are those that require to be polished, and are referred to as the creams to be polished. Hard creams are available as self-shining creams. That is, they don’t require cutting or brushing operation necessarily. Creams are applied by sponge – sea sponge or synthetic sponge.

Figure 1. 4 Applying cream

Cleaner / polishers

Before  starting  finishing  operation,  ensure  that  the  surface  to  be  finished  has  been  clean. The  purpose  of  cleaning  is  that  no  any  dust  or  contaminant  are  present  on  the  upper  surface  of  the  shoe.

The main purpose is to remove:

- The Dirt

- Adhesive overspill

- Other unwanted marks and contamination

Cleaning  can  be  done  before  or  after  removing  the  last  from  the  shoe. The  latter  is  preferable  for  better  shape  retention  of  the  shoe. When necessary, cleaning of lining is also done.

Cleaner/  polishers  are  supplied  as  gels  and  contain  solvent  cleaners  and  polishing  agents,  which  remain  on  the  surface  after  the  solvents  have  evaporated.

Application  is  by  sponge  or  cloth  and  after  drying  can  be  polished  on  a  power  mob.

These  are  very  suitable  for  patents  and  PU  coated  fabrics,  and  have  the  advantage  of  giving  an  acceptable  semi-  bright finish  without  having  to  spray.

Figure 1. 5 solvent cleaners

Sole edge colors

A  water  base  edge  containing  pigment  waxes  and  gums  is  applied  by  hand  to  the  sole  edge.  A machine may be used.  The  edge  must  be  completed  and  evenly  covered  with  the  ink,  but  there  must  be  no  ink  on  the  upper  and  stitching.

Sole  edge  colors  are  very  important  to  improve  the  appearance  of  the  bottom  parts  of  the  shoe,  with  the  application  of  these  coloring  materials,  the  water  resistance  of  the  heel  increases.  Its  final  appearance  is  like  shined  looks  a  original  finish  on  heels  and  soles.

Features and directions-

- Shake well with cap on.

- Shoe must be clean and dry.

- Invert  bottle  and  gently  press  applicator  against  shoe  to  release  product.

- Spread  polish  evenly  and  allow  to  dry  to  a  shine.

- Rinse  applicator  with  water  after  use  and  blot  dry  to  keep  foam  soft,  replace  cap.

- If  spilled,  clean  up  before  dry,  Gently  dab  spill  with  a  dry  cloth,  the  wipe  with  a  damp  cloth.

Figure 1. 6 sole edge colours

Pigments for repairing

Pigment  represent  a  very  important  constituents  of  present  day  leather  and  shoe  finishing  composition.  The  use  of  pigments  in  the  finishing  had  its  start  in  America  during  the  World  War First, because  the  supply  of  dyestuff  from  Germany  was  cut  off.  While  pigmented  finishes  lacked  the  brilliance  and  beauty  of  finishes  based  on  the  dyestuffs,  they  have  the  advantages  of  covering  stains  and  defects  and  presenting  a  uniform  color  over  the  whole  skin  or  hide.  This  advantages  becomes  more  and  more  important  as  time  went  on  since  increasing  amount  of  buffed    side  leather  and  decreasing  amount  of  full- grain  leather ( calf )were  being  finished. As  a  result,  even  when  dyestuffs  become  available  again,  the  use  of  pigments  was  never  given  up.  The  pure  aniline  finish  is  still  used  on  a  considerable  amount  of  calf  and  this  is  covered  by  the  use  of  dye  solution  and  small  amount  of  pigment  if  needed.

Glazed  kid  is  also  finished  with  aniline  finishes  as  are  a  number  of  novelty  leathers.  In  a  general  way,  the  big  volume  leathers  are  finished  with  coating  which  contains  pigments.

Table 1. 1 pigments normally use in leather finishes

| White | Titanium dioxide |
| --- | --- |
| Black | Carbon black, bone black, iron oxide |
| Yellow | Lead chromate |
| Orange | Lead chromate |
| Brown | Iron oxide |
| Blue | Iron blue |
| Green | Chrome green |
| Red | Cadmium sulphide |

For  repairing  of  any  major  or  minor  defects  we  take  any  suitable  pigment  with  addition  of  filler  and  adhesives.  Apply the season in suitable amount, then dry well.  After  that  removing  the  excess  season  with  the  help  of  fine  emery  paper  then  finishing.

Figure 1. 7 repairing pigments

Labels/tags

There should be two labels on your shoes; the first label tells you the origin of the manufacturing and the second label is a breakdown of the construction of the shoe. Footwear must be labelled with the main materials used for the upper, lining and sock, .and outer sole where multiple materials are used, the two main materials in the composition of the footwear must be stated.

The label must be attached to at least one item of footwear in each pair and may be affixed by way of printing, sticking, embossing or use of an attached label; it must be visible, securely attached and accessible. The label may also appear on the packaging but it must be on the footwear itself.

Pictograms for the parts of the footwear

Upper:                               Lining and sock:              Outer sole:

Figure 1. 8 Components of the footwear

Pictograms for the materials used in footwear composition

Leather:                         Coated leather:              Textile:                      Other materials:

Figure 1. 9 materials used in footwear composition

Shoe Boxes:

- All shoes should be packed in a shoebox

- Each shoebox must have a finger hole on the lid of each box. The box should be of a good grade cardboard.

- The lid needs to be fixed to the box.

- The box should not be of a slick coating that will make the box hard to handle.

- All shoes must face one direction.

- All footwear shoeboxes are required to be labeled with a UPC-A, EAN or interleaved 2of 5 labels.

- Stickers should be placed on the lower left or right-hand corner of one end of the box so as not to cover product information. The sticker should not be applied to the box lid except in cases when the lid covers most or whole of the box (overlay lid).

- A sticker with the Foot Locker SKU should be affixed on same side as the barcode sticker.

- American size (as ordered on PO) must always be clearly marked on box

Figure 1. 10 Shoe box

- Shoeboxes should be placed vertically in single layer. Bar code to face up.

- Only one SKU and one size per master carton, unless specified differently on the
purchase order (i.e. case lots)

NOTE: In cases when shoes are not packed in a shoebox, f.e. flip-flops it should be packed in a polybag, the barcode sticker should than be affixed to the polybag.

Tissue paper

It is recommended that you wrap shoes in tissue paper and place them in the original shoeboxes to prevent damage from contact with other shoes. This will protect patent finishes and ornaments from accidental damage during transit. Do not use newsprint to wrap your shoes!

Figure 1. 11 Tissue paper

Silica gel

Shoe Manufacturers Use Silica Gel for Moisture-Control. From manufacturing to storing, delivery, and product pick-up, the shoe manufacturers add silica gel packets to the box to keep shoes safe from mold, odor, and bacteria growth, especially if they're made of leather

Figure 1. 12 Silica gel

Dye/colorant

The kind of leather will dictate what type of dye you should use to achieve the right dye color and the right leather finish. Consider these different leather dyes and their applications:

- Shoe polish: Shoe polish works as a more temporary kind of leather dye. The polish does not penetrate the leather’s fibers, making this dye less permanent, though it is quick and easy to apply.

- Alcohol-based leather dyes: These dyes penetrate the surface of the leather, creating more vivid colors that last a long time. Alcohol may strip the moisture out of the leather, stiffening the material, so you may need a leather conditioner or finisher to restore the material.

- The sun: The sun can be a natural way to dye leather. Leaving your veg-tanned leather out in the sun will darken its hue. This process will take at least an hour, more depending on how dark you want your leather as the sun’s ultraviolet rays will take a lighter leather and turn it to a dark brown over time.

- Oil-based leather dyes: Unlike alcohol-based dyes, oil-based ones do not strip the leather of liquids. Oil-based dyes cut through the surface of the leather to color the fibers to a richer saturation. These may work best for full and top grain leather without surface finishes applied, meaning those fibers will dry out more readily.

- Water-based leather dyes: Water-based dyes have fewer toxins than other dyes but produce more muted colors. They will leave your leather supple.

- Resolene: You can apply leather resolene, or acrylic dye, in thin layers as a leather finish. After drying, the thin layers of acrylic will harden into a shiny, solid, and protective surface atop your leather; this can work for corrected grain leather, whose fibers will have been sanded down so the resolene offers a durable finish. Different finishes and colors are available for resolene, making it a versatile leather dye option.

- Resin: A leather finisher made from natural resin will produce a subtler and softer finish and provide some protection from dirt. Resin-based finishers are not water-resistant and can work well.

To properly dye leather, you’ll need a well-ventilated space and several materials, which you can find at crafting or leather goods store:

- Deglazer: A leather deglazer is a solution that cleans the leather and prepares it for an efficient dye process.

- Spray bottle: After deglazing, you’ll want to spray your leather gently with some water to help it more easily absorb your first coat of dye.

- Paint brush: You will use a paint brush or wool dauber to apply your coat of dye.

- Leather dye: The kind of dye you select will determine the color and affect your leather’s outcome, feel, and quality.

- Scrap rag: An old scrap piece of cloth will be helpful to have during the dyeing process to wipe away excess dye and sop up spills.

- Gloves: Dye can get messy, so you’ll want to wear gloves that cover your forearm for protection. Dishwashing gloves will get the job done.

- A ventilated and protected workspace. You’ll want your work area to be well-ventilated as the dye can release fumes that are dangerous to breathe. Open your windows and turn on a fan if indoors, and if outdoors, be sure it’s a dry day and stay out of direct sunlight. Use a drop cloth or cut open a large trash bag and tape it down on your workspace to keep your station clean.

## Tools and equipment

Blunt knife

In  the  lasting  operation, the  complete  surface  of  the  bottom  of  the  shoe  is  coated  with  a  rubber  solution  or  thermoplastic  adhesive ( hot  melt ).  The  blunt  knife  is  used  to  remove  wax  and  thermoplastic  adhesives  ( hot  melt ).  Could  be  used  on  suede  and  nubuck  leather  and  for  repair  work

Figure 1. 13 blunt knife

Crepe and resin rubber

Generally  used  to  remove  polyurethanes,  latex,  and  rubber  adhesives  from  grained  upper  and  lining.

- Crepe rubber

It  is  a  natural  product  produced  by  coagulation  the  latex  from  the  rubber  tree.  Latex  is  formed  into  thin  sheets, and  then  laminating  and  pressing  a  good  number  of  sheets  together  to  give  a  specific  thickness. Chemically  it  is  a  polymer  made  from  “ Isoprene” monomer  units  which  has  sufficient  strength  not  to  need  vulcanizing. The  crepe  which  is  harder  and  lighter  in  color  provides  better  wear  comfort.

Advantages

- It is comparatively cheap to process.

- It has good wear resistance.

- It  has  self-sealing  ability  and  provides  a  very  good  resistance  to  flex  crack  but  slips  on  dry  surface.

- It is very light in weight (0.88 gms. / c.c.)

- Resilience is high and the strength is moderate.

Disadvantages

- It  is  very  sensitive  to  heat  and  become  soft  and  sticky  in  worm  condition. It has tendency to spread under body weight.

- It has a poor resistance to ageing.  If  it  is  exposed  to  heat  of  sunlight  it  shows  surface  hardening  and  cracking.

- It is sensitive to solvents mainly petroleum and oil.

- Natural rubber

Natural rubber is a cultivation based product.  It  depends  upon  the  suitability  of  the  plantation  of  heveabrasiliensis  tree.  The research work to develop an artificial rubber began.  The styrene and butadiene are the two main raw materials.  A  styrene  butadiene  copolymer  made  in  emulsion  with  a  charge  ratio  of  25% styrene  and  75%  butadiene  were  chosen  as  the  best  possible  general  purpose  rubber.

Figure 1. 14 natural rubber

Sponge

Shoe Shine Sponge is an easy-to-use application that gives your shoes a bright and long-lasting shine. Acord shoe shine sponge is a great tool that can use to shine your shoes with a matter of seconds by quick swipes anywhere you are, whether in the office or even on the go as it easy to carry and take no space.

Step to shine shoes with a sponge

Start by wiping away any loose dirt or dust from the shoe with a cloth. Then apply the shoe sponge. Polish will be automatically released from the sponge. Simply glide the shoe-shine sponge lightly over the leather until you're satisfied with your shine.

Emery paper

Emery  paper  is  a  type  of  paper  that  can  be  used  for  sanding  down  hard  and  rough  surfaces.  Emery paper is sold in sheets of varying sizes.  Emery  products  choose  on  the  base  of  the  level  of  abrasiveness  or  grit.  In  the  shoe  industry  it  is  used  for  the  following  purposes

- To roughen the pleats/ crease on lasted shoe bottom.

- To  roughen  the  surface  of  lasted  upper  and  bottoms  material  to  be  attached  with  adhesives.

- To  roughen  ( finish ) the  edge  of  micro  rubber  sole.

- To  roughen  the  surface  of  bottom  ( leather )  of  shoe  sole  to  give  a  smooth  surface  before  applying  bottom  finish.

- Crust  leather  is  scoured  with  300- 360  grit  of  abrasive  to  smooth  the  surface  of  leather  before  making  corrected  grain  leather.

Emery paper available in the market in various nos.  Such as No- 24- 60, No- 80- 120, No- 150- 240, No- 300- 360 etc.  These numbers represent the grits of the abrasive paper.  An  international  system  is  used  by  the  abrasive  determines  the  number  of  grits  to  a  squire  inch  of  material.  As  such  the  number  grits  is  bigger  it  means  more  numbers  of  grits  are  within  one  squire  inch  area, and  that  will  be  fine  grit.  In  the  same  manner  for  smaller  grit  it  will  be  reverse.  The  coarser  grit  is  commonly  used  for  roughing  edge  of  the  upper  before  cementing  however  a  fine  grit  is  used  to  scour  the  grain  of  leather  sole  to  give  a  smooth  surface.

Figure 1. 15 emery paper

Inking brushes

Brushes  are  differ  greatly  in  terms  of  size,  texture,  material  and  cost.  It  is  made  from  goat,  Siberian  weasel,  pig,  mouse,  buffalo,  wolf  and  rabbit  hair  while  exotic  once  can  be  made  from  tiger,  fowl, deer,  and  even  human  hair.

The inking brushes are three types.

- Big  size  brushes

- Medium  size  brushes

- Small  size  brushes

The  smallest  brushes  are  used  for  very  small  pieces  and  for  fashioning  designs  for  seals.  Medium  brushes  are  most  widely  used  because  it  can  produce  a  variety  of  thickness  of  line  ,  from  very  thin  to  fairy  thick.  The larger brushes are used only for very large pieces.  In footwear, the brushes are used to color the edges.  A  water  based  edge finish  containing  pigment,  waxes  and  gums  is  applied  by  hand  to  the  sole  edge.  A machine may be used.  The  edge  must  be  completely  and  evenly  covered  with  ink,  but  there  must  be  no  ink  on  the  upper  and  sole.  The  flesh  side  of  the  sole  has  a  ¼  inch  margin  of  ink  applied.  This  is  done  to  minimize  the  danger  of  ink  being  applied  to  the  upper  when  the  sole  in  edge  ink  during  finishing.

Figure 1. 16 inking brushes

Suede (wire) brush

This is soft brass wire brush.  Used  to  remove  dust  and  loose  foreign  substances  from  suede  and  nubuck  leather.  Brushed lightly in one direction to give same nap.

Suede  brush  cleans  and  brushes  up  the  knap  in  the  suede.  If  the  suede  is  shiny  use  a  little  piece  of  120  grit  sand paper  they  bring  up  the  knap  even  more.  In  this  way  we  use  shoe  brush  100%  horse  hair  they  provides  the  high  gloss  shine  in  suede  leather. Similarly  use  shoe  shine  cloth,  after  polishing,  it  provides  extra- bright  high  gloss  shine.  The bars removes shiny marks and stains.  The  brush  brings  the  nap  of  the  leather  to  its  original  appearance.

Suede  and  nubuck  wire  brush  to  raise  the  nap  of  the  leather  on  shoes,  boots,  hand  bags  or  any  other  suede / nubuck  items.  Especially good for reaching hard- to- get place.

For  the  cleaning  of  suede  and  nubuck  product  manufacturer  use  the  following  products  for  their  long  life  preservation.

- Suede  and  nubuck  protector

- Suede  and  nubuck  cleaner

- Tarra  go  suede  dye: -  To  restore  color  to  old  suede  shoes  and  accessories.

- Suede and nubuck protector

Suede and nubuck protector provides protection against water and stains.  Keep suede and nubuck shoes, boots, coats, bags etc.  Looking new.  Coats  footwear  with  a  special  polymer  that  creates  an  invisible  shield  to  provide  maximum  protection  against  water  and  stains.

- Suede and nubuck cleaners

The  cleaners  cleans,  conditions  and  softens  suede  and  nubuck  leather.  Removes  dust  and  dirt  from  shoes,  boots,  belts,  clothing  and  more.  The  cleaners  built  in  bristle  top  makes  application  and  cleaning  easy  and  mess  free.

Figure 1. 17 suede & nubuck cleaners

## Common problems and faults of materials

1.3.1 Faults of finishing

Many faults can occur in finishing. Some of them are due to wrong finishing method or some due to environmental factors. Some common faults and their causes are listed in table below:

Table 1. 2 finishing faults and their cause

| Faults | Causes |
| --- | --- |
| Mist or fog | Air pressure is too high. Gun far away |
| Uneven finish | Wrong dressing selection. Poor cleaning |
| Rough finish | Insufficient fluid feed. Gun too far away. Air pressure too high. |
| Streaks | Uneven spray pattern. Disturbance of finish during cleaning or filling. Gun not square to work |
| Runs, sags, tear marks | Over application –flooding .gun not square on worker movement too slow |

1.3.2 Problems with Dressings

Table 1. 3 Problems with Dressings

| Problem | Causes | Remedy |
| --- | --- | --- |
| Flaking | Dressing and material finish is not compatible Inadequate cleaning Contaminated cleaner pots and dirty sponges Other contamination | Check suitability of dressing Clean more thoroughly Identify and eliminate source |
| bloom | High humidity, water in solvents or air supply. Heat or solvent cause fat to move to surface. Solvents may cause whitening | Some anti-bloom cleaners are available or check air supply warm gently to disperse bloom, excessive heat is avoided usually cured by over spraying with butyl acetate (B.A) special cleaners and finishes are required |

## 1.4. Personal protective equipment

In addition to being aware of the mechanical hazards in the finishing section, it is important that you use the correct protective clothing and equipment. Wearing personal protective equipment (PPE) can prevent accidents from happening. As a worker, you are responsible for the following:

- Making sure your uniform is well fitted.

- Keeping all uniforms clean and in good condition, not frayed or badly worn.

- Making sure sleeves are kept buttoned at the wrist, cuffs on overalls and trousers are be eliminated, and trouser legs are long enough to hang outside boots.

- Wearing specific personal safety equipment such as goggles, hearing protection, gloves, and aprons when required.

To ensure that you are protecting yourself, your personal protective equipment (PPE) list should include the following items.

- Clothing

This includes well-fitted pants and jackets with all buttons fastened. Sleeves should be close fitting because sleeves that are loose and flowing are potential fire hazards when working over open gas burners. Health regulations require that all machine operator wear hair nets or use other approved methods for keeping hair under control. Aprons should be made of non-combustible and flame-resistant materials that do not melt under heat.

- Footwear

The OHS Regulation requires that approved footwear must be worn by employees in all industrial occupations. Ensure your footwear is sturdy and provides enough back support to not cause future back problems. Footwear suitable for commercial foodservice establishments must have a non-slip sole and a closed toe and closed back.

Your footwear should be sturdy and comfortable, and if the environment you work requires steeled toes, such footwear should be worn. High leather tops on shoes are a good idea as they will protect your feet from hot grease or liquids.

- Hand protection

The most common type of gloves used in component section are natural rubber latex gloves, synthetic rubber gloves, and vinyl gloves.

- Eye protection

Eye protection in the form of safety goggles or masks should be worn whenever there is a chance of eye injury. Particles flying through the air can easily land in your eye and possibly do permanent damage.

- Hearing protection

Approved hearing protection must be worn when high-level noise conditions exist.

- Respirators

Respirators should be used to protect yourself from inhaling harmful fumes or vapors. The respirator unit should be properly fitted to provide the best protection. Check the components to ensure they are not broken, cracked, or torn and that they do not have holes. Replace faulty components before use. Each unit will have a filter that should be checked regularly and replaced before the expiration date.

## 1.5. Safety of operator and work place

Safety and health occupies a very significant position and in hazardous occupations. It aims to protect the health and strength of all workers. It prevents employment in occupations unsuitable for the age and strength of the workers. It is the policy of the state to make provisions for securing just and humane conditions of work. The constitution provides a broad framework under which policies and programs for occupational health and safety could be established.

The factory occupier must facilitate the following facilities for worker’s welfare and their health:

- To ensure cleanliness of the workplace;

- Make effective arrangement for treatment and disposal of waste and effluent;

- Make suitable and effective provisions for adequate ventilation;

- Maintain temperatures to secure reasonable comfort for workers;

- Remove any dust or fumes from the workplace which may be injurious to workers;

- Prevent overcrowding by maintaining a specific cubic area for each worker;

- Provide sufficient and suitable light;

- Make suitable arrangements to provide clean drinking water conveniently situated for all workers and;

- Provide suitable latrines and urinals to specified standards.

The factory occupier must provide basic safety measures including:

- Securely guarding all parts of dangerous machinery;

- Precautions for working on machinery;

- Emergency devices for cutting off power;

- Maintain hoists and lifts;

- Lifting machines, chains, ropes, and other lifting tackle must be maintained in good condition;

- Ensure walking surfaces are of sound construction;

- Provide protective equipment;

- Measures to remove gas and dust before entering confined places;

- Measures to prevent fires.

The factory occupier must disclose information about:

- Dangers, health hazards, and measures to protect workers from substances or materials in manufacture, transportation, storage etc. to the workers, the chief factory inspector, and the local authority;

- Safety and policy;

- Quantity and characteristics and disposal of substances and waste;

- Emergency plans to workers and the local public;

- Handling, using, transportation, storage and disposal of hazardous substances to workers and the local public.

The employer should maintain up-to-date health records of workers, and to appoint a person experienced in handling hazardous substances to supervise handling, and provide protective measures and regular medical examinations.

Equipment Safety: Extreme care should be taken when operating equipment. Before you attempt to operate any tool or piece of equipment, you must be fully trained by an experienced operator. Make sure that all guards are in place and function properly and that all electrical connections are properly made. You should observe the following precautions when using equipment:

- Understand the correct operating procedures and safety precautions before operating a piece of equipment.

- Ensure that all guards are in place and functioning before any machine is started.

- Report defective or unsafe equipment to a responsible individual to prevent serious injury.

- Do not distract or interfere with the equipment operator.

- Make sure that the cords to electrically powered tools are in good condition, with no frayed parts or bare wires showing and make sure that the tools are properly grounded.

- Keep edge-cutting tools properly sharpened so that they do the job well and do not have to be forced because of dull edges.

- Use tools only for their intended use and make sure the size of the tool is right for the job.

- Report to your immediate supervisor any tool or piece of equipment that is broken or does not function properly.

## Self chek-1

Directions:  Answer all the questions listed below. Examples may be necessary to aid some explanations/answers.

Test I: True or False

- Dispersing and emulsifying agents are water based cleaners.

- In suede leather grains are visible.

- Abrasive  wax  is  a  good  polishing  wax  to  provide  the  high  gloss.

Test II: Multiple Choice

- Which type of wax are used as the final operation to polish and protect the finish that is done in the shoe finishing room. It also enhances the gloss of the shoe

- Burnishing wax

- polishing wax

- repair wax

- abrasive wax

- Which one of the following is used to brighten up the color of shoe and cover faded spots and scratches in addition to restoring shine and suppleness to your leather shoe?

- Cream,

- cleaner,

- tags

- tissue paper

- Which one of the following are problems with dressing

- Uneven finish,

- rough finish

- flaking

- none

- which type of products  manufacturers use For  the  cleaning  of  suede  and  nubuck  product for  their  long  life  preservation.

- Suede and nubuck protector,

- suede and nubuck cleaner ,

- A and B,

- none

Test III: Fill in the blanks:

- Shoe finishing is a _______and __________process.

- ____________machine  is  used  to  provide  the  glaze  on  glace  goat  leather.

Test IV: Short Answer Questions:

Instruction: write short answer for the given question.

- List out and describe some of the personal protective equipment’s?

- Describe briefly about a factory occupier must facilitate related to operator and workplace safety?

# Unit Two: Preparation of workstation and set up the machine

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Setting -up and arranging workstation - Collecting, sorting, laying out and preparing footwear for finishing operation. - Setting-up, adjusting, cleaning and maintaining finishing machineries. - Cleaning and maintaining routine finishing machineries. This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Set-up and arrange workstation according to work specifications and OHS practices. - Collect, sort, laid out and prepare Footwear for finishing operation. - Set-up and adjust clean and maintain Finishing machineries adjusted for operation according to task requirements.

## Setting up and arranging work station

Workstations are used primarily to perform computationally intensive scientific and engineering tasks. They have also found favor in some complex financial and business applications.

2.1.1 Work station set up

To Set up Your Workstation follow the following steps

- Push your hips as far back as they can go in the chair.

- Adjust the seat height so your feet are flat on the floor and your knees are equal to, or slightly lower than, your hips.

- Adjust the backrest to a 100°-110° reclined angle.

- Adjust the armrests so that your shoulders are relaxed.

Figure 2. 1 Workstation setup

## Collecting, sorting, laying out and preparing footwear for finishing operation

2.2.1 Preparation of footwear for finishing operation

Insock Fitting

This operation consists of placing and/or gluing the insock. Nowadays preformed insocks are used, which are only inserted inside the shoe, and there is no need to glue to fix the insock. This type of insocks is used mainly in sports or casual footwear.

Figure 2. 2 Insock fitting

For gluing the insock there are several processes used, namely:

- Mechanical glue application: the glue is applied onto the insock by means of a machine with a glue roller, where the insock passes before an operator inserts it into the shoe,

- Manual glue application: An operator applies the glue with a brush on the back of the insock and then fits it into the shoe.

Edge Inking

This operation is performed when upper leather edges (open edges) are visible and do not have the color of the upper leather (for leather which has not been through-dyed), or when the lining edge is visible.

In these cases, it is necessary to dye the edges in the hue of the upper leather.

Figure 2. 3 Edge Inking

Cleaning / Repair

At this stage the shoe is checked and cleaned, which consists of removing glue residues, which can be found in the area where the sole is joined to the upper and inside the upper. Various tools, equipment and products can be used for cleaning. The type of cleaning to be performed depends on the shoe materials. To remove the glue from the assembling edge, the glue cleaning machine can be used. For the materials in which this process cannot be used, use a rubber crepe, a wooden spatula, a water sandpaper or a flannel cloth soaked in a suitable cleaning liquid.

Figure 2. 4 Cleaning / Repair

Burning Thread Ends

With this operation it is intended to eliminate the small thread ends that sometimes stay from the sole stitching or from the upper stitching.

For the execution of this operation a small burner with an open flame can be used (but be careful not to leave burn marks on the shoe).

## Setting-up, adjusting, cleaning and maintaining finishing machineries

- Brushing machine

This  operation  is  required  for  finishing  of  leather  and  to  give  shining  or  polishing  effect  on  upper  leather.  This  is  also  converting  or  merges  the  course  grain  leather  into  fine  grain  upper  leather.  To  produce  or  bring  the  burnishing  effect,  two  tone  effect,  polishing  effect  to  the  upper  part  of  the  shoes  with  the  help  of  suitable  waxes.

Figure 2. 5 brushing machine

Generally  this  operation  carried  out  in  shoe  room  with  the  help  of  brushing  machine,  with  provision  for  cotton  and  wool  brushing  for  cutting  and  polishing  purpose. In  polishing,  normally  use  roller  of  brushing  machine  at  low  speed  with  the  use  of  polishing  i, e, synthetic  wax  or  carnauba  wax. Whereas  when  high  abrasive  action  is  not  required  then  rollers  of  brushing  machine  at  medium  speed  with  the  use  of  abrasive  wax. If  rules  do  not  follow  the  above  method  provides  uneven  shining  and  creates  problem  in  shoe  finishing.

Brushing  machine  have  surface  preparation  abrasive  brush  with  all  the  necessary  technical  devices  required  to  obtain  a  perfect  finish  on  the  flat  surface.  Machines  are  designed  to  suit  individual  customers  applications  and  requirements. It  provides  the  uniform  brushing  effect  on  the  grain  surface  of  the  shoe.

The technical specification of this machine is-

- Height     175 cm

- Width      130  cm

- Depth      85  cm

- Weight     350  kg

- Power       3  Kw

Machine setups and adjustments

- Switch on the machine whenever required.

- Grain  leather  to  be  brushed  well,  as  well  as  poromeric  materials also.

- Use  either  the  soft / hard  cloth  or  woolen  brush  as  per  the  requirements  of  finishing.

- We  must  maintain  the  following-

- Machine  speed  should  be  900-  1100 rpm,  400- 500  rpm  from  cloth  brushing  as  well  as  woolen  brushing  respectively.

- Use appropriate wax for finishing.

Cloth brushing--------- Abrasive wax

Woolen brushing------ Carnauba wax

- The  course  grain  surface  to  be  polished  or  merged  well.

- To  bring  the  shining  or  polishing  effect  to  the  shoe.

Machine operation instructions

Safety

- Switch off the machine when not in use.

- Switch  on  the  dust  extractor  while  operating  the  machine  in  order  to  prevent  spreading  of  harmful  dust.

- Operators  are  required  to  tie  their  wearing  cloths  of  similar  loose  parts  of  their  dress  at  their  back.

- Do  not  work  more  than  one  person  on  the  machine  at  a  time  in  one  place.

- Wear the mask while on working on the machine.

- Clean the machine thoroughly every day.

- Do  not  wear  loose  sleeved  garments  as  the  sleeves  may become  caught  while  operating  the  machine,  wear  tight  on  overalls.

Ironing machine (long boots)

Specifications

- Long  boot  ironing  machine  is  a  two  station  forming  machine  with  heated  metal  lasts,  suitable  for  long boot.

- Each  station  is  equipped  with  an  independent  regulators  to  adjust  the  stretching  of  uppers  according  to  the  type  and  number  of  the  uppers.

- Each  last  is  equipped  with  a  separate  mini-  safety  switch  protecting each  resistance  independently.

Figure 2. 6 ironing machine

This  machine  is  well  equipped  and  it  is  useful  for  ladies  shoes,  Gentlemen  shoes,  Ironing  leather.  It  contains  and  used  in  boot  ironing  machine,  aluminum  molds,  heating  device,  ironing  long  boots,  pedant  switch  etc.

Features

- This machine equips with heating device.

- The aluminum mold is separated to upper and lower parts.  They expands long boots.

- The aluminum mold can be rotated 180 degree.  It is for the convenience of ironing long boots.

- A  pedant  switch  is  equipped  in  the  machine  for  easy  operations.

- It  is  automatic  machine  saves  man  power  and  increases  efficiency.

- Electric counter device to control weight for easy replacement.

- Low breakdown rate, high machinery life- span.

- High revolving speed results in large production.  Compact machine size contribute to factory space saving.

- Two styles of aluminum molds can be loaded simultaneously.  This machine equips   with heating device.

This  machine  is  useful  to  chase  the  wrinkle  in  ladies  and  gents  long  boot  with  the  help  of  heating  rollers.

Machine operation instructions

Safety

- Switch off the machine when not in use.

- Operators  are  required  to  tie  their  wearing  cloths  of  similar  loose  parts  of  their  dress  at  their  back.

- Do  not  work  more  than  one  person  on  the  machine  at  a  time  in  one place.

- Clean the machine thoroughly every day.

- Wear the mask while on working on the machine.

- Do  not  wear  loose  sleeved  garments  as  the  sleeves  may become  caught  while  operating  the  machine,  wear  tight  on  overalls

Hand irons

Figure 2. 7 hand irons

Hand  irons  are  used  in  footwear  and  garments  industry  to  remove  the  wrinkles  and  fold  marks.  It  is  used  to  improve  the  quality  and  produce  the  shining  of  apparel  and  allied  products.

Generally  the  quality  of  the  footwear  and  garments  is  measured  in  three  characteristics:

- Durability

- Utility

- Emotional  appeal

With  respect  to  the  raw  materials  used,  the  product  design  and  the  constructions  of  the  product.  Durability  factors  are  tensile  strength,  tear- strength,  abrasive  resistance,  color  fastness  and  cracking  and  bursting  strength.  Utility  factors  are  air  permeability,  water  permeability,  thermal  conductivity,  crease  retention,  wrinkle  resistance,  shrinkage  and  soil  resistance.  Appeal  factors  are  eye  appeal  of  fabric  or  leather  /  leather  garments  face,  tactile  response  to  the  fabric  surface,  fabric  hand  and  eye  appeal  of  the  leather  garments  face,  design  and  drape.  The  principals  involved  are  the  same  whether  the  garments  is  made  of  leather,  plastic,  foam  or  textile  such  as  women,  knit  or  felt  fabric.

Pressing and molding process

Molding  is  the  process  that  change  the  surface  characteristics  or  topography  of  a  garment  or  shoe  or  one  of  its  section  by  application  of  heat,  moisture  or  pressure.  Pressing,  pleating,  blocking,  mangling,  steaming,  creasing,  curing  and  casting  are  trade  terms  for  various  molding  processes  in  producing  clothing  and  footwear.

- Pressing has two major divisions.

- Buck  pressing

- Iron  pressing

A  buck  press  is  a  machine  for  pressing  a  garments  or  section  between  two  contoured  and  heated  pressure  surfaces  that  may  have  steam  and  vacuum  systems  in  either  or  both  surfaces.  Before  1905,  all  garments  pressing  was  done  by  hand  irons  heated  directly  by  gas  flame,  stove  plate  heat  or  electricity.

In iron pressing,    a hand iron functions as top pressure surface.  The  two  major  types  of  hand  irons  are  steam  ejectors  and  dry  irons.  Electric hand irons are equipped with thermostats that regulate temperature.  Steam  heated  irons,  whether  ejection  or  dry,  have  fixed  temperatures  depending  on  the  pressure  of  the  steam  supplied  to  the  iron.  Many  hand  irons  are  equipped  with  lift  devices  and  gear  devices  to  control  stroke  rate  and  minimize  operator  fatigue.  Hand  irons  are  made  in  a  variety  of  sizes,  weights,  shapes  and  surfaces,  the  specific  usage  determines  the  combination.

Top- line forming machine

Figure 2. 8 top-line forming machine

This  machine  is  applicable  to  final  moulding  and  finishing,  before  removed  and  packed,  melt  the  glue  of  the  heel  deformed  with  electric  heating  and  then  cool  it  down  for  moulding.  The  heel  after  moulding  will  have  a  straight  and  perfect  shape.  In  addition,  the  heating  and  cooling  will  remove  the  damp  so  as  to  keep  it  from  mold  growth.

- Free  regulated  temperature  and  heating  time  upon  the  material  of  vamp.

- Time  can  be  adjusted  when  the  temperature  reaches  150  degree  centigrade.  It has a good moulding effect.

Machine operation instructions

Safety

- Switch off the machine when not in use.

- Operators  are  required  to  tie  their  wearing  clothes  of  similar  loose  parts  of  their  dress  at  their  back.

- Do  not  work  more  than  one  person  on  the  machine  at  a  time  in  one  place.

- Wear the mask while on working on the machine.

- Clean the machine thoroughly every day.

- Do  not  wear  loose  sleeved  garments  as  the  sleeves  may  become  caught  while  operating  the  machine,  wear  tight  on  overalls.

Spray cabin

On  most  spray  guns  for  top  spraying,  the  following  adjustment  are  required-

- Adjust  material  flow  by  the  screw  that  controls  how  much  liquid  is  being  sprayed  when  trigger  is  fully  depressed.

- Adjust the spray patterns.

- The  shape  of  the  spray  pattern  on  some  guns  can  be  altered  between  round  and  oval.

- “ Pear  drop “ and  “ banana”  shape  are  incorrect  and  likely  to  lead  to  uneven  coverage.

- Take care of the spray pattern position.  This  is  important  on  oval spray  patterns  which  should  be  vertical.

- Adjust air pressure.

Note  that  air  pressure  is  not  controlled  on  the  gun,  but  usually  on  an  air  pressure  controller  somewhere  near.

The quality depends on: -

- Supply  of  clean,  compressed  air

- Selection of suitable guns, nozzle sizes and air caps.

- Good maintenance of the equipment.

- Adequate extraction of the fume generated during spraying.

- Spraying method.

Maintenance of spray gun

Correct cleaning of the gun will give the following results-

- It provides trouble free running.  At  the  end  of  the  day  flush  out  the  gun  and  cup  with  water  or  spirit  such  as  acetone.  The  cleaner  can  be  left  in  the  gun  overnight    providing  the  equipment  is  clean.

- Never totally immerse the spray gun in sprit.  This can damage the packing.

- Oil the needle and air valve daily.

- Clean  air  and  fluid  holes  with  compressed  air  or  a  bristle.  NEVER USE WIRE.

Spraying technique

A  technique  should  be  developed  to  obtain  an  overall  even  coat  of  the  dressing,  without  streaks,  runs,  misted  or  rough  finish.  The following are some factors that are important-

- Check spray patterns

The  spray  patterns  should  be  checked  momentarily  spraying  on  a  piece  of  waste  material  such  as  cardboard  held  squire  to  the  gun.  Uneven  patterns  are  usually  due  to  obstructions  or  damage  at  the  fluid  tips.

- Check air pressure to the gun.  This should be 2.8 to 3.5 Bar. Or  (  40 to 50 psi ).

- Hold  the  shoes  and  boots  putting  the  hand  inside  the  best.  The  hand  is  used  to  present  the  shoe  at  the  correct  angle  ( not  to  move  it  past  the  gun ).  The  hand  will  protect  the  inside  of  the  shoe  from  over  spray.

- A glove should be worn for protection.

- By  holding  the  shoe  on  the  top  of  the  heel  may  result  in  leaving  marks  on  successive  shoes  from  the  accumulation  of  spray  on  the  thumb  and  fingers .  The  shielding  effect  of  fingers  and  thumb  can  also  leave  noticeable  areas  not  sprayed  on  the  side  of  the  heel.

- Have  correct  altitude,  and  distance  of  the  gun  from  the  shoe—

The  nozzle  of  the  gun  should  be  kept  squire  to  the  upper  surface,  at  the  distance  of  30  cm. ( 12  inches ).  The  fluid  flow  should  be  adjusted  to  give  sufficient  dressing  to  “ wet  out “  causing  runs.

The  best  method  may  be  to  start  at  the  toe,  moving  along  the  feather  edge,  up to  back seam,  along  the  top- line ,  over  the  instep,  along  the  other  side  top  line,  down  the  back  seam,  along  the  feather  to  the  toe,  and  up  the  center  of  the  vamp  to  the  top  of  the  quarters.

The  gun  must  be  moved  with  one  hand,  while  the  other  presents  the  shoe  in  the  right  altitude.  The  spray  should  be  started  and  finished  JUST  OFF  the  shoe,  and  must  be  kept  moving  at  a  constant  speed  over  the  surface.

- Ensure  drying

Once  spray  is  completed,  very  great  care  must  be  taken  to  avoid  touching  the  wet  surface,  or  allow  anything  to  contact  it  until  it  is  thoroughly  dry.

Application procedure

- Use  good  quality  sponges  and  clean  the  same  with  water  before  and  after  application.

- For  cleaning  the  shoe  use  clean  starch  free  cotton  cloth  and  change  the  cotton  cloth  regularly.

- Always  allow  some  time  ( 5 minutes )  for  drying  all  water  based  products,  in  between  each  operation.

- Before  and  after  spraying  water  based  products,  clean  the  gun  with  water  only.

- Solvent  based  products  to  be  sprayed  at  less  pressure ( 45Psi ).

- Before  and  after  spraying  solvent  based  products,  clean  the  gun  with  solvent  thinner  or  acetone  only.

- Water based products at more pressure (60- 70 Psi).

- For  safety  measure  clean  the  surface  of  shoe  ( degrease )  after  application  of  abrasive  and  before  applying  the  top  dressing.

Machine operation instructions

Safety

- Switch off the machine when not in use.

- Operators  are  required  to  tie  their  wearing  cloths  of  similar  loose  parts  of  their  dress  at  their  back.

- Do  not  work  more  than  one  person  on  the  machine  at  a  time  in  one  place.

- Wear the mask while on working on the machine.

- Clean the machine thoroughly every day.

- Do  not  wear  loose  sleeved  garments  as  the  sleeves  may  become  caught  while  operating  the  machine,  wear  tight  on  overalls.

Figure 1 machine operation

## Self chek-2

Directions:  Answer all the questions listed below. Examples may be necessary to aid some explanations/answers.

Test I: Multiple Choice

1 which one of the following is intended to eliminate the small thread ends that sometimes stay from the sole stitching or from the upper stitching

- Burning thread ends,

- cleaning,

- edge inking,

- insock fitting

2----------------is the machine  that  change  the  surface  characteristics  or  topography  of  a  garment  or  shoe  or  one  of  its  section  by  application  of  heat,  moisture  or  pressure.

- Ironing machine

- spray cabin,

- molding machine,

- none

Test II: Short Answer Questions:

Instruction: write short answer for the given question.

  - How to setup workstation?

  - Describe the basic concepts of cleaning and maintaining?

  - Describe about benefits of cleaning and maintaining machineries of finishing section?

# Unit Three: Perform Finishing and packaging of the work

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Types of leather finishes. - Finishing processes I and II. - finishing operations - packaging and labeling materials - Checking products as per customer specification - packaging operations - Taking action to prevent accidents and to eliminate risks to personal safety. - Completing production and other records. This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify different types of leather finishes. - Identify and describe finishing processes I and II. - Perform finishing operations - Identify and sort packaging and labeling materials - Check products as per customer specification - Perform packaging operations - Taking action to prevent accidents and to eliminate risks to personal safety. - Complete production and other records.

## Types of leather finishes

Those are the common finishes available in leather used for upper making

Aniline finish

This is the most natural type of leather which earned it the names Pure Aniline or Full Aniline. It is dyed with soluble dyes in vats so that the colour goes all the way through the hide giving the leather its lush and rich colour. In order to maintain the natural feel and look of the hide no protective coating or surface pigmentation is applied. Additionally, the leathers maintain their natural markings and texture which contributes to the authenticity of the leather. However, the lack of treatment might render the leathers more susceptible to scratching, staining, or fading and might show some colour variations on the surface of the hide.

Some aniline leathers are sanded of buffed to create an open nap; the result is called Nubuck. These leathers are the softest kind available and have a beautiful, almost velvety, feel. Yet, they are also the most susceptible to fading or staining due to the open nap.

Figure 3. 1Aniline finish

Why use them?

Quality, natural looks, design aesthetic. They are generally the higher price pointed leathers. And have a very natural surface appearance. You will generally require more aniline leather if you do not wish to see imperfections. Aniline hides fade and mark—they build up a natural patina over time.

Semi Aniline Leathers
First things first: In this case "semi" is actually a misnomer, as it does not mean partial. A more appropriate name for semi aniline leather would be aniline plus pigment due to the way it is created. The process starts the same way as for aniline leathers. However, whereas the treatment of full aniline stops after the dying, semi aniline leathers are further treated. So, after the leather is aniline dyed, a slight protective coating, which can contain some additional pigmentation, is applied to the top of the hide to guarantee a consistent colour. Due to the pigmented coating some blemishes will be concealed leaving a more uniform surface texture. Additional finishes can then be applied to the leather either by hand or by machine (e.g. antique looks or exotic leather optics). In the end, a clear protective coating is applied to make the leather stain repellent and sun resistant. The sheen of this coating can be adjusted to give a matte or glossy finish.

Why use them?

Amazing range of colors, practical leathers, Smoother surface. Or you simply
prefer this look. They are generally the mid-price pointed leathers. And have a light painted type appearance.

Corrected grain or Pigmented:

Just like the other two types above, it is dyed all the way through the hide. However, the surface is then coated with a heavy protective topcoat which includes pigmentation. Any kind of blemishes or imperfections are removed through buffing or sanding and then embossed with an artificial grain. At the expense of a deep rich colour and a luxurious feel, pigmented leathers are treated with spray coating which makes it more durable. Therefore, pigmented leathers are the least natural grain leather but also the most durable.

Why use them?

Hardwearing, practical leathers, great range of colors. Or you simply prefer this look. They are generally the lower price pointed leathers. And have a painted type appearance.
Burnish:

Burnishing is the process of buffing leather edges to a slick, glossy finish. Burnishing is aesthetic it looks nice to have the shiny edge. But it also serves a functional purpose. When you burnish leather, you seal the cut edge by applying friction. The heat from the burnishing tool compresses the fibers of the leather together. By sealing the edge in this way, the leather retains its natural oils and moisture and won’t dry out. It also creates a waterproof barrier so the leather won’t absorb moisture. This helps prevent your thread from rotting.

Figure 3. 2 Burnishing

Burnishing works best on vegetable tanned leather, as even thin, lightweight veg tan is stiff enough to hold up to the friction of a burnishing tool. You can burnish before or after dyeing a leather edge, though we recommend burnishing after to maintain a high gloss finish. This finish is normally given on F/G leather.

Brush Off:

Brush off leather is leather which if brushed by heavy cotton brush gives a two tone effect. Normally, the leather in tannery is finished with two or three coats of finish. The two finish layers are either in contrast colour or contrast shade. The top coat is not fixed and on brushing hard gets off. Hence, the name given to the finish is Brush off. You brush the surface and the top finish is off making the bottom finish layer visible giving leather a two-tone effect. This leather is called brush off.

The brush-off leather consists of a skin whose surface has been sanded and subsequently covered with a layer of uniform dye, it can be glossy or opaque and the ideal use of this leather is footwear, but it can also be used for belts and leather goods in general. Among the various types of leather processing, the brush-off one is very particular as the finishing that is performed on the leather which makes it very uniform to the eyes and to the touch. In practice, it is a polishing that is performed by hand or by machine with precise movements, only an expert craftsman can perfectly succeed in this task. With this treatment, the skin is without any imperfections and the processing guarantees excellent resistance.

Metallic:

Metallic leather or Metallized leather is leather with a metallic look. Metallic effects are generally only found on smooth leather. This effect is achieved by using metal colours in finish, used for ladies or kids shoes Metallic effects are commonly used for fashion reasons in leather shoes, clothing and bags.

Figure 3. 3 Metallic leather shoes from 1859 and 2014

Pull Up:

Pull up is a finishing technique, not strictly speaking a type of leather any kind of leather can be pull up. A popular finishing technique for top grain leather, the pull up finishing technique infuses hot oil and/or wax into the hide. The intricate finish allows for lines to form in the leather when pulled or stretched. This pulling creates the desirable “two tone effect.” “To me, the beauty of pull up isn’t that it looks older or aged, it’s the gorgeous two tone color variation it can achieve,” says Tucker Gasho, the owner of K&H Leatherworks, What makes leather pull up or not is if you get the color change when it’s manipulated. ”

Figure 3. 4 Pull Up

Full grain leather.

Full grain is leather from the upper part of the hide, and the outer surface has not been polished or corrected. If you look very closely at the leather, you can actually see the hair follicles

Full grain leather has no surface alterations. The hide’s natural pores and grain textures are intact and it will develop a patina over time. The tiny pinholes dotting the surface indicate the hide’s open hair follicles. The presence of these hair follicles demonstrates a high quality surface which has not been altered to conceal flaws. This full grain surface breathes. It keeps the user comfortable as it adjusts to body temperature. Full grain leather is the highest quality, most beautiful, and most comfortable leather available.

Characteristics of Full Grain Leather

No matter how advanced the tannery, each and every piece of leather produced is unique in character. A quality hide’s natural origins are displayed on its surface. Look for these essential characteristics of full grain quality:

- Fat wrinkles or growth lines: subtly shaded bands on the grain side of a hide

- Natural variations in grain texture and shading.

- Small scars: on a quality product they do not render large portions of the hide unusable.

- A similar shade of color on the suede side: this demonstrates superior base dye penetration.

- The depth and shading of color on the grain side is comparable to that of a transparent color stain on a piece of fine hardwood furniture

Choosing full grain leather is the smart, long-term solution if comfort, quality, and durability are important.

## Finishing processes

There are 2 types of shoe finishing system or methods.

- System I shoe finishing process

- System II shoe finishing process

- System I shoe finishing process

It is done separately after lasting and sole attaching operations. This system is used for all types of leather finishes /shoe finishing.

System I shoe finishing process as follow: -

- Cleaning

- Thread burning or cutting – Loose thread is cut or burn using split lamp, hot blower or thread trimmer

- Edge coloring - done using pigment, dye, or cream to color the edge of shoe
and to make it similar with shoe surface.

- Shoe repairing

- Shoe ironing

- Base coat application

- Cutting

- Top coat cream application

- Light cutting - using cotton roller, just light touch of shoe surface

- Polishing – using woolen roller

- Socks attaching

- Lacing

- Stuffing - is inserting tissue paper in to the shoe to maintain the shape of shoes.

- Quality checking

- Packing

- System II shoe finishing process

This system is mostly used for pigmented or corrected grain finish leather which heavily coated, cream applied and pass-through heat setting for more penetration finish or cream. The finishing process started before heat setting and sole attaching, after Lasting Operation completed.

System II shoe finishing process as follow: -

- Cleaning

- Thread burning or cutting

- Edge coloring

- Shoe repairing

- Shoe ironing

- Cutting

- Base coat application

- Pass through heat setting

- Ironing

- Cutting operation

- Do rest of lasting operations to de-lasting

- Dry cleaning

- Base coat application as

- film coat

- Drying

- Light cutting

- Top coat application

- Light cutting

- Polishing

- Socks attaching

- Lacing

- Stuffing

- Quality checking

- Packing

## Finishing operations

Following are the basic Footwear finishing operations

- Cleaning (Sole and Upper)

Sole cleaning in lasted upper is done after sole attaching operation due to remove the excessive adhesive. In addition of that upper surface is also clean to remove dirt, adhesive and other unwanted marks prepare the surface for further finishing operations. Cleaning can be done by two ways: -

• Dry cleaning - Dry cleaning in sole can be done by Edge brushing machine, crape rubber or resin rubber. In upper dry cleaning can be done by cotton cloth, crape rubber or brushes.

Figure 3. 5 Dry cleaning

• Wet cleaning - Wet cleaning in upper can be done by mild soap or cleaner depending upon the material of Upper.

Before starting the finishing operation, ensure that the surface to be finished has been cleaned.

The purpose of cleaning the upper of the shoe is to remove,

- The dirt

- Adhesive overspill

- Other unwanted marks and contamination

And  prepare  the  shoe  surface  to  accept  dressing  and  other  finishing  treatments.

Cleaning  can  be  done  before  or  after  removing  the  last  from  the shoe.  The  latter  is  preferable  for  better  shape  retention  of  the  shoe.

Principles to follow when cleaning

- Relate  the  choice  of  cleaner  and  the  method  of  cleaning  to  the  types  of contamination  that  is  being  removed  and  the  types  of  finish  on  the  upper.

For  example- Solvent  based  cleaners  should  be  used  to  soften  natural  rubber  (  because  water  based  will  not ). However,  do  not  try  to  remove  rubber  solution  from  suede  with  a  solvent- based  cleaner  or  “  tide  marks”  will  result.  Suede  has  to  be  cleaned  dry  by  rubbing  with  crepe  rubber.

- Use dry methods in preference to wet.

- Use  the  mildest  treatment  possible  that  will  effectively  remove  the  contaminations.

- The  stronger  the  cleaner,  the  more  finish  it  will  remove  in  cleaner.

- Start  with  the  mildest  cleaner,  and  dry  progressively  stronger  cleaners  until  the  dirt  is  removed.

Cleaning methods

Cleaning can be done both wet and dry methods.  It can be done by hand or by machine.

- Cleaning dry by hand

The items used for this cleaning methods are-

- Crepe rubber- for removing adhesive.

- Soft sponge rubber- for brushing the suede nap.

- Brushes- wire and stiff bristle type. For removing general dust and dirt from suede. For brushing up the suede nap.

- Scouring  pads-  for  removing  general  dust  and  dirt  from  suede.

- Dull knives- for scraping away hard substances like waxes.  Sometimes  warming  the  knife  slightly  to  get  the  knife  under  to  lift  contaminants  from  the  surface.

- Emery  paper-   Various  fine  grades  of  grit  for  raising  and  even up  any  suede  nap  that  has  been  badly  treated  or  discolored.

Cleaning  dry  also  is  the  method  on  all  types  of  fabrics,  as  liquids  leave  “ tide  marks”.  The  best  solution  is  to  avoid  getting  the  shoe  dirty  in  the  first  place  as  heavy  staining  is  impossible  to  remove.

- Cleaning dry by machine

The  machine  can  have  power  driven,  revolving  brushes  of  various  shapes. The  method  requires  that  the  upper  finish  of  material  will  stand  up  to  friction  and  the  heat  generated  by  brush.

Apply  just  sufficient  pressure  to  remove  dirt  without  damaging  the  finish.  Usually, removing some stubborn contaminants by hand method is necessary.

- Cleaning wet  by  hand

Advantages  of  hand  cleaning  is  that  selective  cleaners  can  be  used  and  an  operative  can  give  to  each  shoes  individual  treatment .

- Apply  the  cleaner  by  cloth  ( white  cotton  pieces  are  best )  wrapped  round  the  fingers  by  rubbing  gently  over  the  surface.  Or  by  sponge  when  very  little  rubbing  pressure  is  required. Stubborn  dirt  may  need  extra  rubbing  or  application  of  a  more  suitable  cleaners. Crepe rubber and dull knives can be used.

- Change cloths frequently and wash out sponges regularly.

- Cleaning  wet  by machine

The  machine  has  an  application  for  cleaner  and  usually  a  mild  water  - based  type  is  chosen  for  general  cleaning.

- Adjust  the  amount  of  cleaner  applied  to  the  revolving  brush  to  “ wet-  out”  the  material  without  being  excessive.

- Take care not to over brush and damage the finish.

- Follow the following work method.

- Clean the feather area with the small welt cleaning brush.

- Clean the upper with bigger wet brush.

- Dry the upper on the cloth mop.

- Polish on the woolen brush.

Wetting out and drying out

A  cleaner  to  be  effective  must  “ wet-  out”  the  material  being  cleaned.  If  the  liquid  does  not  spread  out  on  the  surface,  but  runs  over  the  drops,  it  is  not  wetting  the  finish  and  should  not  be  used.

After  cleaning  upper  must  be  allowed  to  thoroughly  dry  out  before  other  operations  are  carried  out,  otherwise  there  were  be  a  lack  of  adhesion  with  the  next  coat  of  dressing.  Bloom may be develop later on the box.

Table 3. 1 cleaning for various soiling materials

| Cleaning materials | Cleaning agent |
| --- | --- |
| Color code Paint ( on material edge ) | Detergent |
| Thread lubricant | Detergent |
| Spray marking | Detergent |
| Silver pen | Solvent |
| Grease | Solvent |
| Thermoplastic adhesive | Solvent / Detergent |
| Rubber adhesive | Blunt knife |
| PU adhesive | Crepe rubber/ Adhesive |
| Neoprene adhesive | Rubber / solvent |

II Repairing

Repairing in shoe refers to bring the shoe in good slandered before continuing with other operation. It can be done by Repair crayons, Repair paste, brush or spray in upper material. The paste spray can be either water or sprit based

During  the  shoe  making  process  upper  material  is  subjected  to  a  variety  of  abrasive  and  stretching  actions,  which  can  adversely  affect  the  appearance  of  the  shoe.

The damage may be due to the following reasons:

- Over  roughing  along  the  featheredge

- Scuffing of the finish during handling.

- Damage to the finish by solvents and heat.

- Grain crack during lasting.

- Increase in shade variation of upper leather.

Repairing  includes  both  upper  and  bottom  repairs,  of  a  minor  nature,  such  as  scratched  uppers  at  upper  roughing,  heel  scouring  and  very  slight  cuts  during  upper  clicking,  lasting  etc.

For  bottoms,  main  repairs  area  may  be  in  the  heel,  especially  at  the  seat  and  heel  breast.

The  purpose  of  these  repairs  is  to  bring  the  shoe  to  a  good  standard,  before  continuing  with  the  other  operations  in  the  finishing.  It  can  be  done  by  repair  crayons,  repair  pastes,  and  spray  paints  for  grainy  upper  materials.  The  paste  or  spray  can  be  either  water  or  spirit  based.

- Never repair deep scratches, cuts etc.  on  the  upper  as  their  repair  will  only  leave  marks  which  would  be  seen  on  the  finished  shoe  and  therefore  should  not  be  accepted.

- For  minor  scratches,  cuts,  use  very  fine  grit  size  abrasives  to  first  even  the  surface,  then  apply  repair  crayons  or  pigment  paste  and  finish  off  with  renovators  etc.

- Pigment and dyes must match the color of the upper.

- If  only  the  finish  is  damaged,  then  use  uniformers  or  renovators.

- For  repair  of  soles  use-

- PU  paint  for  PU  soles

- TPR  paint  for  TPR  soles

- Creams and polishes + color of leather soles.

Figure 3. 6 repairing

- Wrinkle Chasing

Removing the wrinkle on the lasted upper by applying heat. This done by ironing the wrinkled surface or hot air blow. This technique is to apply heat and steam to the surface of the upper, or lining materials, to shrink the surface slightly and tightening the grain thereby removing the creases.

Figure 3. 7 Wrinkle Chasing

- Ironing

Ironing in leather upper required hand irons which are controlled by thermostats used for removing wrinkles. Sometimes the whole leather surface is ironed to improve the finished quality.

Teflon paper is used to protect leather surface from hot metal parts to avoid burning.

Figure 3. 8  teflon paper ironing

Special  electrically  heated  hand  irons  which  are  thermostatically  controlled  are  used  for  removing  wrinkles.  Sometimes  the  whole  leather  surface  is  ironed  to  improve  the  finished  quality.

Figure 3. 9 thermostatic ironing

- Filler or base coat application

Filler of base coat application is done to remove the “open and hungry look” of leather after the lasting process. The purpose is to seal the surface for subsequent dressing. It ensures that topcoat does not sink in and minimum quantity of dressing material used for the desired appearance and luster.

Fillers are available either in cream or liquid forms.  These products are used to fill upper  leathers,  which  have  developed  an  “ open  or  hungry”  look  after  lasting.  They act as a base coat for subsequent top dressings.  They  seal  the  surface  so  that  the  top  coat  does  not  sink  in  and  only  minimum  quantity  has  to  be  applied  to  give  the  desired  appearance  and  luster.  Fillers  also  enhance  the  gloss  and  are  necessary  for  inferior  quality  leather.

They are available as follows:

Table 3. 2 filler or base coat application

| Degree of “ fill” | Usage |
| --- | --- |
| Light | For light leather finishes ( protein finish ) |
| Medium | For resin finished leather |
| Heavy | For leather with open or hungry look |

They are  usually  available  in  black  for  black  leathers  and  colorless  for  other  leather  colors.  Occasionally  the  base  coats  can  be  color  matched  with  helps  to  lessen  shade  variations.  They  are  usually  applied  by  hand  using  sponges  and  they  should  dry  out  “steak  free”. Use  of  correct  fillers  will  also  help  in  cases  where  there  are  problems  of  adequate  adhesion  between  the  top  coat  and  the  shoe  upper.

Table 3. 3 fillers for leather finish

| Leather finish | Leather | Base/ filler |
| --- | --- | --- |
| Protein | Aniline calf, glazed kid | Light filler |
| Resin | Resin, semi-aniline Rub off, pearlized | any type when necessary |
| Pvc | Patent, coated split | None, only color match |
| Wax | Burnish, oil pull-up | Light filler if required |
| Suede, nubuck | Suede, nubuck | None |

Figure 3. 10 smooth finish filler

- Top coat application

The operation is often the final treatment, which gives the shoe its luster and determines its final appearance. Top dressing is either water based or solvent based.

- Mainly wax based: - after brushing they give natural shine

- PU based: -does not require brushing, gives artificial shine

- Protectors: -silicon oil /fluorocarbon

- Renovators: -oil or emulsion

The success of the dressing rests on-

- Correct and through cleaning.

- Compatibility of the dressing with the material finish.

- Condition of the upper before applying the dressing.

- Correct application.

- Correct spray gun adjustment and technique.

Types of dressing and application

- Water based dressing

Water based top sprays of older types were based on wax emulsion and protein.  The  modern  sprays  are  made  from  acrylic  resin  emulsions  and  suitable  for  use  on  protein  and  nitrocellulose  coated  leathers  but  not  for  polyurethane.  They are colorless or with colors.  Liquid dressing are of two types.  Wax based and PU based.

Wax based  dressing  requires  brushing,  which  gives  a  natural  shine  to  the  shoe.

PU based dressing normally does not require brushing.  It gives an artificial shine.

- Check  compatibility  of  the  dressing  with  upper  material  because  non-  compatibility  shows  as  cracks,  peel  off  or  white  marks.

- Apply  dressing  by  hand  or  spraying  according  to  finishing  system  selected.

- Solvent  based  dressing

Solvent  based  dressing  from  strong  coating,  which  are  compatible  with  a  wide  range  of  upper  materials.  They  are  fast  drying  and  are  based  on  cellulose  derivatives,  acrylics  or  polyurethane.  They  can  be  clear  or  colored  and  are  available  in  a  wide  range  of  luster  from  matt  to  high  gloss.

- Check compatibility of the dressing with the upper material.

- Apply dressing by hand or spraying.

- PU  based  dressing

It  is  not  repairable  if  something  goes  wrong  during  finishing  and  one  want  to  refinish  after  the removal  of  first  finish.

- Cutting

Upper smoothing with help of cotton brush (abrasive action) for core filling to get smooth waxed surface for further operation is called cutting. Mainly cotton brushes are used for doing cutting in shoe surface.

- Polishing

Polishing is done after the application of final coat on upper material. This process is with woolen brushing on which carnauba or polishing wax is applied. It gives luster and gloss to the upper of the shoe.

Figure 3. 11 polishing

Shoe  polish  is  a  waxy  paste  cream,  or  liquid  used  to  polish  to  provide  shine,  water proof  and  restore  the  appearance  of  leather  shoes,  boots  etc. And extending the footwear life.

Various  substances  have  been  used  as  shoes  finish  for  hundreds  of  years,  starting  with  natural  substances  such  as  wax  and  tallow.  Modern  polish  formulas  were  introduced  early  in  the  20th   century  and  many  of  those  original  formulations  are  still  in  use  today.  Shoe  polish  is  usually  made  from  a  mix  of  natural  and  synthetic  materials  including  naptha,  turpentine,  dyes  and  gum  arabic,  using  a  chemical  processes.  Shoe  polish  is  usually  flammable,  can  be  toxic  and  if  misused  can  stain  skins.  It  should  be  used  in  a  well-ventilated  area  with  care  taken  to  protect  cloths,  carpet  and  furniture.

Polishes and creams are non- film forming.  Normally  wax  polishes  are  used  and  are  widely  available  either  neutral  or  pigmented  to  match  the  color.  Surface  waxes  can  impart  a  measure  of  water  repellency  to  footwear  although  they  cannot  be  regarded  as  water  proofing  agents.    Waxes  and  creams  are  also  suitable  for  gloss  enhancement  or  to  give  some  special  appearance  ( antique  effect )  on  leather  look  synthetic  material.  Cream could be soft, medium or hard.  Soft  and  medium  creams  are  those  that  require  to  be  polished,  and  are  referred  to  as  the  creams  to  be  polished.  Hard creams are available as self- shining creams.  They do not require polishing or brushing operation.

Examples of Finishing Procedures for Full Grain Calf Leather -Aniline, Semi aniline

The characteristic of this leather is the color transparency and brightness. For finishing this type of leather some of the systems are:

- Clean the Shoe

- Apply the sealing cream;

- Once dry put a small quantity of abrasive wax on a cotton brush and then brush it at a medium speed; and

- Polish it with a woolen brush treated with carnauba wax.

- Clean the Shoe

- Apply the sealing cream;

- Once dry put a small quantity of abrasive wax on a cotton brush and then brush it at a medium speed;

- Apply the top dressing; and

- Polish it with a woolen brush treated with carnauba wax, once the shoe is dry.

- Clean the Shoe

- Apply the sealing cream;

- Once dry put a small quantity of abrasive wax on a cotton brush and then brush it at a medium speed;

- Apply a self-shining cream by sponge; and

- Once dry, brush it with a cotton brush and then polish it with a woolen brush treated with carnauba wax.

- Brush-off Leather

The characteristic of this leather is that it gives a two-tone effect on brushing on a cotton brush.

The suggested systems are:

For Natural Gloss Elegant Look

- Clean the shoe;

- Once dry, brush on a cotton brush at 700-800 RPM with abrasive wax;

- Clean the shoe;

- Apply a suitable cream;

- Once dry, brush on a cotton brush at 700-800 RPM with abrasive wax; and

- Polish it with a woollen brush treated with carnauba wax 700-800 RPM.

For High Gloss

- Clean the shoe;

- Once dry, brush on a cotton brush at 700-800 RPM with abrasive wax;

- Clean the shoe;

- Apply a filler (sealing cream/liquid);

- Once dry, apply a suitable top coat; and

- Polish it with a woolen brush treated with carnauba wax.

For High Gloss

- Clean the shoe;

- Once dry, brush on a cotton brush at 700-800 RPM with abrasive wax;

- Clean the shoe;

- Apply a high shine top dressing, one or two coat as per the requirement; and

- Polish it with a woolen brush treated with carnauba wax.

## Packaging and labeling materials

- Packaging

There are a lot of definitions for packaging, but a broad and well-established one can be: packaging is “a coordinated system of preparing goods for transport, distribution, storage, retailing, and their end-use; the means of ensuring the safe delivery to the final consumer in a sound condition at minimum cost; a techno-economic function aimed at minimizing the costs of delivery while maximizing sales (and hence profit)”.

Packaging material

Packaging can be made from diﬀerent kinds of material. Most important are: paper and cardboard, plastic, textile, wood, metal, glass, and composite materials. In shoemaking production, paper and cardboard are the most often used packaging materials. Less important are plastics and/or metal cans, glass bottles for liquids, and chemicals and wood for wooden pallets for the transportation of goods.

Paper and cardboard
Paper and cardboard are one of the oldest materials which still cover the largest proportion of packaging. They are the most widely used materials in the footwear industry as well. Mostly, they are used as:

- shoe stufng paper or fller paper to preserve the shape of
the shoe

- wrapping paper to protect the footwear in a pair box

- pair box to protect the fnished pair of shoes before delivery to the customer (Figure 3.12 above)

- transport packaging for transporting the fnished products (shoes) from the place of production to the place of sale (Figure 3.12 below)

Figure 3. 12 Example of a pair box and transport box made from corrugated paper

Packaging made from corrugated paper

A corrugated box is a shipping container made of corrugated fiberboard. These are most commonly used to transport and warehouse products during distribution, and are rated according to the strength of the material or the capacity of the finished box. This kind of packaging is probably the most used type of packaging in the footwear industry.

- Labeling

Labelling is a process of inserting identification on boxes or the packaging that’s done. Labelling is the process of writing important information on the packing boxes for identification and forward information regarding the care, moving, types of item, number and other related issues to the customers.

Figure 3. 13 labling

Information Provided in Carton Boxes: -

- Some of the information’s to be provided are:-

- Carton box number

- Order number

- Style and color of the product

- Number of pieces in each color and style

- Total number of pieces

- Contact number

- Net weight

- Dimension of the carton box

The label must be attached to at least one item of footwear in each pair and may be affixed by way of printing, sticking, embossing or use of an attached label; it must be visible, securely attached and accessible. The label may also appear on the packaging but it must be on the footwear itself. If pictograms are used in a retail shop a notice must be displayed that explains to consumers what the symbols mean. The notice must be large enough so that the information can be seen and understood by consumers. If pictogram labels are used where footwear is sold from a place consumer do not have access to (for example, mail order or internet sales) the consumer must be clearly informed of the meaning of the pictograms used.

The pictograms for the parts of the footwear

The pictograms for the materials used in footwear composition

Figure 3. 14 parts of footwear & material used in footwear composition

## Checking products

The shoe must be submitted for a final examination before boxing. Any damages to the upper must be repaired if possible, or the shoes rejected or returned to be pulled up and put in order. If a damage is of such a nature that a part of the upper is shown to be peeling up, this should be firmly stuck down with a glue, such as Croid or something similar that is contained in a tube ready for use. Rubber solution must not be used as it leaves a slight ridge at the edge of the tear, which when rubbed may cause the damage to open up again. After the glue or cement has thoroughly dried, the area around the defect should be finely scoured and the surface filled in with a repairing crayon of an identical color to the original leather. Scratches or cuts that are not too deep can also be finely scoured and filled in in the same way taking care, of course, to brush away any surplus scouring before using the crayon. Damages in the feather caused by the stitching machine, usually because the leather guard has worn and allowed the awl to contact the upper, are difficult to repair, but are best dealt with by melting some crayon on the end of a hot knife and filling in the incisions. The surplus wax is rubbed off with a cloth wrapped around the end of the finger. Sometimes a series of grinning holes appears round the toe due to bad welt sewing. These can also be filled in in the same way. Damages are not the only things to be looked for; sometimes an eyelet is missing, and even a row of stitching may have been missed. Heel breasting may be the cause of many returns, if the knife has been allowed to go too deep, particularly in rubber soles. This, therefore, requires constant vigilance.

It is also not only the outside of the shoe that must be examined but also
the inside. There must be a final check to see that no nails are left sticking
through the insole. Torn linings, burst seams, damaged insoles due to the
welt sewer striking through, are other points that must be checked.

## Packaging operations

Packing is the  science,  art  and  technology  of  enclosing  products  for  distribution,  storage,  sales  and  different  methods  of  uses.  Packaging  also  refers  to  the  process  of  design,  evaluation  and  production  of  packages.

Lacing

Shoelaces, also called shoestrings (US English) or bootlaces (UK English), are a system commonly used to secure shoes, boots, and other footwear. They typically consist of a pair of strings or cords, one for each shoe, finished off at both ends with stiff sections, known as aglets. Each shoelace typically passes through a series of holes, eyelets, loops or hooks on either side of the shoe. Loosening the lacing allows the shoe to open wide enough for the foot to be inserted or removed. Tightening the lacing and tying off the ends secures the foot firmly within the shoe. The laces can be tied in different shapes, most commonly a simple bow.

Common lacing methods

- Straight-bar lacing

Straight-bar lacing appears horizontal and parallel when viewed from the exterior. Formal shoes usually demand straight-bar lacing to preserve their clean, neat look. This is especially true for dress shoes using a closed lacing system such as Oxfords, because the central shoelace crossovers of crisscross lacing prevent the sides of the shoe from coming together in the middle

Figure 3. 15 An Oxford shoe with straight lacing

- Crisscross lacing

The most common lacing method, termed crisscross lacing, is also one of the strongest and most efficient. However, they are reserved for more casual footwear, such as sneakers and boots. Derbies can be straight-bar laced or crisscross laced

Figure 3. 16 Chuck Taylor All-Stars with crisscross lacing

- Other lacing methods

Many shoe lacing methods have been developed with specific functional benefits, such as being faster or easier to tighten or loosen, binding more tightly, being more comfortable, using up more lace or less lace, adjusting fit, preventing slippage, and suiting specific types of shoes. One such method, patented in 2003 as "Double helix shoe lacing process", runs in a double helix pattern and results in less friction and faster and easier tightening and loosening. Another method, called "Rinlers Instant Lace Up", use additional accessories for instant tightening and loosening.

Many other lacing methods have been developed purely for appearance, often at the expense of functionality. One of the most popular decorative methods, checkerboard lacing, is very difficult to tighten or loosen without destroying the pattern. Shoes with checkerboard lacing are generally treated as "slip-ons".

Stuffing

Acid-free paper is absorbent and maintains a shoe's proper moisture level while protecting the shoes from deteriorating. When using paper to stuff shoes for moderate or long-term storage, opt for acid-free tissue instead of newsprint.

Figure 3. 17 Shoe stuffing and wrapping paper

Attaching labels

When you describe the materials and indicate the parts of the footwear, you should use the pictograms or a written indication. Your labelling must describe the materials of the three main parts of the footwear:

- Upper

- Lining and insole

- Outer sole

In each case, the label should state whether the material is made of:

- Leather

- Coated leather

- Textile

- Other materials

If no single material accounts for at least 80% of the product, the label should indicate the 2 main materials used.

Here’s an example of a label that was attached to the sole of the shoe by sticker:

How do you attach the footwear label?

You must attach the labelling to at least one of the pair of shoes. You can do this by printing, sticking, embossing or tying the label to the shoe. The labelling must be visible, securely attached and accessible, and the dimensions of the pictograms must be sufficiently large to make it easy to understand.

Bundling

Bundling is offering several products or services for sale as one combined product or service package. It is a common feature in many imperfectly competitive product and service markets

Most firms are multi-product or multi-service companies faced with the decision whether to sell products or services separately at individual prices or whether combinations of products should be marketed in the form of "bundles" for which a "bundle price" is asked. In a bundle pricing, companies sell a package or set of goods or services for a lower price than they would charge if the customer bought all of them separately. Pursuing a bundle pricing strategy allows a business to increase its profit by using a discount to induce customers to buy more than they otherwise would have.

Boxing

Everyone needs a shoe shipping box. They are the most important thing in the delivery of your shoes and will help you keep them in good condition until they get to their destination. You can spend less money by buying one and more time and effort by ordering it directly from the company (and in many cases, it is delivered free).

You can either tape up your shoebox and wrap with paper, or place it into a larger box for added protection. Use tissue paper, bubble wrap or polystyrene to fill the space in the box. Some shoes, such as soft ballet pumps and flip-flops, will fit into a large jiffy bag and are unlikely to be damaged.

Figure 3. 18 Boxing

Packing

You may be tempted to toss all of your shoes into the bottom of a box when packing for an upcoming move. This not only puts them at risk for physical damage, but also makes it that much more difficult to unpack when you arrive at your destination. If you have shoe boxes, pack the shoes in the boxes, otherwise some packing paper will protect the shoes from scuffing and other damage. It is a good idea to let shoes air out for a couple of days after wearing before packing them.

For shoes in shoe boxes, wrap each shoe in a half piece of packing paper to protect from rubbing or scuffing. Line a box with crumpled paper. Put boxes and shoes in the box. Shoes without boxes can be wrapped as a pair in a single piece of packing paper. Fill in gaps with crumpled paper. Close the box and secure with packing tape. Label the box.

Things to note when packing shoes

- Footwear May Contain Moisture There is a chance that your shoes could mold during moving and storage if they aren’t properly prepped and packed. Be sure to air out leather athletic and work shoes prior to packing so that they are free of moisture when you place them in the box. As an extra measure of security, you may want to powder the insides to ensure all moisture is drawn out of the leather and fabric.

- Protect Your Dress Shoes’ Finish it is recommended that you wrap shoes in tissue paper and place them in the original shoeboxes to prevent damage from contact with other shoes. This will protect patent finishes and ornaments from accidental damage during transit. If you’d like to avoid wrapping and boxing shoes in pairs, you can also place them in a closet door shoe hanger.

- Folded carefully, this innovative storage solution can keep your shoes organized and firmly in place during your move.

Figure 3. 19 Packing

Dispatching

A basic procedure in the expedition of products is the preparation of the Packing List. Packing List is a document which identifies the contents of the package. It should be either inside of the packaging, or in an attached shipping pouch. Some companies use packing list as a bill or invoice since it often indicates charges to be paid.

Packing List is included with a shipment to inform transport agencies, government authorities, and customers, about the contents of a package. Some companies simply include a copy of the invoice in place of a packing list, however this may cause confusion, and does not provide all the necessary details about the content of a package. The preparation of the packing list is usually done at a computer level and follows the guidelines issued by the customer when sending the order or orders. Unlike an invoice, typical packing list does not show financial information. It also has some fields a typical invoice may not have, for example:

- Number of boxes

- Packing Date

- Packed by

- Checked by

- Unit weight

- Unit volume

The product is placed in boxes, usually paperboard, following the customer's instructions. They can be closed manually or with the support of specific equipment such as the stripping machine.

Figure 3. 20 Dispatching

## Prevent accidents and to eliminate risks to personal safety

The finishing department is more related to use of solvents and chemicals at the working floor.  Hence it should cover four main points:

- Health risks resulting from exposure to solvent hazards at work

- Solvent fire and explosion hazards

- Principle of prevention

- Emergency procedures employed

3.7.1 Health risks

The effects of the solvents can be acute or chronic depending upon concentration and length of exposure to the body.  Main effects are irritation and allergic reaction to skin, eyes and respiratory tract.

3.7.2 Fire hazards

Solvents in the work place are major fire and explosive hazards. The improper storage, handling or transport may result in minor fire to major disaster resulting in loss of human life.

Fire in principle needs three elements to survive:

- Fuel

- Heat

- Oxygen

To check any fire, hazard these elements should be controlled. In this respect we can classify the chemicals used in finishing department in two categories:

- Water based products:

There is no risk of flammability during drying or application but some of the residues may ignite if exposed to a naked flame.  Soil deposits must be disposed of correctly.

- Solvent based products

Most of them contain low flash point solvents and they will ignite instantly if come in the contact of naked flame or hot surface.  Safety precautions must be respected.

3.7.3 Principals of prevention

It should base on organizational as well as operational control.

Four basic principle of operational control are:

- Eliminate the hazard (replace as for as possible, solvent based chemicals to that of water based).

- Put a distance / shield between the solvent and worker.  The items being sprayed must kept within the work area of spray booth and there should not be over spraying.  The inhalation of fumes is dangerous and can be avoided by using a proper spray booth, spraying technique and wearing a mouth cover.

- Provide general and local ventilation e.g.  Spray booth with exhaust fans and running water to reduce concentration of airborne contaminants such as fumes, gases, vapors and mists.

- Protect the worker by using personal protective equipment like dust mask respirators, protective goggles, gloves etc.  Water based and spirit products, mainly the spirits, degrease the skin if kept in contact for a prolonged period of time.  Use of after work creams are suggested.

- Spillage, disposal, cleaning.  Major spillage of solvent products should be immediately soaked by dust or sand to prevent spreading and in particular to avoid contaminating drains.

## Completing production and other records

After finalizing the packaging processes all relevant paperwork and documentation is prepared in accordance with enterprise procedures.

A packing list is a very important document for the leather garment and goods industry. Generally, merchandisers or packing in-charge is responsible to make and delivering packing lists to related departments. The packing section does pack as per customer packing requirements that solid or asserts/ratio pack. Packing is done when a PO packing is completed, without giving packing list buyer QC never does a final quality inspection, packing list confirm buyer QC that a PO’s packing is closed. The packing list is very important to export documents, without the packing list commercial department cannot submit exports document for the products shipment, that’s why the packing list consider as important documents.

Documents to be prepared include: -

- product maker, Specific style and buyer name

- Total and size-wise order quantity

- Style description and color

- Size-wise Cut vs. Order vs. pack quantity, carton quantity

- Carton wise case number

- Size-wise carton net and gross weight

- Carton measurements and Total carton

- Total over or short pack quantity

## Self chek-3

Directions:  Answer all the questions listed below. Examples may be necessary to aid some explanations/answers.

Test I: Multiple Choice

1. In which type of leather finish the surface is coated with a heavy protective topcoat which includes pigmentation.

- Aniline,

- corrected grain or pigmented,

- semi aniline ,

- none

2 -----------------is the process of buffing leather edges to a slick, glossy finish.

- Brush off ,

- Burnishing,

- A and B ,

- none

3 -------------------is a leather which if brushed by heavy cotton brush gives a two tone effect

- Brush off,

- burnishing

- A and B ,

- none

4 which one of the following are not essential characteristics of full grain quality leather

- Fat wrinkles or growth lines: subtly shaded bands on the grain side of a hide,

- Natural variations in grain texture and shading. ,

- Small scars: on a quality product they do not render large portions of the hide unusable,

- none

5 which one of the following are the purpose of cleaning upper of the shoe

- Remove dirt,

- remove adhesive overspill,

- remove unwanted marks and contamination,

- All

6 -------is a process of inserting identification on boxes or the packaging that’s done

- Labeling,

- lacing,

- packaging ,

- none

7-----------is a process of inserting acid free paper which absorbs and maintains a shoe's proper moisture level while protecting the shoes from deteriorating.

- Stuffing,

- lacing,

- label attaching,

- bundling

Test IV: Short Answer Questions:

Instruction: write short answer for the given question.

- List and describe types of leather finish

- List the sequence for system I shoe finishing process?

- List the basic Footwear finishing operations?

- What is labeling?

- What is packing?

- List and describe purpose of packing?

## Operation sheet 3.1: Finishing and packaging

- Operation title: performing finishing and packaging operation

- Purpose: To perform finishing and packaging operation

- Instruction: Using the provided steps and given equipment’s perform finishing and packaging operation.

- Tools and equipment:

- Insole

- Polishing machine

- Spray gun

- lace

- box

- sulphite paper

- Steps in doing the task

Stepe-1: Cut excess lines and put finishing insole

Step-2: Polishing manually

Step-3: Polishing through the spraying tool

Step-4: Applying laces

Step-5: Confirmation sample

Step-6: Finishing final control - Control of each foot individually

Step-7: Control of two feet at the same time

Step-8: Preparation of the box to pack the footwear by placing sulphite paper in the base and Placing the foot in the box

Step-9: Wrap and close the pair with the sulphite paper

Step-10: Put identification label on the box

Step-18: Place the box with product in the proper location (structure) to follow to finished product warehouse

Precaution: Be careful on faults caused by finishing operations.

Quality requirement: good finished shoe with the perfect packaging

| # LAP Test | Performance test |
| --- | --- |

Name:  _____________________________   Date:  ________________

Time started: ________________________ 	Time finished:  ________________

Instruction I:	Given necessary templates, tools and materials you are required to perform the following task. The project is expected from each student to do it.

Task 1: perform finishing and packaging operation.

# Unit Four: Checking final quality of the finished pair and dispatch

This learning unit is developed to provide the trainees the necessary information regarding the following content coverage and topics: - Identifying quality check points - Checking footwear against final quality inspection standards. - Identifying, rectifying and returning faults to appropriate section for repair. - Completing and dispatching quality records and documentation This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify quality check points - Check footwear against final quality inspection standards. - Identify, rectify and return faults to appropriate section for repair. - Complete and dispatch quality records and documentation

## 4.1. Identifying quality check points

4.1.1 Quality

Quality  is  the  totality  of  features  and  characteristics  of  a  product  or  a  service  that  affect  its  ability  to  satisfy  the  stated  or  implied  needs  of  a  customer and it is  not  absolute It  is  relatively  to  other  goods  and  services. Quality cannot be achieved by inspection only.  It  can  only  be  improved  by  small  continuous  product  and  process  improvements. Quality  consistency  requires  us  to  concentrate  on  the  process  rather  than  on  the  product  alone. In  a  good  organization  each  error  is  part  of  training  and  learning.  The  key  to  success  is  to  ensure  that  a  mistake  is  not  repeated  again  in  the  organization. Company must recognize that quality is the key to survival in business.

4.1.2 Quality check points

The shoe sample after finishing must be checked for any damage in finish as it may give rise to customer’s complaints. Solvent based dressing may be reasonably durable but wax based dressing may be lost early in wear.

Shoe after finishing must be checked for:

- Uniform gloss and colour;

- Proper colour fastness;

- No damage to the surface;

- No damage to the sole (especially rubber soles). One should be careful while brushing the shoe on cotton rollers, especially if it is a rubber sole. In case the sole comes in contact with the brush, it may be damaged and the dirt could migrate to the leather surface;

- Improper drying may lead to sticking of fibers on leather surface, which are difficult to remove. One may have to go back to the cleaning operation all over again;

- One should not press the shoe hard against the cotton roller. It may lead to excess cooking of wax on the surface;

- ‘Whiteness’ occurs either due to leather spew or improper finishing method.

- For example, excess loading of self-shining cream, especially natural coloured cream on coloured leather. It may also be due to an excess of neutral-coloured hard wax on the roller;

- Cracking of shoe finish;

- Tackiness; and

- Colour variations on different parts etc.

4.1.3 Keys to a good quality system

- Everyone must know what to check and how to check.

- List major and minor defects and display it.

- Check all inputs materials for quality / quantity.

- Make  specifications  clear  to  the  supplier  at  the  time  of  ordering  materials.

- Do not dilute your standards.

- When a mistake occurs look for corrective and preventive action. Corrective action is immediate action taken to solve problem. Preventive  action-  Step  taken  to  prevent  a  recurrence  of  the  problem.

- Keep a defect file a learning system.

- Clearly fix responsibility for quality in each department.

- Quality cannot be controlled at the end of line. It must be built into the system.

- Continuous improvement in small steps is the key to success.

- Have a good monitoring system.

- Good housekeeping is important for good quality.

4.1.4 How Can a Supervisor Control quality on his line?

In  most  factories  supervisors  spend  their  time  pushing  production  and  do  not  have  time  to  check  quality.  Many  supervisors  are  not  aware  of  what  they  should  check  or  how  they  should  check.  Lastly the right tools like gauges, scales etc.  must  be  provided  to  the  person  to  enable  him  do  the  job  well.

- It  is  important  for  a  supervisor  to  drawn  up  a  time  table  of  how  to  spend  the  day.

- In  the  time  table  at  least  2 – 3 hours  should  be  allocated  for  monitoring  quality.  This could be over four half hour sessions.

- During  the  first  session  he  could  look  at  3  operators  in  his  line  and  study  their  operations  in  depth.  For  example-  if  a  sewing  supervisor  is  checking  a  particular  operation  the  following  steps  are  essential.

- Check cleanliness of machine and surroundings.

- Stop  machine  and  check  needle  and  threads  used  to  see  if  this  is  as  per  specifications.

- Check if assembly instructions are available to operator.

- Pick  a  tray  of  10  pairs  and  check  100%  for  this  operations.  See % of defects.  If  it  is  more  than  5%  stop  the  operations  and  find  out  reason  for  faults.

- Demonstrate to operator how to correct fault.

- Ask  operator  to  do  one  pair  correctly  based  on  your  instructions.  If  this  is  okay  allow  him  to  proceed  for  10  prs  &  recheck  tray.

- Discuss quality problems with operators and invite his suggestions.

- If  a  guide  is  to  be  used  make  sure  it  is  used.  Guide improve quality and productivity.

- Repeat such quality checks with other operators also.  You  should  be  able  to  cover  all  operators  in  your  line  at  least  once  a  day.  This  will  prevent  any  major  errors  since  you  have  checked  all  operations  in  detail.

- Check  the  first  five  pairs  of  shoes  coming  out  of  production  daily  to  see  if  there  are  major  problems  to  be  attended  immediately.

- Spend  half  an  hour  a  day  listening  carefully  to  workers  on  how  to  improve  quality.  They can offer excellent solutions.

- When  you  spot  a  problem  take  action  to  correct  it  and  stop  production,  many  supervisors  simply  allow  the  problem  to  multiply  further.  For  example-  if  there  is  a  machine  fault  and  stitching  is  incorrect  stop  the  machine  and  rectify  the  fault.

- Always look for corrective and preventive action in each case.  If  the  same  mistake  is  requiring  for  3- 4  days  the  supervisor  is  not  doing  his  job.

- The  biggest   problems  with  supervisors  are  that  they  see  a  mistake  but  allow  it  to  continue  on  the  ground  that  they  can  do  very  little  in  this  case,  if  this  happens  more  than  once  suck  the  supervisor  immediately.

## 4.2. Checking footwear

Inspection of footwear totally depends upon the quality control in different stages.  This system is based on the following principles:

- Well defined specifications.

- Checking and testing all input materials.

- In- process quality control.

- Final quality check.

- Monitoring quality.

The main objective is to achieve a consistent quality standard for the product.

- Inconsistent Quality: Some consignment is above the required standard and some below the standard.  The variation is too large and not acceptable to customers.

- Lack of Clear Understanding of Quality: Each staff member of a company has a different view on quality and there is no clear company standard for materials or clear identification of major and minor faults.

- Tendency to Save Money on Testing: Testing is the fix parameter of the company.  If the company doing not testing, then it will cause a great loss of the company.

- Insufficient planning to ensure quality.

- Lack of Top management   commitment to quality.

- Tendency to monitor quality at the end of the line instead of in- process quality control.

- Poor documentation.

- Tendency to take only corrective action but not preventive action.

- Lack of understanding on how to achieve quality.

- In most factory good quality is only a matter of chance.

4.2.1 Inspection of footwear

Quality Assessment

It  is  a  normal  in  the  footwear  industry  to  inspect  pair  by  pair  at  the  end  of  the  process.  The  following  guideline  will  help  to  ensure  that  the  inspection  is  done  correctly:

- Do  not  inspect  unless  there  is  an  approved  sample  signed  by  the  customer.

- A  Technical  Specification  sheet  for  each  style  approved  by  the  Production  manager/  customer.

- Sufficient light should be available on a clean table.  Lighting levels should be 1300 lux.

- A  list  approved  by  the  production  manager  showing  major  and  minor  defects  is  allowed  &  2- 3%  minor  defects  are  allowed.  Without  a  list  of  major  and  minor  defects  no  inspections  is  possible.  In  case  a  major  defects  occurs  not  listed  in  the  defect  list,  the  quality  controller  should  not  be  blamed.  This  new  defects  should  be  added  to  the  list  for  future  lots.

- Keep  a  tape,  scale  12”  and  6”,  crepe  rubber  piece  and  stitch  gauge  with  you  always.

- Inspect  plan  wise  and  keep  track  of  size  arise  pairs  passed  as  shown  in  the  chart  below-

Plan  no.          Model.          Color.         Pairs

Size     5      6        7         8          9           10

Order   60    95      70       198       40         32

Pairs                                                          495

This  chart  immediately  gives  the  status  of  inspection  size  wise  and ensures  that  wrong  sizes  are  not  sent.

- Major  defects  should  be  destroyed  or  punched  , so  that  they  are  not  recycled  or  mixed  with  good  shoes.  Keep  the  defective  shoes  with  you  till  you  get  replacement.

- Keep  track  of  defects  as  shown  in  the  quality  inspection  report.  This report gives number of pairs defect wise.  Look  for  the  top  three  defects  and  take  steps  to  correct  them  immediately.  Stoppage of production could also be one of such measures.  If  the  same  defect  reoccurs  for  three  days  the  quality  control  has  failed  and  the  quality  control  inspector  is  responsible. One  should  remember  that  recording  of  defects  is  not  the  main  job.  Corrective  and  preventive  action  is  the  main  job  of  the  quality  control  inspector.

- Take  a  10  minute  break  after  every  two  hours  otherwise  inspection  fatigue  occurs.  During  these  periods  go  to  the  line  and  look  at  ways  to  correct  quality  problems.

- A  shoe  must  be  inspected  by  looking  at  it  with  the  toes  facing  and  with  the  back  strap  flack  seam  facing  you.  Shoes must inspected pair wise always.

- Have  a  10  minutes  discussion  daily  with  production  supervisors,  about  major  problems  and  how  to  correct  these.  If  these  are  not  attended,  to  stop  inspection  and  report  to  the  production  manager.

- Quality  inspectors  suddenly  change  standards  of  inspections  when  a  complaint  is  received.  Many times he rejects up to 70 – 80%.  This  is  simply  ridiculous  because  with  such  a  high  rate  of  rejection  it  means  that  the  quality  control  system  has  collapse  and  the  quality  control  inspector  is  responsible.

- The  following  chart  must  be  clearly  displayed  at  the  quality  control  table-

- Back  height  chart

- Major  and  minor  defects  list

- Key  points  raised  by  the  customer  if  any

- Approved sample and defect file.

- Keep a defect file for each customer.  Whenever  a  complaint  is  received  keep  a  copy  in  this  file  along  with  corrective  and  preventive  action  taken.  The  purpose  of  this  file  is  to  catalogue  defects  and  ensure  that  they  do  not  recur.

- Final inspection by self is not enough.  This  must  be  supported  by  laboratory  test  reports  on  materials  and  the  final  shoes.  Fit and wear trials immensely in quality control.

- Remember that final inspection is only a post- mortem.  It  is  much  better  to  spend  time  on  in  process  quality  control.  You  follow  the  following  rule  always  “  DO  IT  RIGHT  THE  FIRST  TIME”.

## 4.3. Identify rectify and return faults

4.3.1 Finishing faults

- Brushing

Generally  this  operation  carried  out  in  shoe  room  with  the  help  of  brushing  machine,  with  provision  for  cotton  and  wool  brushing,  for  cutting  and  polishing  purpose.

Rollers are also called the Brushes of the Brushing machines.  The  machines  rotates at  variable  speeds,  polishing  is  normally  recommended  at  low  speeds  with  the  use  of  polishing  wax.  Cotton  brush  can  be  applied  at  variable  speed  ,  high  speed  for  leather,  which  needs  more  abrasive  action,  e.g.  Burnishing  leather,  Brush- off  leather  and  corrected  grain  leather  with  heavy  pigmentation.

In  polishing  normally  use  rollers  of  brushing  machine  at  low  speed  with  the  use  of  polishing  i.e.  Synthetic wax or carnauba wax.

Whereas  when  high  abrasive  action  is  not  required  then  roller  of  brushing  machine  at  medium  speed  with  the  use  of  abrasive  wax.

If rules do not follow the faults will occurs.

These are the main reasons for improper brushing-

- Dressing and materials finish is not compatible or inadequate cleaning.

- Due to the dirty sponges and contaminated cleaner pots.

- Due  to  high  humidity,  water  in  solvent,  or  faulty  air  supply.

- Due to excessive heat supply in process.

- Due to over spraying with butyl   acetate.

- Leather with low absorbency.

- Low  adhesion  of  finish  caused  by  silicon,  oil ,  greases  etc.  On the surface of the leather.

- Spew  in  the  process  is  the  fault  of  tannery,  degrease  and  then  apply  the  base  coat.

- Cracking  of  the  grain  during  lasting  normally  caused  by  insufficient  elastic  rectangle.    Apply  a  softener  on  the  grain / flesh  side  few  minutes  before  lasting.

- Whiteness  appearance  due  to  heavy  filler  used  without  its  absorption  capacity.  It deposit on the leather surfaces, then refinish the above shoes.

- Streaks on upper

In  shoe  room,  during  finishing  of  upper  faults  like  streaks  on  upper  occurs.  Normally  this  fault  arises  during  liquid  dressing  due  to  uneven  spray  pattern.  Distribution of finishing during cleaning and filling i.e.  If  cleaning  and  filling  does  not  done  properly  as  per  quality  and  standard  norms. This fault also arises due to poor quality of spraying.

Figure 4. 1 strike on upper

When  leather  is  new,  it  has  a  rich,  smooth  appearance,  overtime,  the  leather  will  begin  to  fade  in  spots  where  it  is  repeatedly  warn.  The  color  will  appear  uneven,  with  light  streaks  appearing  throughout  the  leather.  This  happens  to  leather  furniture,  such  as  on  the  seats  or  leather  jackets  at  the  crease  of  the  arms.  Fortunately,  a  warn  leather  finish  can  be  restored  to  an  almost  new  appearance  with  the  right  materials.

- Instructions

- Apply  a  leather  cleaners  to  a  clean  rag,  wipe  the  entire  surface  of  the  leather  item  to  be  cleaned.  This  not  only  removes  the  surface  dirt  ,  but  it  opens  the  pores  in  the  material ,  allowing  the  re-coloring    balm  to  work  more  effectively.

- Select a re- coloring balm in the appropriate shade.  The  shade  usually  come  in  a  few  basic  color  and  can  fit  a  range  to  shades  in  that  color  family.

- Apply the balm to a clean cloth.  Beginning  at  the  top  of  the  item  to  be  restored,  rub  the  cloth  in  a  circular  pattern.  This will work the color into the leather.  As  the  color  seeks  into  the  pores,  the  streaks  will  disappear  and  the  color  will  be  restored.  Add additional amounts of balm to the cloth, as necessary.

- Buff  the  leather  surface  with  a  clean  cloth  once  the  entire  item  has  been  covered  with  the  balm.  Continue  buffing  until  there  is  no  longer  a  color  transfer  between  the  item  and  the  cloth.  The buffing will create a smooth, even appearance.

- Apply  a  leather  cream  to  protect  your  leather  item  against  further  wear  and  fading.

- Stain  marks

During  finishing  operation,  like  incorrect  dressing ,  uneven  cleaning,  contaminated  dressing  as  well  as  atmospheric  contaminations. Due  to  above  reasons  generally  fault  like  stain  found  on  the  upper  surface  of  the  finished  leather,  because  of  tackiness  and  not  proper  penetration  of  finishes  as  well  as  uneven  dryness  of  the  finished  film.

This  problems  also  arises  due  to  poor  spray  pattern,  due  to  shaking  of  liquid  or  season  not  done  before  apply  it  on  the  surface.

Figure 4. 2 stain marks

- Stain on  upper  finish

- Determine the  types  of  leather:

- Put  a  drop  of  water  on  the  surface  of  the  leather,  if  the  water  soaks  in  the  leather  is  unfinished  and  will  requires  more  tanning  chemicals.  If  it  breaks  up,  the  leather  is  finished  and  you  can  begin  cleaning.

- Take  unfinished  leather  ( like  suede )  directly  to  a  dry  cleaners  for  stain  removal.-  Unfinished  leather  is  extremely  absorbent  and  the  stain  will  be  very  difficult  to  remove.

- Determine  the  depth  of  the  stain

- If  the  stain  is  fresh  and  only  on  the  surface  of  the  leather  item,  begin  cleaning  according  to  the  steps.

- If  the   stain  is  older  or  has  soaked  into  the  leather,  you  will  likely  have  to  re dye  the  leather  professionally  to  remove  the  stain.

- Cleaning  your  leather

- Read  carefully  the  instruction  of  your  leather,  if  possible  because  the  manufacturer  may  recommend  a  certain  conditioner  or  cleaner  to  remove  stain.

- Wipe  the  leather  gently  with  a  cloth  dampened  with  hot  soapy  water  or  use  a  leather  shampoo  to  clean  the  affected  area.  Treat  your  leather  on  a  regular  basis  with  quality  protection  products.

- Uneven  shine

This  fault  arises  due  to  uneven  dressing,  cutting  as  well  as  uneven  polishing.  Apart  from  this  using  un appropriate  wax  as  per  the  finish  on  leather  surface  also  results  as  uneven  shine  on  the  upper

Figure 4. 3 uneven shine

Besides  this  different  absorptions  of  dye  or  finish  on  different  parts  of  skin  and  hide  also  cause  uneven  shine.  In  correct  adjustment  of  the  spray  gun  applying  the  final  coat  or  top  coat  also  results  as  uneven  shine.  Then  there  is  a  possibility  of  low  glass  and  shine  as  well  as  full  appearance  of  shoe  upper  and  finally  we  cannot  get  the  required  quality  in  finished  shoe.

One  of  the  reason  that  leather  is  so  popular  that  in  addition  to  its  great  natural  richness,  it  also  take  well  to  a  large  variety  of  finishes.  There is literally something for every person taste.

Uneven shine finish is called distressed types of finish.  In  this , finish  is  a  combination  of  physically  ‘ DISTRESSING”  the  leather.  For  example  the  process  might  include  using  a  hairbrush  and  then  applying  wipes  of  pigments  to  produce  an  uneven  color.  The  goal  is  to  make  a  item  look  old  or  worn.

Another  way  to  distress  leather  is  to  spot  with  water  and  then  as  it  dries,  it  will  shrink  slightly  giving  it  a  pinched  look.  If  you  prefer,  you  can  simply  wear  it  and  let  natural  take  its  course.

## 4.4. Quality records and documentation

4.4.1 Preparation of Daily Production Report

Daily production (Feeding & Packing) report must be prepared by the lasting in-charge on same day and must be placed on table of the production manager by 9.00 AM. The report must have the details of feeding & packing of shoes in all the lines. Only packed shoes will be considered as the production of the department rest everything must be considered as WIP of lasting department. The format for the daily production report is given below-

Table 4. 1 daily production report chart

| XYZ SHOE FACTORY |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LASTING DEPARTMENT |  |  |  |  |  |  |  |  |  |  |  |  |
| DAILY FEEDING & PACKING REPORT |  |  |  |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  | ATTENDANCE: |  |  |  |
|  |  |  |  | TYPE | SIZES |  |  |  |  |  |  | TOTAL |
|  |  |  |  | M | 38 | 39 | 40 | 41 | 42 | 43 | 44 |  |
| FEEDING |  |  |  | W | 34 | 35 | 36 | 37 | 38 | 39 | 40 |  |
| LINE NO. | PLAN NO. | MODEL | COLOUR | C | 33 | 34 | 35 | 36 | 37 | 38 |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |
| PACKING |  |  |  |  |  |  |  |  |  |  |  |  |
| LINE NO. | PLAN NO. | MODEL | COLOUR |  |  |  |  |  |  |  |  | TOTAL |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |
| REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |
| PERPARED BY: |  |  |  |  |  |  |  |  |  | APPROVED BY: |  |  |

| XYZ SHOE FACTORY |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Final packing report |  |  |  |  |  |  |  |  |  |  |  |  |
| PACKING REPORT |  |  |  |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  | ATTENDANCE: |  |  |  |
| PACKING LABEL: |  |  |  | TYPE | SIZES |  |  |  |  |  |  | TOTAL |
|  |  |  |  | M | 38 | 39 | 40 | 41 | 42 | 43 | 44 |  |
|  |  |  |  | W | 34 | 35 | 36 | 37 | 38 | 39 | 40 |  |
|  | PLAN NO. | MODEL | COLOUR | C | 33 | 34 | 35 | 36 | 37 | 38 |  |  |
|  |  |  |  |  |  | SIZES |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |
| PACKING |  |  |  |  |  |  |  |  |  |  |  |  |
| LINE NO. | PLAN NO. | MODEL | COLOUR |  |  |  |  |  |  |  |  | TOTAL |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |
| REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |
| PERPARED BY: |  |  |  |  |  |  |  |  |  | APPROVED BY: |  |  |

The ready shoes must be packed in the shoe boxes with the packing slip duly filled.

The dispatch of the shoes must be done in specific size assortment.

Table 4. 2 footwear inspection certificate

| Footwear inspection certificate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Style ref: 4130368 |  | Order No: 27732 |  | Customer: DUNNES STORES |  |
| Supplier:XYZ |  | Location: India |  | Quantity to Inspect: 84 pairs |  |
| Inspected by: GUSTO FOOTWEAR |  | Date Inspected 31-01-13 |  | Order Qty- 804 Pairs |  |
| Previous most common fault: |  |  | Inspected carton -No.'s 13,67,50,32,25,1&10 |  |  |
| Confirmation Sample Available: YES |  |  |  |  |  |
| Size Range From 37 to 42 |  |  | Sizes Inspected- ALL SIZES |  |  |
| FAULTS | MAJOR | MINOR | FAULTS | MAJOR | MINOR |
| Dirty/Stained on heel grip | Nill | nill | Dirty/col-Stained on sole | NIL | NIL |
| Socks pasting problem | NIL | NIL | Heel Fault | NIL | NIL |
| Colour Fault (VP) | NIL | NIL |  |  |  |
| Stitching problems | NIL | NIL |  |  |  |
| Lining/Counter flaw/faults | NIL | NIL | Poor Sole Attachement |  | 1 |
| Adhesive mark on leather | NIL | NIL | Poor Heel Attachement | NIL | NIL |
| Sole balancing problem | 1 | NIL | Toe Shape/ tillted | NIL | NIL |
| Spot mark on leather | NIL | NIL | Damage welt | NIL | NIL |
| Qtr Fixing/ Eyelet facing prob | NIL | NIL |  |  |  |
| Trim/Buckle/Wrinkle on top ine binding | NIL | NIL | Over roughing |  | 1 |
| Yellowing/Loose leather | 2 | NIL | Poor finishing |  | 1 |
| Wrinkle on vamp | 1 | NIL | GRAND TOTAL | 4 | 3 |
| PACKAGING & LABELLING |  |  |  |  |  |
| Swing Tickets(MN) | NIL | NIL | BAR CODE STICKER REF | NOT OK | NOT OK |
| Damaged box end label(MJ) | NIL | NIL | Egg crates(MJ) | NIL | NIL |
| EAN number in lining(MJ) | NOT OK | NOT OK |  |  |  |
| Shipping mark(RP/Bulk)(MJ) | NIL | NIL | Ticketing incur. Position(MJ) |  | NIL |
| COMMENTS ON DEFECTS & ACTIONS TO BE TAKEN |  |  |  |  |  |
| 7 pair of major & minor are above mentioned faults were found in this shipment. |  |  |  |  |  |
| Date referred to QC Dept |  |  |  | RESULT Pass | Pass |
| Packing Details |  |  |  |  |  |
| Note: Quality is ok but packing is not ok by my end so pleases recheck it again |  |  |  |  |  |
|  | Signature of Inspector with Company seal. |  |  |  |  |

## Self chek-4

Directions:  Answer all the questions listed below. Examples may be necessary to aid some explanations/answers.

Test I: Multiple Choice

1 one of the following is Key to a good quality system

- Everyone must know what to check and how to check,

- List major and minor defects and display it,

- Check all inputs materials for quality / quantity,

- all

2 which one of the following are the main reasons for improper brushing-

- Due  to  the  dirty  sponges  and  contaminated  cleaner  pots,

- Due  to  high  humidity,  water  in  solvent,  or  faulty  air  supply,

- Due  to  excessive  heat  supply  in  process,

- all

3 which type of fault mostly arises  due  to  uneven  dressing,  cutting  as  well  as  uneven  polishing

- Un even shine,

- streaks on upper,

- A and B ,

- none

Test II: Short Answer Questions:

Instruction: write short answer for the given question.

- What is quality?

- How Can a Supervisor Control quality on his line?

- What are main area of quality control?

- What is the main objective to achieve a consistent quality standard for the product?

- Describe footwear dispatching?

# REFERENCES

- Former TTLM level ii perform footwear finishing operation

- www.researchgate.net

- www.nzdl.org

Experts Who Developed this Learning Guide

| No | Name | Educational Background | Region | E-mail | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Yohannes Fisseha | BSC in Leather Engineering | Addis Ababa, Lideta Manufacturing college | yohannesfiseha71@gmail.com | module Developers Team Members |
| 2 | Yabetse Fikadu | BSC in Leather Engineering | oromia: bishoftu polytechnic college | Yabets1030@gmail.com |  |
| 3 | H/Gebrale Abebe | BSC in leather and leather product technology | Addis Ababa,Misrak poly technic colleget | haile3a@yahoo.com |  |