Surveying Technology

Level-II

Based on December 2022, Curriculum Version 2

Module Title: - Control survey points

Module code: EIS SUR2 M04 06 22

Nominal duration: 250 Hours

Prepared by: Ministry of Labour and Skill

December, 2022

Addis Ababa, Ethiopia

Table of Contents	     Page

UNIT ONE:INTRODUCTION OF CONTROL POINT	6

1.1.Diffinition of control point	7

1.2.Surveying  terminology and Control Survey	7

1.3. Materials, Tools and equipments of control point	7

Self check 1.1	15

Written test	15

UNIT TWO :TRAVERSE	16

2.1.Introduction of traverse	17

2.1.1. Traverse stations or traverse points.	17

2.2. Types of traverse	18

2.2.1 Types of close traverse	19

2.3. Horizontal and vertical angels	20

2.4. Azimuth and bearing	27

2.4.2. Back azimuth	29

2.5. Fieldwork activities	31

2.6. Computing coordinates	31

2.7. Area computation	51

2.7.1.Area Computation by coordinate method	51

2.7.2.Area Computation by meridian system	55

Exercise1	55

Self check-2	59

3.1. Introduction Triangulation	63

3.2. Triangulation stations	64

3.3. Establishing  monuments	65

3.4. Measuring angle and base line	65

3.5.Computing  triangulation	67

2.5.1.Adjustment of Chain of Single Triangles	67

3.4.2  Adjustment of a quadrilateral	69

3.5.3. Eccentric Stations (Reduction to centre)	73

3.5.3. Computation of lengths	76

3.5.4. Coordinate calculations	78

UNIT FOUR:   INTERSECTION	82

4.1.Introduction of Intersection	83

4.2.Computing Intersection	85

UNIT FIVE: RE SECTION	90

5.1.INTRODUCTION OF  RESECTION	91

5.2. Writing the description for station	92

5.3.  Establishing Resection	93

5.4. computing resection	97

UNIT SIX: TRILATRATION	104

6.1. TRILATRATION SYSTEM	105

6.2. ADJUSTMENT OF MEASURED DISTANCES	106

Acknowledgment

Ministry of Labor and Skills wish to extend thanks and appreciation to the many representatives of TVET instructors and respective industry experts who donated their time and expertise to the development of this Teaching, Training and Learning Materials (TTLM).

Introduction to the Module

In Surveying filed; the Control survey points helps to know the Traversing, Triangulation. Intersection, Re section and Trilatration

This module is designed to meet the industry requirement under the surveying technology occupational standard, particularly for the unit of competency: Control survey points

This module covers the units:

- Introduction

- Traversing

- Triangulation.

- Intersection

- Re section

- Trilatration

Learning Objective of the Module

- Perform Control Survey points

- Perform Traversing.

- Compute Triangulation.

- Establish Intersection

- Establish Re section

- Describe Stations

Module Instruction

For effective use this modules trainees are expected to follow the following module instruction:

- Read the information written in each unit

- Accomplish the Self-checks at the end of each unit

- Perform Operation Sheets which were provided at the end of units

- Do the “LAP test” giver at the end of each unit and

- Demonstrate the given project

- Read the identified reference book for Examples and exercise

# UNIT ONE: INTRODUCTION OF CONTROL POINT

This learning guide is developed to provide you the necessary information regarding the following content coverage and topics: - Definition of control point - Surveying terminology and Control Survey - Materials, Tools and equipments - Safety requirements This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Definite Control Survey points - Apply Safety requirements - Identify materials, tools and equipment

## 1.1.Diffinition of control point

The determination of the precise position of a number of stations, usually spread over a large area, is referred to as control surveying. Control surveys can be horizontal or vertical.

The objective of horizontal control surveys is to establish a network of control stations whose position is specified in terms of latitude and longitude, where as the objective of vertical control survey is to provide the elevations of fixed benchmarks with respect to the mean sea level datum. These control surveys are located where other surveys can be conveniently and accurately tied in to them.

The results of the control surveys are used as a basis from which surveys of smaller extent can be originated. Boundary surveys, construction, route surveys, topographic and hydrographic surveys and other may be involved.

Horizontal control can be carried out by precise traversing, by triangulation, by trilateration, and perhaps by same combination of these methods. Intersection, resection, and satellite positioning are also the other methods in horizontal control surveys. The exact method used depends of the terrain, equipment available, information needed and economic factors.

## 1.2.Surveying  terminology and Control Survey

- Azimuth :An angle measured from north direction up to a given line.

- Bering : is an angle measure from north or south direction up 900

- Station: is a point that is the instrument is set up on.

- Bench mark: is a point that is the elevation is known

- Base line: a line that the value is measured

## 1.3. Materials, Tools and equipments of control point

A. Theodolite

Theodolite Survey is used to measure horizontal and vertical angles. It is also used for levelling, indirect measure of distances and prolonging a line etc. The line of sight of Theodolite can be rotated through 180o in vertical plane about its horizontal axis. Theodolite is a precision instrument for measuring angles in the horizontal and vertical planes.

Parts of Theodolite and their Functions A Theodolite consist of following parts, are as under

Telescope

Figure 1.1 Theodolite

Uses of Theodolite in Surveying

Theodolite uses for many purposes, but mainly it is used for measuring angles, scaling points of constructional works. For example, to determine highway points, huge buildings’ escalating edges Theodolites are used. Depending on the job nature and the accuracy required, Theodolite produces more curved of readings, using paradoxical faces and swings or different positions for perfect measuring survey. Followings are the major uses of Theodolite:  Measuring horizontal and vertical angles

The Theodolite helps us a good within the engineering field. This instrument plays a major role in measurement horizontal angles, vertical angles, bearing, etc. To use Theodolite, it is necessary to know about Theodolite parts, types of Theodolite and for what it is used wisely in the field.

B. Tripod: Tripod is nothing but a stand on which Theodolite is mounted. It should place in such a way that Theodolite should be in exact levelled position. The tripod has legs with steel shoes at their ends. These hold the ground strongly without any movement when placed. Tripod has an external screw which helps to attach the Theodolite by tribratch plate in fixed position.

Figure 1.2 Tripod

C. Plumb Bob: Plumb bob is tool having a cone shaped weight attached to a long thread. The weight is hanged using thread from the centre of tripod stand and centering of Theodolite is done.

Figure 1.3 Plumb

D. Magnetic Compass: Simpler Theodolites may contain circular compass box in the centre of upper plate. When we select north as reference meridian it will be useful.

Figure 1.4  Magnetic Compass

E. Ranging Poles

Ranging poles (see Fig. 7) are used to mark areas and to set out straight lines on the field. They are also used to mark points which must be seen from a distance, in which case a flag may be attached to improve the visibility.

Ranging poles are straight round stalks, 3 to 4 cm thick and about 2 m long. They are made of wood or metal. Ranging poles can also be home made from strong straight bamboo or tree branches.

REMEMBER: Ranging poles may never be curved.

Ranging poles are usually painted with alternate red-white or black-white bands. If possible, wooden ranging poles are reinforced at the bottom end by metal points.

Figure 1.5 Ranging pole

F. Page; A boundary peg is a hardwood stake that is placed at each corner of your property. And it is set down by the surveyor. It is used to indicate the boundary line between your parcel of land and the ones adjacent to it.

Figure 1.6 Page

G. Tapes: Is an instrument used to measure horizontal distance

Figure 1.7 Tape

H. Paper: is a thin sheet material produced by mechanically or chemically processing cellulose fibres derived from wood, rags, grasses or other vegetable sources.

Figure 1.8 Parer

I. Pencil: A pencil is a writing or drawing implement with a solid pigment core in a protective casing that reduces the risk of core breakage, and keeps it from marking the user's hand.

Figure 1.9 Pencil

J. Calculator:

Figure 1.10.Calculatore

K. GPS: GPS, or the Global Positioning System, is a global navigation satellite system that provides location, velocity and time synchronization. GPS is everywhere. You can find GPS systems in your car, your smart phone and your watch. GPS helps you get where you are going, from point A to point B.

The use of GPS is to establish control point.

Figure 1.11.GPS

1.4. Safety requirements:

- To minimize the possibility of damage always  keep the accessory port door closed

- Do not use out of the accepted rang of energy or out of the recommended rang of power which limited by the manufactured organization

- Do not aims the telescope at the sun use the solar filter to avoid casing internal damage to the instrument when observing the sun.

- Protect the instrument from heavy shocks or vibration

- Turn the power off before removing battery

- Adverse use can lead to injury, malfunction and damage.

- The person responsible for the product must ensure that it is used in accordance with the instruction

How to Use a Theodolite

- Mark the point at which the theodolite will be set up with a surveyor’s nail or a stake. This point is the basis for measuring angles and distances.

- Set up the tripod. Make sure the height of the tripod allows the instrument (the theodolite) to be eye-level. The centered hole of the mounting plate should be over the nail or stake.

- Drive the tripod legs into the ground using the brackets on the sides of each leg.

- Mount the theodolite by placing it atop the tripod, and screw it in place with the mounting knob.

- Measure the height between the ground and the instrument. This will be used a reference to other stations.

- Level the theodolite by adjusting the tripod legs and using the bulls-eye level. You can make slight tunings with the leveling knobs to get it just right.

- Adjust the small sight (the vertical plummet) found on the bottom of the theodolite.  The vertical plummet allows you to do ensure the instrument remains over the nail or stake. Adjust the plummet using the knobs on the bottom.

- Aim the crosshairs in the main scope at the point to be measured. Use the locking knobs on the side of the theodolite to keep it aimed on the point. Record the horizontal and vertical angles using the viewing scope found on the theodolite’s side.

| # Self check 1.1 | # Written test |
| --- | --- |

Test-I  Multiple Choose

Instruction: select the correct answer for the give choice. You have given 1 Minute for each question. Each question carries 2 Point.

1. Which instrument use to measure azimuth.

A. Rage pole                    B. Compass                 C. Page            D. Tape

2. The use of tape to measure ___________________

A. Vertical distance  B. Paper distance  C. Horizontal distance   D. Elevation distance

3.Which instrument use for collecting control point

A. Theodolite            B. Computer         C. Protractor                 D. Leveling

Test II: short Answer writing

Instruction: select the correct answer for the give choice. You have given 1 Minute for each question. Each question carries 2 Point

A                                                              B

1. GPS                                                        A. Measure angle

2.Thedolite                                                  B. To measure direction

3.Rage pole                                                 C. Alignment

4. Compass                                                  D. Measure distance

5. Tape                                                        E. To measure angle

Part III: Short answer writing

Direction:  Give short answer to the following questions. Time allotted for each item is 2mniut and each question carry 4 point.

1. What is azimuth

2. What is the use of Theodolite

3.What is the difference between azimuth and Bering

Note: Satisfactory rating – above 60%           Unsatisfactory - below 60%

You can ask you teacher for the copy of the correct answers

# UNIT TWO :TRAVERSE

This unit to provide you the necessary information regarding the following content coverage and topics: - Introduction of traverse - Types of traversing - Horizontal and vertical angels - Azimuth and bearing - Fieldwork activities - Computing coordinates - Areal computation This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify types of traverse - Measure horizontal and vertical angels. - Apply Traverse fieldworks. - Compute azimuth and bearing. - calculate Area

## 2.1.Introduction of traverse

Traversing is a series of horizontal distances and angles are measured. This is generally cheaper due to shorter sights and convenience to carry it out under less favorable weather conditions than the other methods. Its disadvantage is that, there are fewer checks available for locating mistakes in the work and the whole system can rather easily sway or bend. To check a traverse it is necessary to form a loop returning to its starting point or to tie it in to previously established control points.

### 2.1.1. Traverse stations or traverse points.

Any temporary or permanent point of reference over which the theodolite is set up is called a traverse station. On most surveys the traverse station is a peg, called a hub, driven flush with the ground and having a tack driven in its top to mark the exact point of reference for measurement. On pavements, the traverse stations may be a driven nail, a cross cut in the pavement or curb, or a tack set in a hole drilled with a star drill and filled with lead wool. In land surveying the stations are often iron pipes, stones, or other more or less permanent monuments set at the corners, In mountainous country, often the station marks are cut in the natural rock.

2.1.2.Purpose of traverse:

Traversing is a convenient, rapid method for establishing horizontal control. It is particularly useful in densely built up areas and in heavily forested regions where lengths of sight are short so that neither triangulation nor trilateration is suitable. Traverses are made for numerous purposes, including:

- Property surveys to locate or establish boundaries

- Supplementary horizontal control for topographic mapping surveys

- Location and construction layout surveys for highways, railways, and other private public works.

- Ground control surveys for photogrammetric mapping.

Frequently, the traverse is employed to densify the number of coordinated control points in a triangulated or trilaterated horizontal control network.

## 2.2. Types of traverse

There are two types of traverse

A.  Open traverse.                                                     B. Closed traverse.

A. Open traverse

Open traverse is originates at a point of known position and terminates at a point of unknown position. No computational check is possible to detect errors or blunders in distances or directions in this type of traverse. To minimize errors, distances can be measured twice, angle turned by repetition or reiteration methods, magnetic bearings observed on all lines, and astronomic observations made periodically.

In spite of these precautions, an open traverse is a risky proposition and should not be used for any of application where high accuracy is required, since the results are uncertain and will always be subject to question. Open traverse are necessary in certain types of mine surveying. Hence, there are applications of an open traverse when circumstances dictate their use.

Open traverse

V -4

V - 3

V - 2

V-1

Known Point

Figure:2.1 Open traverse

B. Closed traverse:

Closed traverses is originate at a point of known position and close on another point of known horizontal position. A point of known horizontal position is given either by geographic latitude and longitude or by X and Y coordinates on a rectangular coordinates grid system. In Fig traverse MABCDQ originates at known point M with a backside along line MN of known azimuth and closes on known point Q, with a foresight along line QR also of known azimuth. This type of traverse is preferable to all others since computational checks are possible which allow detection of systematic errors in both distance and direction.

R

Q

B

D

C

A

N

M

Figure:2.2 Clouse link traverse

A traverse that originates and terminates on a single point of known horizontal position is called a closed - loop traverse. In fig traverse 1–2 –3 -4 -5 -6 -7-1, which originates and close on point 1,is an example of a closed – loop traverse. This type of traverse permits an internal check on the angle, but there is no way to defect systematic errors in distance or errors in the orientation of the traverse. A closed loop – traverse should not be used for major projects.  To minimize the chance of errors in a closed – loop traverse, all distance measuring equipment must be carefully calibrated and astronomic observation must be made periodically.

A closed – loop traverse that originates and terminates on a point of assumed horizontal position provides an internal check on angles but no check on systematic errors in distance.  An additional disadvantage is that the points are not located on any datum and have no relationship with points in any other survey, although the relative positions are determined. The use of such a method of running a traverse is to be discouraged except under extenuating circumstances, where no feasible alternative is possible.

### 2.2.1 Types of close traverse

A. Loop traverse

Closed-loop Traverse: Closed Traverse creates a closed geometrical shape (polygon). A closed traverse is one that either begins and ends at the same point. Therefore the angles can be closed geometrically and the position closure can be determined mathematically.

Figure:2.3. loop traverse

Closed-link Traverse:

A link traverse is connected to at least two points, at the beginning and at the end of traverses ,whose coordinates have been previously determined. Calculations can be made to check for errors.

Figure:2.4. Link traverse

## 2.3. Horizontal and vertical angels

Horizontal angle: An angle is defined as the difference in direction between two convergent lines. A horizontal angle is formed by the directions to two objects in a horizontal plane.

method of turning angles

Each of the types of traverse discussed above can also be described according to the method of turning the angles in the traverse. Thus, there are traverses by intersection angles, deflection angles, Azimuth method, each of these methods for running traverse are shown below.

Interior angles are measured clockwise or counter-clockwise between two adjacent lines on the inside of a closed polygon figure.

Figure:2.5. Interior angle

In a loop traverse the angles can be measured internally (interior angles) or externally (exterior angles)

7

6

5

4

3

2

1

Traversing by Interior Angles

Figure:2.6. Interior angle

Notice that at Interior angles the loop traverse is routed and numbering at anticlockwise direction;

Exterior angles are measured clockwise or counter-clockwise between two adjacent lines on the outside of a closed polygon figure.

2

3

4

5

6

7

1

Traversing by Exterior angles

Figure:2.7. Exterior angle

Notice exterior angles; the loop traverse is routed and numbering at clockwise direction

Angles to the right are turned from the back line in a clockwise or right hand direction to the ahead line.

Deflection angles

The deflection angle can be measured clockwise (right deflection angle) or anticlockwise (left deflection angle) It is measured from the prolongation of the back tangent to the fore tangent. Most of the time the surveyors prefer to measure the intersection angle and then figure out the deflection angle, since the different between them is 1800 so we can state that:

Right deflection angle = Intersection angle - 1800

Left deflection angle = 1800 – Intersection angle

Figure:2.8. Deflection angle

Azimuth angle

Traversing by azimuth is not a suitable method since the magnetic north – south meridian are not parallel at control points due to the local magnetic declination, in addition some magnetic fields such as electric lines, iron objects near the control points and even solar magnetic storms can  disturb the magnetic meridian position, consequently a high accuracy is not reachable.

MN

MN

MN

MNN

Figure:2.9. Azimuth angle

Angles to the left are turned from the back line in a counterclockwise or left hand direction to the ahead line.

Figure:2.10. Angle to left

Angles to the right are turned from the back line in a counterclockwise or left hand direction to the ahead line.

Figure:2.11. Angle to righte

A vertical angle is formed by two intersecting lines in a vertical plane, one of these lines horizontal.

A vertical angle is an angle formed by two connected lines in the vertical plane*, that is, between a low point and two higher points. Since these angles are in the vertical plane, the lines that form them will usually be lines of sight

A vertical angle is measured in the direction of gravity

Figure:2.12. Vertical angle

Zenith angle: is the complementary angle to the vertical angle and is formed by two intersecting lines in a vertical plane, one of these lines directed toward the zenith.

A Zenith angle is measured from the upper end of the vertical line continuously all the way around, Figure F-3. It ranges from 0° directly upward (zenith) to 90° on the horizontal to 180° directly downward (nadir) to 270° on the opposite horizontal to 360° back at the zenith. This is typical of angle measurements with closed-standards optical transits and theodolites.

Figure:2.13.Zenith angle

Example 1

Figure .2.13. shows combined Vertical (red) and Zenith (blue) angle circles with an instrument at their center as a reference.

Convert a 56°40’ zenith angle to a vertical angle.

V=90000'00'' -56040'00''

Figure:2.14. Zenith angle

Example 2

Convert a 227°30’ zenith angle to a vertical angle.

Figure:2.15. Zenith angle

Example 3

Which zenith angles are the same as a -36°10’ vertical angle?

Recall that there are two vertical angles one on each side of the zenith circle.

Figure:2.16.Vertical angle

## 2.4. Azimuth and bearing

2.4.1. Azimuth:

-It is an angle measured clockwise from a reference north.

-It is always measured clock wise and north direction

-Its range is [0,360]

- Unlike bearing the direction will not mentioned

- Azimuth sometime called Whole Circle Bearing

Example.1

Azimuth of line OA =θ

Azimuth of line OB =β

Range of Azimuth in different quadrants

In I Quadrant

In I Azimuth between 00 - 900

In II Quadrant Azimuth is between 900 - 1800

In III Quadrant Azimuth is between 1800 -2700

In IV  quadrant Azimuth is between 2700 - 3600

The relationship between azimuth and bearing

The relationship b/n bearing and azimuth in different quadrants is different.

In I quadrant Angular Value of bearing = Azimuth

In II quadrant Azimuth = 180 0 - angular value of bearing

In III quadrant Azimuth = 180 0+ angular value of bearing

In IV Quadrant Azimuth = 3600-angular Value of bearing

### 2.4.2. Back azimuth

Similarly to bearing the back azimuth is the reverse of forward azimuth

Back azimuth of AB = Azimuth of BA

The angular difference b/n forward and backward azimuth is equal to 1800

Computing a back azimuth from forward azimuth

If a forward azimuth is less than 1800, Backward azimuth = forward Az + 1800

If a forward azimuth is greater than 1800 ,Backward azimuth = foreword azimuth +1800

Example

Find the backward azimuth of the following lines having foreword azimuth

Line                                   Forward azimuth

AB                                        43011’20’’

BC                                        1120 20’15’’

CD                                        1970 18’36’’

DE                                         3200 17’40’’

Solution

Since Az AB < 900

Az BA = Azimuth of AB +1800

=   43011’20’’ + 1800

= 2230 11’20’’

Since 900 < Az BC< 1800

Back azimuth of line BC= 112029’15+1800

= 290029’15’’

Since 1800 < Az CD< 2700

Back azimuth of CD = Az CD-1800

=197018’36’’-1800

= 170 18’36’’

D. Since 2700 < Az DE < 3600

Back azimuth DE = Az DE -1800

= 320017’40’’-1800

= 1400 17’40’’

## 2.5. Fieldwork activities

Select equipment, tools and material for measuring angle distance, direction and one coordinate for calculating traverse.

## 2.6. Computing coordinates

General Step (for a loop traverse)

Angular Stage

I .Computing the angular closure error ec=  ∑ α – 1800 (n – 2)

II. Determining the angular permissible error = 1’ √n  or30’’√n   0r 10’’√n

III .Determining the angular correction   c = ec/n

IV. Appling the angular correction to interception angles α(corrected)  = α ± c

(the correction is introduced into the angle with the error appositive sign)

V .Checking the angular correction: ∑α(corrected) = 1800 (n – 2)

VI. Computing the azimuth of every line ABC = AAB + α(corrected) ± 1800

VII. Determining the bearing upon the azimuth of every line.

In first quadrant Br. = Az

VIII Computing the differences in X and Y: ∆X2 = dist1 * sin Azimuth1-2

∆Y2 =  dist1 * cos Azimuth1-2

II Determining the total error in X: X e = ∑∆X total error in Y: Ye = ∑∆Y ____________

III Determining the linear absolute error Ae = √ ∑∆X2+ ∑∆Y2

IV Computing the Precision or liner relative error: P = 1D/Ae

V Comparing the achieved Precision with the expected Precision or Project Precision, It must to be fulfilled that: P > PP (otherwise the field data are not acceptable)

VI Computing the correction for ∆X and ∆y: C = ∑∆X   * dist

D

C = ∑∆Y  *dist

D

VII Appling the correction to ∑∆X and ∑∆Y :∑∆X(corrected) = ∑∆X ± C

∑∆Y(corrected) = ∑∆Y ± C

VIII Checking the linear correction now it must be fulfilled That :

∑∆X = 0

∑∆Y = 0

IX Computing the final coordinate of every traverse vertex :

Xb = Xa ± ∑∆X(corrected)

Yb = Ya ± ∑∆Y(corrected)

X Filling out the calculus compendium

Example1  (loop traverse)

2.6. Areal  computation

2.6.1. coordination  methods

2.6.2. meridian system

1100 53’ 10’’

2000 54’ 14’’

790 07’ 00’’

2190 57’ 02’’

550 30’ 06’’

354. 51

318.54

274.17

469. 65

246. 57

365.01

Azimuth

1 – 2

1100 00’ 00’’

MS

MN

6

5

4

3

2

1

530 38’ 02’’

Known data:  Pp = 1 : 10 000      X1 = 5000.00      Y1= 2000.00

I – Determining the angular closure error :ec=  ∑ α – 1800 (n – 2)

ec =  7190 59’ 34’’ – 1800 (6 – 2 ) =  7190 59’ 34’’ – 7200 00’ 00’’

ec = 26’’

II – Determining the angular permissible error: pe = 1’ √n

pe = 1’√6 = 1’ x 2.4494897 = 2’ 27’’  then  ce<petherefore  the current angular error is acceptable.

III - Determining the angular correctionc = ec

n

c = 26’’  c = 4.3’’

6

IV - Appling the angular correction to interception anglesα(corrected)  = α ± c

(the correction is introduced into the angle with the error appositive sign)

α(corrected 1 ) = 550 30’ 06’’ + 5’’ = 550 30’ 11’’

α(corrected 2 ) = 2190 57’ 02’’ + 4’’ = 2190 57’ 06’’

α(corrected 3 ) =790 07’ 00’’ + 4’’ = 790 07’ 04’’

α(corrected 4 ) = 530 38’ 02’’ + 4’’ = 530 38’ 06’’

α(corrected 5 ) =2000 54’ 14’’ + 4’’ = 2000 54’ 18’’

α(corrected 6  ) =1100 53’ 10’’ + 5’’ = 1100 53’ 15’’

Total = 26’’     Now ∑α = 1800 (n – 2 ) = 7200 00’ 00’’

VI Computing the azimuth of every line  ABC= AAB + α(corrected) ± 1800

Azimuth of line 1 – 2 is known = 1100 00’ 00’’

Azimuth of line 2 – 3 = Azimuth of line 1 – 2 + α2 ± 1800

Azimuth of line 2 – 3 = 1100 00’ 00’’ + 2190 57’ 06’’ - 1800

Azimuth of line 2 – 3 = 3290 57’ 06’’ - 1800 = 1490 57’ 06’’

Azimuth of line 3 – 4 = Azimuth of line 2 – 3 + α3 ± 1800

Azimuth of line 3 - 4 = 1490 57’ 06’’+ 790 07’ 04’’ - 1800

Azimuth of line 3 – 4 = 2290 04’ 10’’ – 1800 = 490 04’ 10’’

Azimuth of line 4 – 5 = Azimuth of line 3 – 4 + α4 ± 1800

Azimuth of line 4 – 5 = 490 04’ 10’’ + 530 38’ 06’’ + 1800

Azimuth of line 4 – 5 = 1020 42’ 16’’ + 1800 = 2820 42’ 16’’

Azimuth of line 5 – 6 = Azimuth of line 4 – 5 + α5 ± 1800

Azimuth of line 5 – 6 = 2820 42’ 16’’ + 2000 54’ 18’’ -1800

Azimuth of line 5 – 6 = 4830 36’ 34’’ -1800 = 3030 36’ 34’’

Azimuth of line 6 – 1 = Azimuth of line 5 – 6 + α6 ± 1800

Azimuth of line 6 – 1 = 3030 36’ 34’’ + 1100 53’ 15’’ – 1800

Azimuth of line 6 – 1 = 4140 29’ 49’’ -1800 = 2340 29’ 49’’

Azimuth of line 1 – 2 = Azimuth of line 6 – 1 + α1 ± 1800

Azimuth of line 1 – 2 = 2340 29’ 49’’ + 550 30’ 11’’ – 1800

Azimuth of line 1 – 2 = 2900 00’ 00’’ – 1800 = 1100 00’ 00’’

VII Determining the bearing upon the azimuth of every line.

Azimuth 1 - 2 = 1100 00’ 00’’

Bearing 1 -2 = 1800 – Azimuth = 1800 - 1100 00’ 00’’ = 700 00’00’’ SE

Azimuth 2 - 3 = 1490 57’ 06’’

Bearing 2 – 3 = 1800 – Azimuth = 1800 - 1490 57’ 06’’ = 300 02’ 54’’ SE

Azimuth 3 - 4 = 490 04’ 10’’

Bearing 3 – 4 = Azimuth = 490 04’ 10’’ NE

Azimuth 4 - 5 = 2820 42’ 16’’

Bearing 4 – 5 = 3600-  Azimuth = 770 17’ 44’’ NW

Azimuth 5 - 6 = 3030 36’ 34’’

Bearing 5 – 6 = 3600 – Azimuth = 560 23’ 26’’ NW

Azimuth 6 -1 =  2340 29’ 49’’

Bearing 6 – 1 = Azimuth – 1800 = 540 29’ 49’’ SW

Linear Stage

I Computing the differences in X and Y: ∆X1 = dist1 * sin Azimuth1-2

∆Y1 =  dist1 * cos Azimuth1-2

∆X Computing

∆X1 = 365.01 x sin (1100 00’ 00’’) = 342.997

∆X2 = 246. 57 x sin (1490 57’ 06’’) = 123.465

∆X3 = 469. 65 x sin (490 04’ 10’’) = 354.822

∆X4 = 274.17 x sin (2820 42’ 16’’) = -267.458

∆X5 = 318.54 x sin (3030 36’ 34’’) = -265.290

∆X6 = 354. 51 x sin (2340 29’ 49’’) = -288.601

∑∆X = - 0.065

∆Y Computing

∆Y1 = 365.01 x cos (1100 00’ 00’’) = -124.841

∆Y2 = 246. 57 x cos (1490 57’ 06’’) = -213.432

∆Y3 = 469. 65 x cos (490 04’ 10’’) = 307.688

∆Y4 = 274.17 x cos (2820 42’ 16’’) = 60.296

∆Y5 = 318.54 x cos (3030 36’ 34’’) = 176.321

∆Y6 = 354. 51 x cos (2340 29’ 49’’) = -205.880

∑∆Y = 0.152                                                          ___________

III Determining the linear absolute error Ae = √ ∑∆X2+ ∑∆Y2

____________

Ae = √ -0.0652 + 0.1522  =0.165

IV Computing the Precision or liner relative error: P = 1

D

1 Ae

P =  2 028.45    = 1: 12 293 ≈ 1: 12 200

0.165

V Comparing the achieved Precision with the expected Precision or Project Precision, It must to be fulfilled that: P > PP (otherwise the field data are not acceptable)

P > PP since 1: 12 200 > 1: 10 000 therefore OK

VI Computing the correction for ∆X and ∆y: C = ∑∆X   * dist

D

C = ∑∆Y  *dist

D

C =∆X

C = ∑∆X   * dist

D

C1 =   - 0.065__ x    365.01 =   0.012 m = 12 mm

2028.45

C2 =   - 0.065__  x 246. 57 = 0.008 m = 8 mm

2028.45

C3 =   - 0.065__  x 469. 65 = 0.015 m = 15 mm

2028.45

C4 =   - 0.065__  x 274.17 = 0.009 m = 9 mm

2028.45

C5 =   - 0.065__  x 318.54 = 0.010 m = 10 mm

2028.45

C6 =   - 0.065__  x 354. 51 = 0.011 m = 11mm

2028.45

∑CX = 0.065 m = 65 mm

C= ∆Y

C1 =   0.152__  x  365.01 =  0.027m = 27 mm

2028.45

C2 =   0.152__  x 246. 57 =  0.019 m = 19 mm

2028.45

C3 =   0.152__  x 469. 65 = 0.035m = 35 mm

2028.45

C4 =   0.152__  x  274.17 = 0.021m = 21 mm

2028.45

C5 =   0.152__  x 318.54 = 0.024 m = 24 mm

2028.45

C6 =   0.152__  x 354. 51 =  0.026 m = 26 mm

2028.45

∑CY = 0. 152 m = 152 mm

VII  Appling the correction to ∑∆X and ∑∆Y : ∑∆X(corrected) = ∆X ± C

∑∆Y(corrected) = ∆Y ± C

In this case the correction is going to be applied with positive sign since the error is negative. (correction must be applied with appositive sign of error)

Correction in ∆X

∆X1 (corrected) = 342.997 + 0.012  = 343. 009

∆X2(corrected) = 123.465 + 0.008 = 123.473

∆X3(corrected) = 354.822 + 0.015 = 354. 837

∆X4(corrected) = -267.458 + 0.009 = - 267.449

∆X5(corrected) = -265.290 + 0.010 = - 265.280

∆X6(corrected) = -288.601 + 0.011 = - 288.590

VIII Checking the linear correction now it must be fulfilled That : ∑∆X = 0

Correction in ∆Y

In this case the correction is going to be applied with negative sign since the error is positive  (correction must be applied with appositive sign of error)

∆Y1(corrected)  = -124.841 -  0.027 = - 124. 868

∆Y1(corrected)  =  -213.432 - 0.019 = - 213. 451

∆Y1(corrected)  =  307.688 - 0.035 = 307.653

∆Y1(corrected)  =  60.296 -  0.021= 60.275

∆Y1(corrected)  =  176.321 - 0.024 = 176.297

∆Y1(corrected)  =  -205.880 - 0.026 = - 205. 906

VIII Checking the linear correction now it must be fulfilled That :  ∑∆Y = 0

IX Computing the final coordinate of every traverse vertex :

Xb = Xa ± ∑∆X(corrected)

Yb = Ya ± ∑∆Y(corrected

Computing coordinate X :

X2  = X1 + ∆X1 = 5000.00 + 343. 009 = 5 343. 009

X3  = X2 + ∆X2 = 5 343. 009 + 123.473 = 5 466.482

X4  = X3 + ∆X3 = 5 466.482 + 354. 837 = 5 821. 319

X5  = X4 - ∆X4 =  5 821. 319 - 267.449 = 5 553. 87

X6  = X5 - ∆X5 =  5 553. 87 - 265.280 = 5 288.59

X1  = X6 - ∆X6 =  5 288.59 - 288.590 = 5000.00  OK.

Computing coordinate Y :

Y2 = Y1 - ∆Y1 = 2000.00 - 124. 868 = 1 875.132

Y3 = Y2 - ∆Y2 = 1 875.132 - 213. 451 = 1 661.681

Y4 = Y3 - ∆Y3 = 1 661.681 + 307.653 = 1 969.334

Y5 = Y4- ∆Y4 =  1 969.334 + 60.275 = 2 029. 609

Y6 = Y5 - ∆Y5 = 2 029. 609 + 176.297 = 2 205. 906

Y1 = Y6 - ∆Y6 = 2 205. 906 - 205. 906 = 2000.00

Computing coordinate X :

X2  = X1 + ∆X1 = 5000.00 + 343. 009 = 5 343. 009

X3  = X2 + ∆X2 = 5 343. 009 + 123.473 = 5 466.482

X4  = X3 + ∆X3 = 5 466.482 + 354. 837 = 5 821. 319

X5  = X4 - ∆X4 =  5 821. 319 - 267.449 = 5 553. 87

X6  = X5 - ∆X5 =  5 553. 87 - 265.280 = 5 288.59

X1  = X6 - ∆X6 =  5 288.59 - 288.590 = 5000.00  OK.

Computing coordinate Y :

Y2 = Y1 - ∆Y1 = 2000.00 - 124. 868 = 1 875.132

Y3 = Y2 - ∆Y2 = 1 875.132 - 213. 451 = 1 661.681

Y4 = Y3 - ∆Y3 = 1 661.681 + 307.653 = 1 969.334

Y5 = Y4- ∆Y4 =  1 969.334 + 60.275 = 2 029. 609

Y6 = Y5 - ∆Y5 = 2 029. 609 + 176.297 = 2 205. 906

Y1 = Y6 - ∆Y6 = 2 205. 906 - 205. 906 = 2000.00

| Station | sid | Adjusted Horizontal Angle | Azimuth | Bearing | Distan | Prelim ∆X | C | Adjusted ∆X | Prelim ∆Y | C | Adjusted ∆Y | Coordinates |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | X | Y |  |
| 1 |  | 55030’11’’ |  |  |  |  |  |  |  |  |  | 5000.00 | 2000.00 |  |
|  | 1 – 2 |  | 110000’00’’ | 70000’00’’SE | 365.01 | 342.997 | 0.012 | 343. 009 | -124.841 | -0.027 | - 124. 868 |  |  |  |
| 2 |  | 219057’0’’ |  |  |  |  |  |  |  |  |  | 5343.009 | 1 875.132 |  |
|  | 2 – 3 |  | 149057’06’’ | 30002’54’’SE | 246. 57 | 123.465 | 0.008 | 123.473 | -213.432 | - 0.019 | - 213. 451 |  |  |  |
| 3 |  | 790 07’ 04’’ |  |  |  |  |  |  |  |  |  | 5 466.482 | 1 661.681 |  |
|  | 3 – 4 |  | 490 04’ 10’’ | 49004’10’’NE | 469. 65 | 354.822 | 0.015 | 354. 837 | 307.688 | - 0.035 | 307.653 |  |  |  |
| 4 |  | 530 38’ 06’’ |  |  |  |  |  |  |  |  |  | 5 821.319 | 1 969.334 |  |
|  | 4 – 5 |  | 282042’16’’ | 77017’44’’NW | 274.17 | -267.458 | 0.009 | - 354. 837 | 307.688 | - 0.021 | 60.275 |  |  |  |
| 5 |  | 200054’18’’ |  |  |  |  |  |  |  |  |  | 5 553. 87 | 2 029. 609 |  |
|  | 5 – 6 |  | 303036’34’’ | 56023’26’’NW | 318.54 | -265.290 | 0.010 | - 265.280 | 176.321 | - 0.024 | 176.297 |  |  |  |
| 6 |  | 110053’15’’ |  |  |  |  |  |  |  |  |  | 5 288.59 | 2 205. 906 |  |
|  | 6 – 1 |  | 234029’49’’ | 54029’49’’SW | 354. 51 | -288.601 | 0.011 | - 288.590 | -205.880 | - 0.026 | - 205. 906 |  |  |  |
| 1 |  |  |  |  |  | ∑∆X = - 0.065 |  | ∑∆X = 0 | ∑∆Y = 0.152 |  | ∑∆Y = 0 | 5000.000 | 2000.000 |  |
| ec = 26’’ pe = 1’√6 = 1’ x 2.4494897 = 2’ 27’’ then ce < pe therefore the current angular error is acceptable. 1___ Ae= 0.165 P = 2 028.45 = 1: 12 293 ≈ 1: 12 200 Pp = 1 : 10 000 0.165 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

General Step (for a Link  traverse

D

C

3

2

1

B

A

Figure:2.14.Link traverse

I – Computing the azimuth of the two known line of traverse:

- Azimuth of the traverse first line (known line)

- Azimuth of the traverse last line (known line)

Bearing (A – B) = tan ∆xand  Bearing ( C – D) = tan ∆x

∆y                                 ∆y

∆x(A – B) = xB– xA∆y(C – D)  = yD– yC

After knowing bearing of both lines the azimuth is determine according to the bearing quadrant regarding:

+  ∆x    First quadrant           +  ∆x    Second quadrant

-  ∆y                                   -  ∆y

-  ∆x   Third quadrant            -  ∆x    Fourth quadrant

-  ∆y                                      + ∆y

II -  First azimuth calculate

A2- 1 = A1-2 + α  ± 1800

III – Computing the angular closure error

The angular error is the difference between the last line azimuth by data (previously calculated) and the last line azimuth by calculate. Then:

ec=  Azimuth of Known line (by data)  - Azimuth of Known line(by calculus)

II Determining the angular permissible error = 1’ √n  or30’’√n   0r 10’’√n       ‘

III Determining the angular correction   c = ec

n

IV Appling the angular correction to interception angles α(corrected)  = α ± c

(the correction is introduced into the angle with the error appositive sign)

V Computing the azimuth of every line  A2-3 = A1-2 + α(corrected) ± 1800

(Second azimuth calculate)

VI Checking angular correction: Azimuth: A C – D = A C – D

A(last line by calculate)  = A(last line by  previously calculated)

VII Determining the bearing upon the azimuth of every line.

Linear Stage

I Computing the differences in X and Y: ∆X1 = dist1 * sin Azimuth1-2

∆Y1 =  dist1 * cos Azimuth1-2

II Determining the total error in X :X e = Known ∆X – Previous  ∆X

Known ∆X = X of first known point – X of last known point

Known ∆X = XB– XC   and   Previous ∆X = ∑∆X

Y e = Known ∆Y – Previous ∆Y

Known ∆Y = Y of first known point – Y of last known point

Known ∆Y = YB– YC   and   Previous ∆Y = ∑∆Y

___________

III Determining the linear absolute error Ae = √ Xe2 + Ye2

IV Computing the Precision or liner relative error: P = 1

D

Ae

V Comparing the achieved Precision with the expected Precision or Project Precision, It must to be fulfilled that: P > PP (otherwise the field data are not acceptable)

VI Computing the correction for ∆X and ∆y: C = Xe  *dist

D

C = Ye * dist

D

VII Appling the correction to ∑∆X and ∑∆Y :∑∆X(corrected) = ∆X ± C

∑∆Y(corrected) = ∆Y ± C

VIII Checking the linear correction now it must be fulfilled That :

∑∆X(corrected) = Known ∆X

∑∆Y(corrected) = Known ∆Y

IX Computing the final coordinate of every traverse vertex :

X2 = X1 ± ∑∆X(corrected)

Y2 = Y1 ± ∑∆Y(corrected)

X Filling out the calculus compendium

Example

Known points coordinates

Field Data unadjusted

270.86

502.74

346.21

448.62

2370 08’ 45’’

1760 10’ 00’’

930 46’ 15’’

1160 52’ 45’’

2710 38’ 00’’

3

2

1

D

C

B

A

X                           Y

A      4 807.53                      2 640.11                    B       5000.00                    2 200.00

C      5 407.36                    1 524. 85

D      5 651. 71                  1 0 66.26

A      4 807.53                      2 640.11

B       5000.00                    2 200.00

C      5 407.36                    1 524. 85

D      5 651. 71                  1 0 66.26

Pp = 1 : 2000 (Project’s Precision)

Angular Stage

I – Computing the azimuth of the two known line of traverse:

- Azimuth of the traverse first line (known line)

- Azimuth of the traverse last line (known line)

Tan of Bearing (A – B)  =∆x   and  Tan of Bearing ( C – D) = tan ∆x

∆y                                              ∆y

∆x(A – B) = xB– xA∆y(C – D)  = yD– yC

Bearing (A – B) = tan ∆x   = tan 5000 - 4 807.53    = 192.47  = 0. 4373224

∆y            2 200  -2 640.11    - 440.11

Bearing (A – B)  = 23.620836 = 230  37’   15’’  SE

Azimuth (A – B )= 1800 – Bearing = 1800 – 230 37’ 15’’ = 1560 22’ 45’’

Tan of Bearing (C – D )= tan ∆x  = XD – X C=  5 651.71 - 5 407.36

∆y      YD - YC       1 066.26 -1 524. 85

Tan of bearing = 244.35    = 0. 5328288 = 28.04998 = 280.03’ 00’’ SE

- 458.59

Azimuth( C -  D ) = 1800 – 280 03’ 00’’ = 1510 57’ 00’’

The bearing of line C – D is on second quadrant since ∆x is positive and ∆y is negative. Now  the angle whose tan = 0. 5328288 is found by means of calculator.

II -  First azimuth calculate

AB- 1 = AA-B + αB  ± 1800= 1560 22’ 45’’ + 2710 38’ 00’’ – 1800 = 2480 00’ 45’’

A1- 2 = AB-1 + α1  ± 1800 = 2480 00’ 45’’ + 1160 52’ 45’’ – 1800 = 1840 53’ 30’’

A2- 3 = A1-2 + α2  ± 1800 = 1840 53’ 30’’ + 930 46’ 15’’ – 1800 = 980 39’ 45’’

A3- C = A2-3 + α3  ± 1800 = 980 39’ 45’’ + 1760 10’ 00’ – 1800 = 940 49’ 45’’

AC- D = A3-C + αC  ± 1800 = 940 49’ 45’’ + 2370 08’ 45’’ – 1800 = 1510 58’ 30’’

The angular error is the difference between the last line azimuth by data (previously calculated) and the last line azimuth by calculate. Then:

ec=  Azimuth of Known line (by calculus)  - Azimuth of Known line(by data)

ec = 1510 58’ 30’’ - 1510 57’ 00’’ = 1’ 30’’

III Determining the angular correction   c = ec

n

c = 1’ 30’’ = 90’’ = 18’’

5

IV Appling the angular correction to interception angles α(corrected)  = α ± c (the correction is introduced into the angle with the error appositive sign)

α(corrected B)  = αB – c = 2710 38’ 00’’ – 18’’ = 2710 37’ 42’’

α(corrected 1)  = α1 – c = 1160 52’ 45’’ – 18’’= 1160 52’ 27’’

α(corrected 2)  = α2 – c = 930 46’ 15’’- 18’’ = 930 45’ 57’’

α(corrected 3)  = α3 – c = 1760 10’ 00’ – 18’’ = 1760 09’ 42’’

α(corrected C)  = αC  – c = 2370 08’ 45’’ – 18’’ = 2370 08’ 27’’

V Computing the azimuth of every line  A2-3 = A1-2 + α(corrected) ± 1800 (second azimuth calculate)

AB- 1 = AA-B + αB  ± 1800= 1560 22’ 45’’ +2710 37’ 42’’– 1800 = 2480 00’ 27’’

A1- 2 = AB-1 + α1  ± 1800 = 2480 00’ 27’’ +1160 52’ 27’’– 1800 = 1840 52’ 54’’

A2- 3 = A1-2 + α2  ± 1800 = 1840 52’ 54’’+ 930 45’ 57’’– 1800 = 980 38’ 51’’

A3- C = A2-3 + α3  ± 1800 = 980 38’ 51’’ +1760 09’ 42’’– 1800 = 940 48’ 33’’

AC- D = A3-C + αC  ± 1800 = 940 48’ 33’’ +2370 08’ 27’’ – 1800 = 1510 57’ 00’’

VI Checking angular correction: Azimuth: A C – D = A C – D

A(last line by calculate)  = A(last line by  previously calculated)

1510 57’ 00’’ = 1510 57’ 00’’

VII Determining the bearing upon the azimuth of every line.

Azimuth (A – B)  = 1560 22’ 45’’

Bearing A – B )=  1800 - 1560 22’ 45’’ = 230 57’ 15’’ SE

Azimuth (B – 1)  = 2480 00’ 27’’

Bearing B – 1)  = 2480 00’ 27’’ - 1800 = 680 00’ 27’’ SW

Azimuth (1 – 2)  = 1840 52’ 54’’

Bearing 1 – 2)  =  1840 52’ 54’’ - 1800 = 40 52’ 54’’ SW

Azimuth (2 – 3)  = 980 38’ 51’’

Bearing 2 – 3)  = 1800 - 980 38’ 51’’ =   810 21’ 09’’ SE

Azimuth (3 – C)  = 940 48’ 33’’

Bearing 3 – C)  = 1800 - 940 48’ 33’’ =   85011’’ 27’’ SE

Azimuth (C – D)  = 1510 57’ 00’’

Bearing C – D)  = 1800  - 1510 57’ 00’’ =  290 03’ 00’’ SE

Linear Stage

I Computing the differences in X and Y: ∆X1 = dist1 * sin Azimuth1-2

∆Y1 =  dist1 * cos Azimuth1-2

∆X1 = dist(B -1) x  sin Azimuth (B -1 )  = 346.21 x sin 2480 00’ 27’’

∆X1 =346.21 x -0.92723288251153584069008559247961 = -321.017

∆X1 = -321.017

∆X2 = dist(1 – 2) x  sin Azimuth (1 - 2 )  = 448.62 x sin 1840 52’ 54’’

∆X2 = 448.62 x -0.085097995228611273565354210232506

∆X2 = -38.177

∆X3 = dist(2- 3) x  sin Azimuth (2 -3 )  = 502.74 x sin 980 38’ 51’’

∆X3 = 502.74 x 0.98863207178419810056141302150322

∆X2 = 497.025

∆X4 = dist(3 – C)  x  sin Azimuth (3 -C )  = 270.86 x sin 940 48’ 33’’

∆X4 = 270.86 x 0.99647945997790129458998939279526

∆X4 = 269.906

∑∆X = 407.737 (previous ∆X)

Known ∆X = X of first known point – X of last known point

Known ∆X = XC– XB   and   Previous ∆X = ∑∆X

Known ∆X= XC – XB = 5 407.36  -  5000.00  = 407.360

II Determining the total error in X :X e = Previous  ∆X -  Known ∆X

X e = 407.737 - 407.360  = +  0.377

Computing ∆Y:∆Y1 =  dist1 * cos Azimuth1-2

∆Y1 =  dist (B – 1)   x  cos Azimuth B – 1  = 346.21 x cos 2480 00’ 27’’

∆Y1=  346.21 x  -0.37448522212411582973520444955708

∆Y1 = -129.650

∆Y2 =  dist(1 – 2)  x  cos Azimuth1 - 2 = 448.62 x cos 1840 52’ 54’’

∆Y2 = 448.62 x -0.99637258653983011563241867552944

∆Y2 = -446.990

∆Y3 =  dist ( 2 – 3)  x  cos Azimuth 2 – 3  = 502.74 x cos 980 38’ 51’’

∆Y3 = 502.74 x -0.15035500204410950664114139092481

∆Y3 = -75.589

∆Y4 =  dist ( 3 - B )   x  cos Azimuth 3 - B = 270.86 x cos 940 48’ 33’’

∆Y4 = 270.86 x -0.083837258078674138296930728016803

∆Y4 = -22.708

∑∆Y = - 674. 937 (Previous ∆Y)

Known ∆Y = Y of first known point – Y of last known point

Known ∆Y = YC– YB   and   Previous ∆Y = ∑∆Y

Known ∆Y= YC – YB =  1 524. 85 - 2 200.00 = - 675. 150

Y e = Previous ∆Y - Known ∆Y

Y e =   674. 937 –  ( - 675. 150)  = +  0.213

_________

Determining the linear absolute error Ae = √ Xe2 + Ye2

_______________

Ae = √ - 0.3772 + + 0.2132

Ae = 0.433

IV Computing the Precision or liner relative error: P = 1

D

Ae

P = 1 :1 568.43    = 1: 3 622 ≈ 1 : 3 600

0. 433

V Comparing the achieved Precision with the expected Precision or Project Precision, It must to be fulfilled that: P > PP (otherwise the field data are not acceptable).

1 : 3 600 > 1 : 2000

OK

VI Computing the correction for ∆X and ∆y: C = Xe  *dist

D

∆X                                                             C = Ye * dist

D

C1  =- 0.377   x 346.21  = 0. 0002403 x 346.21= + 0.083

1 568.43

C2  =- 0.377   x 448.62  = 0. 0002403 x 448.62 = + 0.108

1 568.43

C3  =- 0.377   x 502.74  = 0. 0002403 x 502.74 = + 0. 121

1 568.43

C4  =- 0.377   x 270.86 = 0. 0002403 x 270.86 = + 0.065

1 568.43

∑C = + 0.377   OK

∆Y

C1 = + 0.213    x 346.21 = 0.0001358 x 346.21 = 0.047

1 568.43

C2 = + 0.213    x 448.62 = 0.0001358 x 448.62 = 0.061

1 568.43

C3 = + 0.213    x 502.74 = 0.0001358 x 502.74 = 0.068

1 568.43

C4  =+ 0.213    x 270.86 = 0.0001358 x 270.86 = 0.037

1 568.43                                              ∑C = 0.213   OK

VII Appling the correction to ∑∆X and ∑∆Y :∑∆X(corrected) = ∆X ± C

∑∆Y(corrected) = ∆Y ± C

∆X

∆X1 (corrected) = ∆X1 (previous)  + C1 = -321.017 - 0.083 = - 321.100

∆X2 (corrected) = ∆X2 (previous)  + C2 = -38.177 - 0.108 = - 38.285

∆X3 (corrected) = ∆X3 (previous)  + C3 = 497.025 - 0. 121 = + 496.904

∆X4 (corrected) = ∆X4 (previous)  + C4 = 269.906 - 0.065 = + 269.841

VIII Checking the linear correction now it must be fulfilled That :

∑∆X(corrected) = Known ∆X = 407.360

∆Y1 (corrected) = ∆Y1 (previous)  - C1 = -129.650 - 0.047 = - 129.697

∆Y2 (corrected) = ∆Y2 (previous)  - C2 = - 446.990 - 0.061= - 447.051

∆Y3 (corrected) = ∆Y3 (previous)  - C3 = -75.589 - 0.068 = - 75.657

∆Y4 (corrected) = ∆Y4 (previous)  - C4 = -22.708 - 0.037 = - 22.745

VIII Checking the linear correction now it must be fulfilled That :

∑∆Y (corrected) = Known ∆Y= - 675. 150

IX Computing the final coordinate of every traverse vertex :

X2 = X1 ± ∑∆X(corrected)

Y2 = Y1 ± ∑∆Y(corrected

XB = 5000.00

X1 = XB - ∆X1 = 5000.00  - 321.100 = 4 678.900

X2 = X1 - ∆X2 = 4 678.900 - 38.285 = 4 640.615

X3 = X2 + ∆X3 = 4 640.615 + 496.904 = 5 137.519

XC= X3 + ∆X4 = 5 137.519 + 269.841= 5 407.36

YB = 2 200.00

Y1 = YB - ∆Y1 = 2 200.00 - 129.697 = 2 070.303

Y2 = Y1 - ∆Y2 = 2 070.303 - 447.051= 1 623.252

Y3 = Y2 - ∆Y3 = 1 623.252 - 75.657 = 1 547.595

YC = Y3 - ∆Y4 = 1 547.595 - 22.745 = 1 524. 85

Table 4.2. Field book

| Station | sid | Adjusted Horizontal Angle | Azimuth | Bearing | Distance | Prelim ∆X | C | Adjusted ∆X | Prelim ∆Y | C | Adjusted ∆Y | Coordinates |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | X | y | Y |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A - B |  | 1560 22’ 45’’ | 230 57’ 15 SE |  |  |  |  |  |  |  |  |  |  |  |
| B |  | 2710 37’ 42’’ |  |  |  |  |  |  |  |  |  | 5000.00 |  | 2 200.00 |  |
|  | B - 1 |  | 2480 00’ 27’’ | 680 00’ 27’’SW | 346.21 | -321.017 | +0.083 | - 321.100 | -129.650 | 0.047 | - 129.697 |  |  |  |  |
| 1 |  | 1160 52’ 27’’ |  |  |  |  |  |  |  |  |  | 4 678.900 |  | 2 070.303 |  |
|  | 1 - 2 |  | 1840 52’ 54’’ | 40 52’ 54’’SW | 448.62 | -38.177 | + 0.108 | - 38.285 | -446.990 | 0.061 | - 447.051 |  |  |  |  |
| 2 |  | 930 45’ 57’’ |  |  |  |  |  |  |  |  |  | 4 640.615 |  | 1 623.252 |  |
|  | 2 - 3 |  | 980 38’ 51’’ | 81021 09’’SE | 502.74 | 497.025 | + 0.121 | + 496.904 | -75.589 | 0.068 | - 75.657 |  |  |  |  |
| 3 |  | 1760 09’ 42’’ |  |  |  |  |  |  |  |  |  | 5 137.519 |  | 1 547.595 |  |
|  | 3 - C |  | 940 48’ 33’’ | 85011’’ 27’’ SE | 270.86 | 269.906 | + 0.065 | + 269.841 | -22.708 | 0.037 | - 22.745 |  |  |  |  |
| C |  | 2370 08’ 27’’ |  |  |  |  |  | ∑∆X= 407.360 |  |  | ∑∆Y = 675.150 | 5 407.36 |  | 1 524. 85 |  |
|  | C - D |  | 1510 57’ 00’’ | 290 03’ 00’’ SE |  |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ec = 1510 58’ 30’’ - 1510 57’ 00’’ = 1’ 30’’ Ae = √ - 0.3772 + + 0.2132 Ae = 0.433 P = 1 : 1,568.43 = 1: 3.622 ≈ 1 : 3 600 0. 433 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.7. Area computation

### 2.7.1.Area Computation by coordinate method

Sometimes it is necessary to compute the area of a tract of land, which may be regular or irregular in shape; land is ordinarily bought and sold on the basic of cost per unit area. To compute volumes of earthworks to be cut or filled in planning a highway it is necessary to compute the areas of cross sections

In IS a unit the area is measured in square meters, hectares or square kilometres.

1 ha = 10 000 m2

1km2= 1000 000m2

The relationship between acres and hectares is:

1hectares = 2.472 acre

1acre = 0.4047 hectare

Area enclosed in a traverse

One of the principal purposes of conducting a boundary survey is to acquire the data needed to determine the area of a tract of land. Usually a traverse is run along the perimeter of the tract and the traverse is balanced. Since the coordinates of the corner Points are thus already available. It is particularly convenient to use these traverse coordinates to compute the area enclosed in the traverse. In fact the area enclosed in cross-section con be compute by its coordinates as a closed traverse.

As a general rule we can say that:

5

4

3

2

1

##### X

###### Y

The area enclosed in a traverse is equal to the product of abscissas multiplied by the difference of backward ordinate minus forward ordinate, and divided by half.

Figure:2.18. Area

As a way of checking; we can say: The traverse area is the product of ordinate multiplied by the different of forward abscissa minus backward abscissa, the partial products are summed and its half in found.

| Point | X | Y | Positive Product | Negative Product |
| --- | --- | --- | --- | --- |
| 1 | X1 | Y1 |  |  |
| 2 | X2 | Y2 | X1y2 | X1y2 |
| 3 | X3 | Y3 | X2y3 | X2y3 |
| 4 | X4 | Y4 | X3y4 | X3y4 |
| 5 | X5 | Y5 | X4y5 | X4y5 |
| 1 | X1 | Y1 | X5y1 | X5y1 |
|  |  |  | Sum 1 | Sum2 |

Another procedure for determine area through a loop traverse coordinates is by multiplying its abscissa and ordinates in cross way on follow:

Note: the traverse stations are listed sequentially and that the first station (station1) is listed sequentially and that the first station (station 1) is listed both at the beginning and at the end. Then by numbering the traverse stations sequentially in a clockwise direction

Adjust the quadrilateral ABCD by dragging any vertex. The area is calculated using this method as you drag. A detailed explanation follows the diagram

Figure:2.19.Area by coordinate

Example

Figure:2.20. Area by coordinate

Figure:2.21. Area by coordinate

I. Distance calculation from coordinate

Figure:2.1. Distance from coordinate

II. Azimuth calculation from coordinate

### 2.7.2.Area Computation by meridian system

### Exercise1

Given data:

| Point | Easting | Northing |
| --- | --- | --- |
| 46 156 257 258 | 78291.06 78862.87 79250.45 79470.58 | 803.44 129.23 88.68 191.87 |

measured:

| from | to | Distance |
| --- | --- | --- |
| 257 311 312 | 311 312 156 | 108.33 132.55 154.31 |

| Station | Target | Direction (GRAD) |
| --- | --- | --- |
| 257 311 312 156 | 258 311 257 312 311 156 46 312 | 0.000 250.417 0.000 183.276 0.000 190.542 0.000 141.082 |

Wanted: Computation of the closed-route traverse from point 257 to point 156.

Exercise 2:

Adjust the following closed-route traverse and compute the coordinates of the traverse stations A, B, C, D and E.

Exercise 3:

Adjust the following closed-route traverse and compute the coordinates of the traverse stations B, C, D and E.

Exercise 4

The sketch below shows a link traverse with its measured traverse angles and already reduced distances. Adjust the link traverse, do all necessary checks and compute the final coordinates. The entire computation has to be done in the respective computation table!

given coordinates and azimuths:

point             easting[m]       northing[m]

10		           1000.00	1200.00

30		           1407.36	  524.85

Azimuths

from 20-10	156'45"

from 30-40	15157'00"

| Self check Two | Written test |
| --- | --- |

## Self check-2

Test-I Matching

Instruction: select the correct answer for the give choice. You have given 1 Minute for each question. Each question carries 2 Point.

A                                                          B

1.Azimuth                                        A. Azimuth and Bering are equal

2. Station                                         B. The value is less than 900

3.Bering                                           C. Starting from north

4. Back azimuth                                 D. The Theodolite is set up on

5.Quadirant one                                  E. 1800

Test II: short Answer writing

Instruction: write short answer for the given question. You are provided 3 minute for each question and each point has 5Points.

1. The value of back azimuth and forward azimuth is _____________

2. The value of azimuth do to north is _______________

3. If the traverse station is starting and terminating is the same it is _________________Traverse

Part III: Short answer writing

Direction:  Give short answer to the following questions. Time allotted for each item is 2mniut and each question carry 4 point.

1. What is Traverse.

2. List and describe types of traverse.

3. What is the difference between azimuth and bearing .

5. There are two types of close traverse you can describe it.

Note: Satisfactory rating – above 60%           Unsatisfactory - below 60%

You can ask you teacher for the copy of the correct answers

| Operation sheet one | Traverse computation |
| --- | --- |

- Operation title: Traverse computation

- Purpose: To calculate coordinate

- Instruction: Using below operation sheet

- Tools and requirement:

1.Theodolite

2.Tape

3.Compass

4.Range pole

5.Page

6.Calculator

7.Paper

8.Pancel

- Steps in doing the task

1.Reconnasance the project aria .

2.Draw free hand sketch .

3.Sellect equipment, tools and material for collecting data.

4.Maesure maximum horizontal distance .

5.Maesure maximum horizontal angle.

6.Maesure minimum azimuth.

7.Maesure minimum coordinate.

8.Calculate the required coordinate

- Quality Criteria: Measure angle and distance accurately.

- Precautions: use digital Theodolite

| LAP test one | Practical Exercise |
| --- | --- |

Task 1.  Calculate coordinate of each point.

Given

Coordinate       1. X =1000         Y= 1000

2. 1-2 =760

Required            Calculate azimuth

"        Coordinate by traverse method

UNIT THREE: Triangulation.

This unit to provide you the necessary information regarding the following content coverage and topics: - Introduction of Triangulation - Triangulation satiation - Establishing monuments - Measuring angles and base lines - Computing triangulation This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Describe triangulation - Apply placing construction monuments - Measure angles

## 3.1. Introduction Triangulation

Triangulation is consists of a series of joined or overlapping triangles in which an occasional line (called the base line) is measured and all other sides of the triangles are calculated from angles measured at the vertices of the triangles. The lines of a triangulation system form a network that ties together all the triangulation stations at the vertices of the triangles. A triangulation has the following advantages:

- More redundancies or checks are available i.e more than one route can be followed to compute the length of a line.

- There is little tendency for the system to sway or bend i.e azimuths can be easily and accurately carried or established throughout system.

- Outstanding landmarks such as steeples, water tanks, etc can be located by establishing directions from different stations.

Its disadvantages are it needs long-range intervisibility, which in turn requires the erection of special towers and signals, making the system the most expensive. Moreover, a good weather is required to attain intervisibility.

Unlike triangulation, it is not easy to position transmission towers, steeples, water tanks, etc. by the EDM because to do so requires reflectors on these landmarks. These landmarks can be located if angular measurements are made to them.

In combined triangulation and trilateration systems all sides and angles in the joined or over lapping triangles are measured. This method provides the strongest control network.

Triangulation as a form of horizontal control is applied when a large area is to be surveyed and when the methods of traversing would not be expected to maintain a uniformly high accuracy over the entire area. The methods of triangulation require a maximum number of precise angle measurements and a minimum number of distance measurements. The triangles are developed in to a net of interconnected figures, and certain lines, called base lines, must be measured in order to compute the other sides in the net.

Figure:3.1 Triangulation

Triangulation is necessary to control the location of large bridge structures, state and federal highways, dams, canals, and other engineering works of a massive nature. A large project, such as boundary location, power development, water resources development, flood control, irrigation, or reclamation, requires triangulation in order to maintain the necessary accuracy throughout the system.

Once a triangulation system, whether large or small, has been developed, measured and adjusted, the points in the system then furnish control for subsequent traversing, minor triangulation, trilateration, intersection, and resection needed for day-today engineering operations.

## 3.2. Triangulation stations

The points forming the triangulation stations are selected on the basis of visibility as for example on the top of hills or church steeples, or radio towers, or water tanks. As a result, the points are not uniformly spaced. Some of the points are obviously inaccessible and  it is necessary to establish eccentric stations from them and to determine the distance and direction s from the main stations to the eccentric stations.

Sometimes it is necessary to build special towers for making the observations. These towers contain one tower built inside another so that the towers supporting the instrument operator are independent.

For triangulation of lower accuracy, a small pole signal about 2m high guyed in place, or an object already in place might be used for sighting. As most triangulation works of higher accuracy are carried out between late afternoon or an automobile headlight may be used as signal. Signals should be free from phase.

In general, the types of signal used depend on the length of the line accuracy required whereas its form depend s on the locality and availability of materials.

Triangulation stations should be marked and referenced very carefully for use at later days. The signal used must permit centring the instrument if the station is to be occupied

(for instance, an iron pipe set vertically so that a pole can be inserted in to it for sighting and this pole can be removed when centring the instrument over the station).

When towers are required due to flat terrain, heavy timber or other factors, it is necessary to determine the minimum height of the towers required from the equation for the earth’s curvature and atmospheric refraction.

Example :

Determine the height of a tower to permit seeing the surface of the earth at a distance of 10 km.

Solution

h

d

Figure:3.2 Triangulation station

h=0.0675d2  where  d is the distance in km

h is the height of the tower in m.

h = 0.0675 * (10)2 = 6.75m

## 3.3. Establishing  monuments

Triangulation can be used for:

1. The establishment of accurately located control points for surveys of areas.

2. The accurate location of engineering works such as the location of large bridge structures, state and federal highways, dams, canals, tunnels, and others in order to maintain the necessary accuracy throughout the system.

3. The establishment of accurately located control points in connection with aerial surveying.

## 3.4. Measuring angle and base line

- Base line measurement

Since the computed sides of triangulation system can be no more accurate than the baselines, every precaution to ensure accuracy is taken in measuring these lines. The length of any baseline is determined primarily by the desirability of strong figures in the net. Ordinarily, the longer the baseline , the easier it will be found to secure strong figures. Short baselines are connected to the triangulation system through a base net.

Fig.3.3 Base net

Because of errors in angular measurements, the accuracy of the computed lengths will decrease as the distance from the baseline increases. On extensive triangulation the accuracy is maintained by measuring additional base lines.

Base lines can be measured either by tapes or by EDM equipment. For measurement of higher precision, invar tape is always used. Some of the activities required while taping are:

- Clear obstructions along the baseline;

- Setting stakes on line at distances apart equal to the length of the tape to be used;

- Fastening copper stripes to the top of these stakes for making the tape lengths;

- Providing support for the tape;

- Running levels to determine difference in elevation where required;

- Recording field temperature and pull and applying systematic corrections; etc.

Once systematic corrections have been applied to the measured lengths, the remaining step is to reduce all the triangle lengths to equivalent sea level distances in all extensive triangulation surveys.

In the figure, A and B are two points on the earth’s  surface at an average elevation of h above sea level. Since the lengths of the arcs are proportional to their radii,

Where 	S= sea level length, m

Fig 3.4 Reduction to sea level                       D = measured length at average elevation of h above sea level

R=6,372,200m

Example

The elevation of measured base line is 540m above mean sea level. The recorded length of the base line is 1035.407m. Compute the length of the base line at sea level

Solution

S 	= RD / (R+h)

= (6372200* 1035.407) / (6372200+540)

= 1035.319m

- Angle measurement

The instruments to be used for measuring angles for triangulation depend on the accuracy desired in locating the positions of the triangulation stations. If first-order work  is desired, directional theodolites should be used with which directions can be read directly to 0.2”. For second-order work, it is necessary to use instruments capable of being read to 1”. For third –order triangulation, engineer’s transits that can be read to 20” or 30” may be used if the angles are measured by repetition.

When the directional theodolites is used, it is set up over a particular station and pointed to each of the desired stations. For first-order triangulation the set of readings is repeated from 8 to 16 times , while for second order work they are repeated from  4 to 8 times.

## 3.5.Computing  triangulation

Before the length computation begin, it is necessary to make triangulation adjustments.

### 2.5.1.Adjustment of Chain of Single Triangles

When chain of single triangles are used, it is necessary to make the following adjustments.

- Station adjustments: the sum of the angles around each point should exactly be 3600.

- Figure adjustments: the sum of the angles in each triangle should exactly be 1800.

For the angles about a point the difference between the sum of measured angles and 3600  is balanced equally between the numbers of angles.

In the same fashion the difference between the sum of the measured angles in each triangle and 180 is balanced equally between the angles.

Example

- For the triangles shown, the measured angles are given . Make station and figure adjustment to the angles.

A

2

1

3

5

4

B

C

8

7

6

9

10

D

Measured angles

- 41   16  10

- 53   33  20

- 265  07  45

- 91    16  10

- 268  43  30

- 35    07  00

- 78    42  30

- 246  11  00

- 60     01  05

- 249   59  15

solution

  - station adjustment

| Station | angle | measured | adjusted |
| --- | --- | --- | --- |
| A | 1 | 41 16 10 | 41 16 05 |
|  | 2 | 53 36 20 | 53 36 15 |
|  | 3 | 265 07 45 | 265 07 40 |
|  |  | 360 00 15 | 360 00 00 |
| B | 4 | 91 16 10 | 91 16 20 |
|  | 5 | 268 43 30 | 268 43 40 |
|  |  | 359 59 40 | 360 00 00 |
| C | 6 | 35 07 00 | 35 06 50 |
|  | 7 | 78 42 30 | 78 42 20 |
|  | 8 | 246 11 00 | 246 10 50 |
|  |  | 360 00 30 | 360 00 00 |
| D | 9 | 60 01 05 | 60 00 55 |
|  | 10 | 299 59 15 | 299 59 05 |
|  |  | 360 00 20 | 360 00 00 |

  - Figure Adjustment

| Triangle | angle | Angle value after station adjustment | Angle value after figure adjustment |
| --- | --- | --- | --- |
| ABC | 2 | 53 36 15 | 53 36 27 |
|  | 4 | 91 16 20 | 91 16 32 |
|  | 6 | 35 06 50 | 35 07 01 |
|  |  | 179 59 25 | 180 00 00 |
| ACD | 1 | 41 16 05 | 41 16 18 |
|  | 7 | 78 42 20 | 78 42 34 |
|  | 9 | 60 00 55 | 60 01 08 |
|  |  | 179 59 20 | 180 00 00 |

N.B. give the larger correcting factor, to large measured angle.

Check the station

A= 1 + 2 + 3 = 3600 00’ 25’’

Adjust angle 3 =2650 07’ 40’’ – 000 00’ 25’’ = 2650 07’ 15’’

### 3.4.2  Adjustment of a quadrilateral

When a chain of quadrilaterals are used, there are two condition which must be satisfied.

- Geometric condition: the sum of the interior angles must equal to            (n-1)*1800 ; where n is the sum number of sides of the figure.

- Trigonometric condition: The sine of each angle must be proportional to the opposite side of that triangle.

Consider the diagram below;

a

b

c

d

e

f

g

h

Base line

A

D

Let AD be the base line and angles a to h be measured angles.

A. Adjustment of quadrilateral by  approximate method

- Geometric condition

- the sum of interior angles of a triangle should be 1800.

For ABC ; a + b + c + d  = 180

For BCD; c + d + e + f =180

For ABD; a + b + h + g =180

For ACD ; h + g + f  + e = 180

- The sum of interior angles of quadrilateral should be 3600.

a + b + c + d + e + f + g + h  = 360

- The sum of opposite angles ( at the intersection of the diagonal) should be equal.

a + b = e + f

c+ d = g + h

- Trigonometric condition

Log { (sin a) ( sin c) ( sin e) ( sin g ) } – log{ ( sin b)( sin d)( sin f)( sin h)} = 0

Logsina+ Logsinc+ Logsine+ Logsing = Logsinb + Logsind + Logsinf + Logsinh

This geometric condition to be satisfied for quadrilateral adjustment

Steps in the adjustment of quadrilaterals

- Correct each of the eight angles so that their sum will be exactly 360.

- Adjust the vertically opposite angles so that their sum should be equal.

- Record the log sines for every alternate angles

- For each angle record the tabular logarithmic sine difference for 1” opposite each logarithm.

- Find the average required change (α ) in log sines by dividing the differences between the sums by eight.

- Find the average difference () for 1”.

- The ratio of (α/) gives the number of seconds or arc to be applied as a correction . this correction is added to each of the four angles whose sum of log sines is the smaller and subtracted from each of the other four angles   whose sum of log sines is the larger, and thus the correct value of angles are obtained.

#### Example

Measured angle

a = 38   44   06

b = 23   44   38

c = 42   19   09

d = 44   52   01

e = 69   04   21

f = 39   37   48

g = 26   25   51

h = 75   12   14

= 360 00 08

1.Make the necessary adjustment for the following quadrilaterals.

b

c

d

e

f

g

h

a

A

D

B

C

Solution

Step 1. Diff. = 08”		correction = 08”/8 = 1”

Step 2. b + c = 660 03’ 45’’

F + g = 660 03’ 37’’

= 08”/4 = 02” (angle f and g should be increased by 2” and angle b and  c are decreased by 2” )

a + h = 1130 56’ 18’’

d + e = 1130 56’ 20’’

02”/4 = 0.5” ( add 0.5” to a and h and subtract from d and e

step 3. compute the log sines for every alternative angles.

Log sin a = -0.203621777		log sin b = -0.395087669

Log sin c =-0.17182419		Log sind = -0.15152906

Log sin e = -02963892                        log sin f = -0.195294325

Log sin g = -0.35152145                     log sin h = -0.014645342

Step 4. Compute the logarithmic sine difference for 1”

Log sin (a + 1”) - Log sin a = 2.6*10-6

Log sin (c +1”) - Log sin c = 2.3*10-6

Log sin (e + 1”) - Log sin e =0.9*10-6

Log sin (g + 1”) - Log sin g = 4.3*10-6

Log sin (b + 1”) - Log sin b = 4.8*10-6

.etc

step 5

7.2434434 – 7.2433935 = 49.9 * 10-6

therefore α = (49.9 * 10-6 ) / 8 = 6.24 * 10-6

step 6

= 20.2 * 10-6 / 8 = 2.53 * 10-6

step 7

= 6.24/2.53 = 2.47”

Correction: add 2.47” to all angles whose sines of log sine is smaller ( i.e for angles a, c, e and g )  & vice-versa( subtract 2.47” from b, d, f, and h )

Note : If the corrections are properly applied, the sum of the log sines of the corrected angles will agree with in 1 or 2 in the last 7th decimal place.

3.5.2. Adjustment and computation of a centre-point triangle

B     3	     4	S

2		   5

8

7 	 9

1 6

F

Figure:3.3. Adjustment centre-point

The procedure for solving the centre point triangle is as follows:

  - the angles in any triangle must sum to 1800 . (i.e.4+3+8=5+9+6=1+2+7=1800)

  - the angles at the centre station must sum to 360 without altering any previous adjustment . (i.e 8+9+7=3600)

  - the side condition :

Log sin 1 Log sin 3 Log sin 5=Log sin2+ Log sin4+ Log sin6

### 3.5.3. Eccentric Stations (Reduction to centre)

In the measurement of triangulation angles it often is not possible to centre a theodolite over a station directly below the target (church spire, lighthouse), which makes direct angle measurement equally difficult or impossible.

A field procedure that can be adopted to obtain angles at the spire or other mark is to set the theodolite at a point as near as possible to the spire. From this eccentric station, angles are observed to all points in the network as if the inaccessible point (spire) itself was being occupied.

Since the angles observed at the eccentric station will be slightly different to those, which would have been observed at the inaccessible or main station. In computing the corrections to be applied to the measured angles or directions, the simplest method is to calculate first the directions of the lines with respect to the line between the eccentric station and the true station as a meridian. Corrections are applied to the measured angles to give those applicable to the main station. This correction procedure is known as reduction to centre.

M

Y

e2	e1

C

E	1

2

Figure:3.4. reduction to centre.

In figure above, stations C ,Y and M are triangulation stations, station C being inaccessible. At the eccentric station E, the directions to C,Y and M are observed and the distance EC measured. This distance should be as small as possible and measured as accurately as possible. The observed directions to Y and M are converted to their equivalents at station c by applying corrections e1 and e2.

In the triangle ECY,

e1’’ =

The value of L is not known but its approximate value can be taken as EY and EM which is obtained by a preliminary computation of the triangles using E as the triangulation station.

The sin of the correction is given by sin1  and if the ratio d/D < 1/140 the formula cannot be used, but the ratio is not likely to be less than this in practice.

Therefore;

e1 = 	and 	e2 =

So that;

Direction CY = direction EY + e1

Direction CM = direction EM + e2

Angle MCY in main triangulation scheme

= direction CY – direction CM

= direction EY + e1 – (direction EM + e2)

Example

At inaccessible station B in a triangulation network an eccentric station E was set up nearby in order to obtain angle ABD. The following measurements were taken:

DAE = 7335m		AED = 95016’40.52”

DDE = 5556m		DEB = 169027’47.18’’

B

E

D

A

Solution

Converting the observed angles taken at E into clockwise directions gives, with direction EB as a reference direction:

ED = 169027’47. 18’’ = 1

EA = 169027 47.18’’ + 95016’40. 52’’

= 264044’27. 7’’ = 2

Reduction to centre

Station		A			D

Distance 		7335m			5556m

264044’27.7’’    	169047’27.7’’

sin 		-0.99579		+0.18287

d/sin1’’		1.349 *106		1.349 *106

e		-183.14’’		+44.40’’

angle AED 	= direction of BA  - direction BD

= direction of EA + e1 – [direction of ED + e2]

= 264044’27. 7’’ - 003’3.14’’ – [169027’27. 7’’ + 44.40”]

= 95013'12.46''

### 3.5.3. Computation of lengths

Two sides of each triangle are computed by using the law of sines, since one side of the triangle is always known and the three angles have been measured and adjusted. In computing the sides of the triangles in a quadrilateral, such as that in fig. below, the solutions of two triangles is sufficient to compute the positions of the forward triangulation stations. The two triangles chosen must be the strongest route through the quadrilateral.

a = 63017'28.12''

b = 84018'20.37''

c = 17052'26.69''

d = 30041'16.98''

e = 47007'55.96''

f = 66035'55.30''

g = 35034'51.76''

h= 14031'44.82''

C

D	f

g	e

h

Figure:3.5.Trangulation length computation

In the figure above, the side AB is the known or measured line of the quadrilateral, and the strongest route is obtained by considering triangle ABC and triangle DAC in that order. To check the accuracy of the fieldwork or the consistency of the figure adjustment, the two triangles ABC and triangle BDC can be computed in that order. This computation gives a check on the length of the side CD, which is the forward side of the quadrilateral. However, only the result for the strongest route will be used in the further computations.

If the coordinates of stations A and B are known, the length of line BA computed by:

DAB = [ (EB - EA)2 + (NA - NB)2 ]1/2

If 	EA=1,442,416.25		 NA=	622,516.21	&

EB=1,445,317.844		NB=621,754.14

DAB= 3000.000m

With the side AB known, the sides AC and CB are computed by using the law of sines.

Also,

the line CA = AC is then used as the starting side of triangle DCA.

Computation of lengths in triangles

Sides		Stations	Angles	Adjusted angles		Distances

Triangle ABC

AB										3000.000

C		e		47007'55.96''		            0.73292548

B		c+d		48033'43.67''		            0.74967381

A		b		84018'20.37''		            0.99506538

180000'00.00''

AC										3068.554

CB										4072.987

Triangle CAD

CA										3068.554

D		h + g		50006'36.58''		            0.76727890

C		f		66035'55.30''		            0.91774558

A		a		63017'28.12''

AD										3670.311

CD										3572.554

### 3.5.4. Coordinate calculations

A) Computation of Azimuths

If the coordinates of stations A and B are known, the azimuth from north of the line AB is determined by:

tan  AAB =

AAB = tan-1

AAB = 104042'56.6''

Azimuth of AB = 104042'56.60''			Azimuth of BA  = 284042'56.60''

-b = -84018'20.37''		                           +(d + c)  = + 48033'43.67''

Azimuth AC    = 20024'36.23''			azimuth of   BC  = 333016'40.27''

-a = -63017'28.12''		              azimuth  of CA   = 200024'36.23''

Azimuth AD = 317007'08.11''		      	           + angle f   = + 66035'55.3''

Azimuth of CD   = 267000'31.53''

- azimuth CB = -153016'40.2''

( f + e) = 113043'51.2''		(check)

B) Position Computation

The coordinates of all the stations throughout the triangulation system are computed by using the strongest route of triangles in the network. When a station is reached whose coordinates are fixed, the positions of the intermediate stations can then be adjusted. If the triangulation system is of great extent and high precision, this adjustment should be made by application of the least-squares principle. If the system is moderate in extent, an application of the compass rule will give highly satisfactory results. In this case a traverse extending from one fixed point to another fixed point and including all the intermediate triangulation stations is selected in as direct a line as possible. This traverse is then adjusted by the compass rule.

| Operation sheet two | Triangulation computation |
| --- | --- |

- Operation title: Triangulation computation

- Purpose: To calculate coordinate

- Instruction: Using below operation sheet

- Tools and requirement:

1.Theodolite

2.Tape

3.Compass

4.Range pole

5.Page

6.Calculator

7.Paper

8.Pancel

- Steps in doing the task

1.Reconnasance the project aria .

2.Draw free hand sketch .

3.Sellect equipment, tools and material for collecting data.

4.Maesure minimum horizontal distance .

5.Maesure maximum horizontal angle.

6.Maesure minimum azimuth.

7.Maesure maximum coordinate.

8.Calculate the required coordinate

- Quality Criteria: Measure angle and distance accurately.

- Precautions: use digital Theodolite

| LAP test two | Practical Exercise |
| --- | --- |

Task 1.  Calculate coordinate of "C" by triangulation method

Given

1.Angle "A" =580400200

2.   "      "B" =610200300

3.   "      "C" = 600000020

Coordinate       A. X =1000         Y= 1000

B. X = 1040         Y =1030

Required            Calculate azimuth of "AB" from coordinate

"        distance of "AB" from coordinate

"        Coordinate of "C"

# UNIT FOUR: INTERSECTION

This unit to provide you the necessary information regarding the following content coverage and topics: - Introduction of intersection - Computing intersection This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Describe intersection - Apply placing construction monuments - Measure angles

## 4.1.Introduction of Intersection

The horizontal control of a new point can be determined by measuring angles from two points of known coordinates. The method of intersection is applicable to situation in which a point is to be established in the net work of triangulation system. but the point is not occupied (i.e it is a point of no instrument set up.)

The horizontal control of a new point can be determined by measuring angles from two points of known coordinates. This process is called intersection.

The coordinates of A and B are known, to determine the coordinates of point M by measuring the angle a ά and  as follows.

Figure:2.1

## 4.2.Computing Intersection

Exercise

Given that the coordinates of A are X;. 6000. Y= 2320 and of B are, X:6894 25, Y:2246. The angular measurements of a at A is 30° 28' 15" & e on B is 94° 18' 45" then compute the

Coordinates of point P

GIVEN

| Point | X | Y |
| --- | --- | --- |
| A | 6000.00 | 2320 |
| B | 6894.25 | 2246 |

| Station | Target | Direction |
| --- | --- | --- |
|  | p | 0°00"00. |
| A | B | 30° 28'15" |
|  | A | 0° 00'00. |
| B | P | 94° 18'45.” |

=94018’45”

+=124047’00

Azimuth AB =Tan-1(XB-XA)/(YB-YA)

=Tan-1(6894.25-6000)/(2246-2320)

=tan-1(874.25)/(-74)=94043’49.8”

Azimuth of BA=AzAB+1800=274043’49.

Azimuth of AP=AZAB-a,94043’49.8”-30028’15”

| Operation sheet Four | INTERSECTION |
| --- | --- |

- Operation title: Triangulation computation

- Purpose: To calculate coordinate

- Instruction: Using below operation sheet

- Tools and requirement:

1.Theodolite

2.Tape

3.Compass

4.Range pole

5.Page

6.Calculator

7.Paper

8.Pancel

- Steps in doing the task

1.Reconnasance the project aria .

2.Draw free hand sketch .

3.Sellect equipment, tools and material for collecting data.

4.Maesure minimum horizontal distance .

5.Maesure maximum horizontal angle.

6.Maesure minimum azimuth.

7.Maesure maximum coordinate.

8.Calculate the required coordinate

- Quality Criteria: Measure angle and distance accurately.

- Precautions: use digital Theodolite

# UNIT FIVE: RE SECTION

This unit to provide you the necessary information regarding the following content coverage and topics: - Introduction of resection - Writing the description for station - Establishing Resection - Computing re section This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Introduce to intersection & resection - Establish intersection points - Computing re section - Identify resection references - calculate station coordinates

## 5.1.INTRODUCTION OF  RESECTION

Resection is a method of locating a point by taking angle observations from it to at least three known stations in a network. The situation is illustrated below:

Figure:5.1 Resection diagram

Point P can be fixed by observing angles P'' and P' subtended at resection point P by control stations A, B, and C.

A summary of the method used to compute the coordinates of station P as follows:

- From the known coordinates of A, B, and C calculate lengths b and c, and angle α at station B.

- Subtract the sum of angles X, Y, and α in figure ABCP from 3600to obtain the sum of angles X + Y.

3600- (α + p' + p'') = X +Y

- Calculate angles X and Y using the following:

X+ y = 360 - (α + p' + p")

1/2 ( X + y)= 1800-1/2 (α + p'+ p")------------------(a)

- From angles X and azimuth BA, calculate azimuth BP in triangle ABP. Then solve for length AP using the law of sines, where α1=1800 - X - p''. Calculate the departure and latitude of BP followed by the coordinate of P.

- In the manner outlined in step 4, use triangle BCP to calculate the coordinates of P to obtain a check.

## 5.2. Writing the description for station

The task is mainly given remarks and descriptions for those control points of stations in order to have more clear information.

Writing description is making references to the X and Y values of the stations.

Some of the reasons for writing descriptions for stations are

- To find the Bench mark easily

  - To re- establish a bench mark which is destroyed during construction or maintenance.

Any reference that can be seen, well defined and exist able permanently such as concrete. or Metallic poles, concrete fences, buildi"9 corners etc can be used.

There are few points to be reminded during writing descriptions for stations.

1. The measured distance. should be horizontal.

2. The distance from station to reference points should not exceed 30 m.

3. The field book should be neat, readable and sequentially arranged, since the description for stations, reference points and others are being documented.

## 5.3.  Establishing Resection

The method of Resection is a convenient way of determining the position of unknown point by occupying the unknown point and measuring the horizontal angles between at least.

Three points of known coordinates

Figure:5.2 Resection diagram

The horizontal angles and coordinates of A, B & C are known, and the coordinates of a new point P are to be determined by measuring the angles M & N being the instrument is set up on point P

## 5.4. computing resection

##### Example

1. In the figure above, angles X and Y were measured.

P' = 30042’37’’

P'' = 25012’15’’

Coordinate points A, B, and C have coordinates (in metre) of:

XB = 12750.000	XA = 16820.540	XC = 22190.788

YB = 10271.000	YA = 10105.772	YC = 11317.229

Calculate the coordinates of P.

Solution

- Compute b, c, and α

c 	=

=

=

c 	= 4073.892

b	=

=

b = 5505.197

α = azimuth of AB – azimuth of AC

=  AAB - AAC

AAB =  tan –1

=  tan –1 = 272019’27. 9’’

AAC =  tan –1

= tan –1    = 77017’15. 43’’

α =  AAB - AAC

=  272019’27. 9’’ – 77017’15. 43’’

= 195002’12. 47’’

- Compute X + Y

1/2(X + Y )  = 360 – (α +p'' + p')

= 360 – (195002’12. 47’’ +30042’37’’ + 25012’15’’)

= 109002’55. 53’’

- Calculate X and Y

1/2 ( X+Y) =1800-1/2(α + P' + p")

=1800-1/2(195002'12.47"+30042'37"+25012'15")

=54031'27.76"

1/2(X-Y)=tan-1[cot(Z+450)tan1/2(X+Y)]

Z + 45 = 76040'36.93"

- Compute i) ABP,ii) α1 iii) BP iV) ∆XBPand ∆YBP and finally iiv) XP and YP

ABP	= ABA + X

= 92019'27.9'' +72054'22'' = 165013'49.9''

α1  	= 180 - A - P = 180 - 72054'22'' - 30042'37''=76023'01''

BP	= AB * sin76023'01''/sin30042'37''  = 7752.912m

∆XBP = BP * sin ABP = 7752.912 * sin 165013'49.9'' = 1976.454m

∆YBP = BP * cos 165013'49.9'' = -7496.751m

XP = XB + ∆XBP = 12750 +1976.454 = 14726.454

YP = YB + ∆YBP= 10271-7496.751 =2774.249

- To check use triangle BCP.

Example

- The coordinates of two control points A and B are XA= 602,105.32 and YA= 126,118.90m; XB= 601,048.82m and YB = 125,613.48m. The clockwise angle at A from B to unknown point P is 52018'46.2'';the counter clockwise angle at B from A to P is 37028'16''. Compute the coordinates of P by intersection.

SOLUTION

= 52018'46.2'' and  = 37028'16''

To compute coordinates of P , we should have to compute the azimuths of AP (AAP) or azimuth of BP (ABP) and length AP or BP.

Azimuth AP (AAP) = azimuth of AB (AAB) + A

Azimuth BP (ABP) = azimuth of BA (AAB) - B

AAB = tan-1  = tan -1	= 64026'02.3'' + 1800

= 244026'2.3''

ABA= azimuth of AB +or - 1800 = 64026'2.3''

AAB = 2440 26’ 2.3’’	ABA = 64026’2.3’’

+A  = 52018’46.2’’   	- B = 37028’16.0’’

AAP = 296044’48.5’’	ABP = 26057’46.3’’

Using the sine Rule

DBP = DAB * sinA/sinB  and		 DAP = DAB * sin B/sinP

DAB = 1171.171m

A + B + P =1800

52018’46.2’’ +  37028’16’’ + P =180

P = 90012’57.8’’

DAP = DAB * sinB/sinP

= 1171.171 * sin 37028’16’’/sin 90012’57.8’’

= 712.500m

DBP = DAB * sin A/sinP

= 1171.171 * sin 52018’46.2’’/sin 90012’57.8’’

= 926.825m

The coordinates of P along AP can be computed as

EP	= EA+DAP*sin (AAP)

= 602,105.32+712.500*sin2960 44’ 48.5’’

= 601,469.055

NP	= NA+DAPcos (AAP)

=126,118.90+712.500*cos2960 44’ 48.5’’

=126,439.560

To check, compute along BP

EP  	= EB + DBP*sinABP

= 601,048.82 + 926.825*sin260 57’ 46.3’’

= 601,469.054

NP	=NB+DBPcosABP

=125,613.48+926.825*cos260 57’ 46.3’’

=126,439.560

2) The coordinates in feet of three control points are as follows

Point                  Y                            X

A                   98,202.66                38,762.50

B                   110,002.65              61,252.84

C                   89,102.32                78,565.12

- From an unknown point P1 lying southerly from A, the clock wise angle from A to B is measured as 220 30’ 14.6’’ and that from B to C as 290 48’ 50.6’’

Compute the coordinates of P1.

Answer

Xp1 = 89781.053

Yp1 = 55772.609

| Operation sheet two | computing resection |
| --- | --- |

- Operation title: computing resection  computation

- Purpose: To calculate coordinate

- Instruction: Using below operation sheet

- Tools and requirement:

1.Theodolite

2.Tape

3.Compass

4.Range pole

5.Page

6.Calculator

7.Paper

8.Pancel

- Steps in doing the task

1.Reconnasance the project aria .

2.Draw free hand sketch .

3.Sellect equipment, tools and material for collecting data.

4.Maesure minimum horizontal distance .

5.Maesure maximum horizontal angle.

6.Maesure minimum azimuth.

7.Maesure maximum coordinate.

8.Calculate the required coordinate

# UNIT SIX: TRILATRATION

This unit to provide you the necessary information regarding the following content coverage and topics: - Trilatration system - Adjustment of measured distances This guide will also assist you to attain the learning outcomes stated in the cover page. Specifically, upon completion of this learning guide, you will be able to: - Identify trilatration - Compute trilatration

## 6.1. TRILATRATION SYSTEM

A trilateration net work, like triangulation, may consist of a series of single triangles, braced quadrilaterals and centre point polygons or combinations of these but, unlike triangulation, the observed quantities are the distances not the angles.

The methods used in trilateration computations are very similar to those used in triangulation and proceed as follows.

- Using the measured or calculated baseline length and all the measured lengths, the angles in the network are calculated using the cosine rule. This is carried out by breaking down all the figures into their constituent triangles and proceeding as follows.

In triangle ABC of figure below, the angles are given by:

Cos α =

Cos  β =

Cos γ =

Figure:6.1 Trilatration diagram

- These computed angles are adjusted by the method discussed in adjusting triangulation systems.

- The strength of the figure of the system is evaluated.

- Starting at the baseline, all the lengths of the sides in the network are computed using the adjusted angles. In addition the bearings of all the triangle sides throughout the network are computed, again using the adjusted angles.

- The computed bearings and distances obtained in (4) are used to calculate coordinates throughout the network.

A disadvantage of trilateration is that, compared to triangulation, the number of geometric conditions for adjustment is less than that for equivalent figures.

## 6.2. ADJUSTMENT OF MEASURED DISTANCES

Before the lengths of the lines in the trilateration net can be used in any subsequent computations, their slope lengths determined by the instrument and corrected for atmospheric conditions must be reduced to the corresponding sea-level distance.

a) Reduction Of Slope Distance By Vertical Angles

The positive angle α and the negative vertical angle β are measured from stations A and B , respectively. The angle α is the refraction angle of the line of sight, assumed to be the same at both stations.

Figure:6.2 Slop distance

In the triangle AVB:

(β + α ) + (90 – ) + [(900- θ ) + α ] =1800

2 α  =  + θ – β

α  = /2 – β/2 + θ/2

In the triangle ABB1

Angle BAB1 = θ – α +  /2 = θ – (θ /2 – β/2 + /2) +/2

δ = BAB’=

where      	θ is vertical angle at A and

β is vertical angle at B

AB1= S cos δ

BB1= S sin δ

or the refraction angle α is 1/7 of the curvature;

δ = θ -  + /2 = θ -/7 + /2

= θ + 5 / 7

B1B2 =  BB1 tan (/2)

Finally, sea level (chord) length A'B' can be computed from

A'B' = AB2 () where hA is the elevation of station at A.

Arc A'B' = R  		 where  is in  radian

b) Reduction of Distance Observations Using Elevations

A procedure for reducing long slope distances to their ellipsoid lengths is discussed below:

Fig.

Figure:6.3.Slop distance in vertical

| Operation sheet two | Practical Exercise |
| --- | --- |

- Operation title: Triangulation computation

- Purpose: To calculate coordinate

- Instruction: Using below operation sheet

- Tools and requirement:

1.Tape

2.Compass

4.Range pole

5.Page

6.Calculator

7.Paper

8.Pancel

- Steps in doing the task

1.Reconnasance the project aria .

2.Draw free hand sketch .

3.Sellect equipment, tools and material for collecting data.

4.Maesure minimum horizontal distance .

5.Maesure minimum azimuth.

6.Maesure minimum coordinate.

7. Calculate all distance

8.Calculate the required coordinate

- Quality Criteria: Measure angle and distance accurately.

- Precautions: use digital Theodolite

| LAP test | Practical Exercise |
| --- | --- |

Task 1.  Calculate all distance and coordinate by trilatration method

Given

Distance

A-B=25m

B-C=30m

C-A=28M

4. Azimuth AB is 1100

5. Coordinate A= X=1254  Y= 1428

Required            Calculate all angle

"       Azimuth of BC, CA

"        Coordinate of  B, C

Reference

1.Dr AM Chandra .Plane surve

2.S.K.Roy (2010) Fandamental of surveying. (second edition).

3.A.M.Chandra (2007) Plane surveying

4.Charles D. Ghilani and paul R.WOLF.Introduction to geomatics 12 Edition.

5. Dr AM Chandra. Problem solving with theory and objective type question.

6. Dr AM Chandra. Higher Survey.

7. John wiley and sons . Elementary survey

8. S.K.ROY. Fundamental survey

9. CHARLES D. GHILANI. Elementary Surveying. Thirteenth Edition.

Participants of this Module (training material) preparation

| No | Name | Qualific ation | Field of study | Organization/ Institution | Mobile number | E-mail |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Egigayehu Gizachew | A | CTOM | Hossana poly technique college | 0912141058 |  |
| 2 | Addis shimellis | A | Geoinformatics | Athlete. kenenisa poly technique college | 0920165853 | Addisshimellisabebe@gmail. com |
| 3 | Getachew Nigussie | B | Surveying technology | Yirgalem industrial and constriction technique college | 0913249671 | Getalen14@gmail.com |
| 4 | Tigabu | B | Surveying technology | Man buk TVETE technique college | 0918216048 | Abatetigabu76@gmail.com |